| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, Italy, 14-22 July 2011 | Document: JCTVC-F564 |
| --- | --- |

| Title: | Near Lossless Coding for Screen Content |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wen Gao(1) Gregory Cook (1) Mingyuan Yang (2) Jin Song (2) Haoping Yu (1) (1). Huawei Technologies (USA) 2330 Central Expressway Santa Clara, CA 95050 (2). Huawei Technologies Co., LTD Section B, Bantian, Longgang District Huawei Base, Shenzhen, China | Tel: +1 408 330 4816 Email: wen.gao@huawei.com gregory.cook@huawei.com yangmingyuan@huawei.com songjin@huawei.com haoping.yu@huawei.com |  |
| Source: | Huawei Technologies (USA) |  |  |

_____________________________

# Abstract

This contribution proposes a near lossless coding method that only involves a few modifications to the existing intra coding tools t. This method is designed for encoding texts and graphics at high quality and high coding efficiency, and screen sharing is one of potential applications of this method, where computer display signal is treated and processed and delivered as video. Typical screen capture video consists of a mix of cameral-captured video, computer generated graphics, texts, etc., and fidelity or the quality of the text and graphics are extremely important. The major problems with today’s solution include poor compression quality, low frame rate, and long latency, and poor quality of text can cause eye-fatigue. In summary, neither compression ratio nor compression quality with the existing solutions is satisfactory. To address this problem, a set of modifications to the existing tools are proposed to achieve both near-lossless quality and high compression efficiency in coding of screen compound video. For purpose of evaluation, these modifications and the associated syntax element changes have been integrated into HM3.0-dev-SDIP branch. Two test sequences proposed to the SCC Ad-Hoc group are used in the tests. . Compared to baseline HM3.0-dev-SDIP software, the proposed method can maintain the high coding efficiency of HM while achieving near lossless quality for the text regions.

# Introduction

Interactive video applications have attracted more and more attention from both consumer and enterprise markets, which include video conferencing, web-conferencing, on-line gaming, etc. Screen sharing is one of the important features besides document sharing and application sharing, and it is one of the key technologies in VDI (Virtual Desktop Infrastructure).

Fidelity or the quality of the text and graphics are very important in screen sharing applications, and poor quality rendering of text can cause eye-fatigue. The major problems with the today’s solution include poor compression quality, low frame rate, and long latency. In summary, both compression ratio and compression quality are not satisfactory.

HEVC inter-frame coding is rather efficient with the existing SKIP and DIRECT mode in compressing the non-camera captured video content such as text and graphics. This is mainly due to the characteristics of this type of content, e.g. no motion blurring, no optical filtering, etc. However, the intra coding in HEVC is not satisfactory because of the lack of a lossless mode. Compressing these non-camera captured contents with degraded quality will not only cause eye-fatigue but also introduce extra information (compression noise) to the subsequent inter-frame coding. Therefore, having a lossless mode will improve not only compression quality but also compression efficiency as well. . To achieve this goal, we propose a near lossless mode for HM that can compress text with high efficiency and high fidelity. The reason we cannot achieve lossless compression is that the transforms defined in current HEVC draft is not completely reversible. The additional syntax elements associated with these tools have been integrated into the HM software HM 3.0-SDIP branch.

# Near Lossless Coding Mode

## Existing HM software

For illustration purpose, the encoding/decoding process in HM is summarized in the Figure 1.

Figure 1 – HEVC Encoding/Decoding Process

Note that the quantization and de-quantization modules and transform and inverse transform modules appear in pairs in Figure 1. In the following description, if we remove one module, for example the quantization module, it also implies that the corresponding reverse module, e.g., de-quantization module, is removed as well in the encoder. Consequently, the decoder would not use the de-quantization module either.

## Transform Bypass Lossless Mode

When applying the existing HM software 3.0 to encode compound video with high fidelity (for example, using QP=0), we observe that certain regions, especially the regions comprising text, consumes a larger number of bits in the bit stream when the transform is used, compared to a lossless compression scheme when the transform and the following quantization modules are bypassed. For illustration purposes, we call this scheme as transform bypass lossless mode, as shown in Figure 2.

Figure 2 – Transform Bypass Mode

## Transform without Quantization

On the other hand, in certain blocks of a text region, the current HEVC coding scheme with the transform module included  may generate in a shorter bit stream, even without quantization module. However, sometimes we won’t obtain perfect reconstructed blocks for this case since the transforms used in the current HEVC is not perfectly reversible. We observed that the reconstructed pixels may be different from the original pixels by up to +/- 1 level. We call this scheme the transform without quantization mode, as shown in Figure 3. Thus the combination of transform bypass mode and the transform without quantization mode is called as near-lossless coding mode.

Figure 3 – Transform without Quantization Mode

## RDO process for near-lossless coding mode

Since quantization is not used in either the transform bypass lossless mode or the transform without quantization mode, the RDO process is simply to select the mode that results in the least number of bits, in other words, the RDO cost function used in this proposed lossless coding only contains the bit rate portion.

## Prediction Residuals

When the transform bypass lossless mode is used, the prediction residuals are directly sent to the entropy coder. The non-zero prediction residuals in this mode for text region often appear in random positions and do not concentrate in the left-top corner of the block as generally occurs for transform coefficients. Consequently, the distribution of prediction residual is quite different from the regular transform coefficients. For example, the probability of large absolute level of prediction residuals is still significant.

## Pixel based intra prediction (DPCM) in transform bypass mode

In the current HEVC draft, pixels within a prediction unit (PU) are predicted only from pixels in its neighboring PUs. In this contribution, when transform bypass lossless mode is used and when the intra prediction direction is horizontal or vertical, pixels within a prediction unit (PU) are predicted not only from pixels in its neighboring PUs but also from its neighboring pixels within the PU. For example, in Fig. 4, the 4x4 PU to be predicted is represented by Z(i,j), for i=0,…,3 and j=0,…,3. The pixels from neighboring top and left PU are X(i) and Y(i) for i=0,…3, respectively.

# .

Fig. 4 an example of 4x4 prediction unit (PU)

In current HEVC draft, if vertical intra prediction is used, the PU is predicted from its top PU as P(I,J)=X(J) for I=0,…,3 and J=0…,3 where P(I,J) are prediction version of Z(I,J). In this contribution, the 0-th row of the PU is predicted from its top neighboring PU as P(0,J) = X(J) for J=0..,3 while others rows in PU are predicted from its row above, i.e., P(I,J)=Z(I-1,J) for I=1,2,3 and J=0,..3.

In current HEVC draft, if horizontal intra prediction is used, the PU is predicted from its left PU as P(I,J)=Y(I) for I=0,…,3 and J=0…,3. In this contribution, the 0-th column of the PU is predicted from its left neighboring PU as P(I,0) = Y(I) for I=0..,3 while others columns in PU are predicted from its left column, i.e., P(I,J)=Z(I,J-1) for I=0,…,3 and J=1,2,3.

This prediction method is applied to both luma and chroma components.

# Syntax Elements for Near Lossless Coding Mode

## Transform Bypass Flags

In the near lossless encoding regions, a pair of transform bypass flags is introduced at the PU level. The RDO process will determine whether the transform is bypassed for coding of the PU luminance (luma) and chrominance (chroma) components. For example, if transform bypass lossless mode is chosen for the luma pixels of the PU, a luma transform bypass flag is set to 1, indicating the transform and quantization modules are bypassed. Otherwise, a luma transform bypass flag is set to 0 indicating only the quantization module is bypassed and the transform module is in use.

Both flags are encoded in the bit stream through a CABAC entropy encoder. Context models are introduced for luma transform bypass flag and chroma transform bypass flag respectively. The entropy encoder chooses the context with an index that is equal to the summation of the same type of flags in the upper PU and left PU. If either PU is not available, its flag is implied as 0.

## Prediction Residual Encoding in Transform Bypass Lossless Mode

When the transform bypass flag is 1, the level values of luma pixels are encoded differently from the existing schemes which currently are a combination of truncated Golomb-Rice code and Exponential Golomb code (EG). Instead, a fixed length code is used to encode the value of level - 3 when the level is greater than or equal 3. For example, assume the levels are within the range [1, 255], the coding scheme is shown as following:

| Level | B0 | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 |  |  |  |  |  |  |  |  |  |
| 2 | 1 | 0 |  |  |  |  |  |  |  |  |
| 3 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 4 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 5 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| … |  |  |  |  |  |  |  |  |  |  |
| 255 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 |

Note that B0 and B1 are encoded using a CABAC entropy encoder while B2-B9 are encoded in CABAC bypass mode.

In addition, instead of bypass coding, a context mode is introduced when encoding signs of luma and chroma prediction residuals.

# Test Results

To test the performance of the proposed near lossless coding tool, we integrated this method mentioned above in HM3.0-dev-SDIP branch. We used two sequences that are submitted to SCC Ad-Hoc group as our test sequences. We used the common test condition specified JCTVC-E700[1] with the following modification: The internal bit depth is set to 8. The test results are shown in the following Fig. 5 and Fig. 6. The detailed data is included in the accompanying excel spreadsheet.

Fig. 5 – Simulation results for VideoCaptureWinMoving Sequence

Fig. 6 – Simulation results for VideoCaptureSlideEditing Sequence

In summary, using HM3.0 as the anchor, BD performance of HM3.0-dev-SDIP is shown in the following table:

| Sequence | BD-Y rate | BD-U rate | BD-V rate |
| --- | --- | --- | --- |
| ScreenCaptureWinMoving_1280x720_30.yuv | -13.5 | -12.1 | -11.9 |
| ScreenCaptureSlideEditing_1280x720_30.yuv | -13.0 | -10.8 | -10.6 |

Using HM3.0-dev-SDIP QP=0 as the anchor, rate performance of near lossless coding is shown in the following table:

| Sequence | total rate |
| --- | --- |
| ScreenCaptureWinMoving_1280x720_30.yuv | -53.0% |
| ScreenCaptureSlideEditing_1280x720_30.yuv | -41.7% |

# Conclusions

We recommend further exam the proposed transform bypass lossless coding mode and associated syntax elements in a CE group.

# Patent rights declaration(s)

Huawei Technologies (USA) may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Frank Bossen, Common test conditions and software reference configurations, JCT-VC Document, JCTVC-E700, Geneva, March 2011.