| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F568 |
| --- | --- |

| Title: | CE3: Cross-check report for Samsung's test JCTVC-F247 by Motorola Mobility |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jian Lou Koohyar Minoo Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-3427 lou@motorola.com +1-858-404-2521 kminoo@motorola.com +1-858-404-3893 limin.wang@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

This contribution document contains cross-check results of Samsung’s interpolation filter. The simulations were conducted using HM3.0 software. The cross-check has been completed successfully and the results match those provided by Samsung. Attached spreadsheet contains detailed data of the results.

# Introduction

The target of this core experiment is to verify the performance of Samsung’s interpolation filter part 5 [1]. The simulations were conducted using HM3.0 software [2] with Samsung’s modifications. The anchor results are obtained with default interpolation filters in HM3.0 and the tested results are obtained with the proposed interpolation filters.

Different 8 taps filters are used for uni- and bi-directional predictions. Table 1gives the interpolation filter coefficients.

Table 1 Samsung’s interpolation filter coefficients

| Uni-diretiona | 1/2 | { -1, 4, -10, 39, 39, -10, 4, -1} |
| --- | --- | --- |
|  | 1/4 | { -1, 4, -10, 57, 19, -7, 3, -1} |
| Bi-direction | 1/2 | { -1, 4, -11, 40, 40, -11, 4, -1} |
|  | 1/4 | { -1, 4, -11, 57, 20, -7, 3, -1} |

# Simulation environment

## Software

The software used to run the simulations is HM3.0. The binaries were compiled using Microsoft Visual Studio 2008 64bit. The coding conditions used for the simulations are specified in [3]. The macros “CE3_SAM_VARIABLE_FILTER_COEFF” and “TEST5” were enabled in TypeDef.h. YUV file output during the decoding was disabled when collecting the decoding time statistics.

## Hardware

All the simulation experiments were conducted on the PCs with identical settings, i.e., Microsoft Windows 7 64bits, Intel Xeon 3.0GHz with 8G memory. Therefore, both the encoding time and decoding time comparisons are reliable.

# Simulation Results

The encoder reconstructed YUV files match the decoder reconstructed YUV files. The experimental results of the rate-distortion and speed comparison are summarized in Table 2. Detailed results are included in the attachment (JCTVC-F568.xls).

Table 2 Rate-distortion and speed performance comparison of Samsung's interpolation filter part 5

|  | Random access |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.1 | 0.1 | 0.2 | 0.3 |
| Class B | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | -0.1 | -0.2 | -0.1 | -0.2 |
| Class D | -0.1 | -0.2 | -0.2 | -0.4 | -0.1 | -0.2 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | -0.1 | -0.2 | 0.0 | 0.0 |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  |
|  | Low delay |  |  | Low delayLC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.3 | 0.4 | -0.2 | 0.1 | -0.2 |
| Class C | 0.0 | 0.0 | 0.0 | -0.2 | 0.2 | -0.1 |
| Class D | 0.1 | -0.1 | 0.5 | -0.1 | -0.1 | 0.2 |
| Class E | 0.0 | 0.1 | -0.2 | -0.5 | -0.3 | -0.9 |
| All | 0.0 | 0.1 | 0.2 | -0.2 | 0.0 | -0.2 |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | Low delay P |  |  | Low delay LCP |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | -0.6 | -0.3 | -0.3 | -0.5 |
| Class C | -0.1 | -0.1 | -0.1 | -0.2 | -0.2 | -0.5 |
| Class D | 0.1 | 0.0 | -0.2 | 0.2 | -0.2 | -0.1 |
| Class E | -0.3 | 0.1 | -0.6 | -1.3 | -1.1 | -0.6 |
| All | 0.0 | 0.0 | -0.4 | -0.3 | -0.4 | -0.4 |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 98% |  |  | 101% |  |  |

# References

[1] E. Alshina and A. Alshin, “CE3: DCT derived interpolation filter test by Samsung,” JCT-VC document, JCTVC-F247, July 2011

[2] https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-3.0

[3] JCT-VC, “Common test conditions and software reference configurations”, JCTVC-E700, March 2011

Motorola Mobility Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).