| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F569_r2 |
| --- | --- |

| Title: | Adaptive Scan for Large Blocks for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Yu Krit Panusopone Jian Lou Limin Wang | Tel: Email: | +1-858-404-2251 yyu@motorola.com |
| Source: | Motorola Mobility |  |  |

_____________________________

# Abstract

This contribution proposes an adaptive scan for big block size for intra coding to the new video coding standard, HEVC (High Efficiency Video Coding). Without increasing complexity at both encoder and decoder, average coding gains of 0.3% and 0.2% are achieved for low complexity and high efficiency intra coding cases, respectively.

- Introduction

In the HEVC, three possible coding scans, namely zigzag, horizontal and vertical scans, are used for 4x4 and 8x8 TU blocks in intra coding. Given a TU block of 4x4 or 8x8 in intra mode, which of the three scans is used depends upon intra prediction direction associated with the TU block. For example, if the prediction direction is DC, zigzag scan is used. If the prediction direction is horizontal, vertical scan is used. If the prediction direction is vertical, horizontal scan is used. Table 1 summarizes the relationship between intra prediction direction indices ranging from 0 to 33 and three regular scan patterns with 0 for zigzag scan, 1 for horizontal and 2 for vertical.

Table 1: Mapping table between intra prediction direction indice and scan patterns

| Index Mode TU | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32x32 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 16x16 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 8x8 | 1 | 2 | 0 | 0 | 1 | 1 | 0 | 2 | 2 | 0 | 0 | 1 | 1 | 0 | 0 | 2 | 2 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 2 | 2 | 2 | 2 | 0 | 0 |
| 4x4 | 1 | 2 | 0 | 0 | 1 | 1 | 0 | 2 | 2 | 0 | 0 | 1 | 1 | 0 | 0 | 2 | 2 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 2 | 2 | 2 | 2 | 0 | 0 |

However, in the current HM3.0, only zigzag coding scan is used for large, such as 16x16 and 32x32, TU blocks in intra coding. In this contribution, two new coding scans, namely new horizontal and new vertical scans, for large, such as 16x16 and 32x32, TU blocks in intra coding, are proposed to further improve coding efficiency. At the same time, three scans, namely zigzag, horizontal and vertical, are used to scan sign and level for high efficiency case.

- The Proposed Adaptive Scan

The new horizontal and vertical scans are illustrated in Fig. 1-Fig.2, where a current block of NxN, N can be either 16 or 32, is partitioned into four quadrants.

Fig. 1. New horizontal (left) and vertical (right) scans for 16x16 Block

Fig. 2. New horizontal (left) and vertical (right) scans for 32x32 block

The new horizontal and vertical scans in Fig. 1-2 can be used for coding the coefficients of a TU block of 16x16 or 32x32. The current HM supports two kinds of configurations. One is low complexity configuration and the other is high efficiency configuration.

Coefficient Coding for Low Complexity Configuration

In low complexity configuration, we propose that the quantized coefficients of a TU block of 16x16 or 32x32 in intra mode are coded along with one of three scans, namely zigzag, new horizontal or new vertical scan, inversely depending upon intra prediction direction associated with the TU block.

Table 2 shows a mapping between intra prediction directions and scan patterns for 16x16 and 32x32 TU blocks in intra mode, where intra prediction directions are represented by indices ranging from 0,1,2,…,33, and scan patterns are represented by 0 for zigzag scan, by 1 for regular horizontal, by 2 for regular vertical, by 1’ for new horizontal scan, and by 2’ for new vertical scan.

Table 2: Mapping table between intra prediction directions and scan patterns for 16x16 and 32x32 TU blocks in intra mode.

| Index Mode TU | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32x32 | 1’ | 2’ | 0 | 0 | 1’ | 1’ | 0 | 2’ | 2’ | 0 | 0 | 1’ | 1’ | 0 | 0 | 2’ | 2’ | 0 | 0 | 0 | 1’ | 1’ | 1’ | 1’ | 0 | 0 | 0 | 0 | 2’ | 2’ | 2’ | 2’ | 0 | 0 |
| 16x16 | 1’ | 2’ | 0 | 0 | 1’ | 1’ | 0 | 2’ | 2’ | 0 | 0 | 1’ | 1’ | 0 | 0 | 2’ | 2’ | 0 | 0 | 0 | 1’ | 1’ | 1’ | 1’ | 0 | 0 | 0 | 0 | 2’ | 2’ | 2’ | 2’ | 0 | 0 |
| 8x8 | 1 | 2 | 0 | 0 | 1 | 1 | 0 | 2 | 2 | 0 | 0 | 1 | 1 | 0 | 0 | 2 | 2 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 2 | 2 | 2 | 2 | 0 | 0 |
| 4x4 | 1 | 2 | 0 | 0 | 1 | 1 | 0 | 2 | 2 | 0 | 0 | 1 | 1 | 0 | 0 | 2 | 2 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 2 | 2 | 2 | 2 | 0 | 0 |

Coefficient Coding for High Efficiency Configuration

In high efficiency configuration, the quantized coefficients of a TU are coded in three parts. They are significant map, levels, and signs.

The significant map of a current TU is of the same size as the TU. We propose that the elements in significant map of a TU block of 16x16 or 32x32 in intra node are coded along with one of three scans, namely zigzag, new horizontal, and new vertical scan, depending upon the intra prediction direction associated with the TU block of 16x16 or 32zx32. Table 2 shows a mapping between intra prediction directions and scan patterns for significant map.

In HM3.0, level and sign coding of a TU block is based on 4x4 block. If a TU is larger than 4x4, it is partitioned as multiple 4x4 blocks. The multiple 4x4 blocks within a TU are processed along the zigzag scan pattern, and within each 4x4 block, the coefficient levels and signs are also coded along the zigzag scan.

We propose use of three scan patterns, namely zigzag, horizontal, and vertical scans, in inverse direction for coefficient level and sign coding within each 4x4 block. Specifically, given a coding block of 4x4, the scan for coding coefficient levels and signs is selected based upon the intra prediction direction associated with the TU block of NxN, where N can be 4, 8, 16, and 32.

Table 3 shows a possible mapping between intra prediction directions and scan patterns selection. Note that intra prediction direction is refereed to the block associated with the TU block of NxN while the scan patterns are for the coding order of coefficient levels and signs within a coding block of 4x4, and for the processing order of multiple blocks of 4x4 within a TU of 4x4.

Table 3: Mapping table for intra prediction directions and scans patterns, where intra prediction directions are indexed from 0 to 33, and scan patterns are represented by 0 for zigzag, 1 for horizontal, and 2 for vertical

| Index Mode TU | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32x32 | 1 | 2 | 0 | 0 | 1 | 1 | 0 | 2 | 2 | 0 | 0 | 1 | 1 | 0 | 0 | 2 | 2 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 2 | 2 | 2 | 2 | 0 | 0 |
| 16x16 | 1 | 2 | 0 | 0 | 1 | 1 | 0 | 2 | 2 | 0 | 0 | 1 | 1 | 0 | 0 | 2 | 2 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 2 | 2 | 2 | 2 | 0 | 0 |
| 8x8 | 1 | 2 | 0 | 0 | 1 | 1 | 0 | 2 | 2 | 0 | 0 | 1 | 1 | 0 | 0 | 2 | 2 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 2 | 2 | 2 | 2 | 0 | 0 |
| 4x4 | 1 | 2 | 0 | 0 | 1 | 1 | 0 | 2 | 2 | 0 | 0 | 1 | 1 | 0 | 0 | 2 | 2 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 2 | 2 | 2 | 2 | 0 | 0 |

We also propose use of three scan patterns, namely zigzag, horizontal, and vertical scans, for processing the multiple blocks of 4x4 within a TU of NxN. Specifically, the processing order of the multiple blocks of 4x4 within a TU block of NxN is selected based upon the intra prediction direction associated with the TU of NxN.

For example, within a 8x8 TU, there are four 4x4 blocks. The four 4x4 blocks may be coded along either zigzag, horizontal, or vertical scan patterns, as shown in Fig. 4, depending upon the intra prediction direction associated the 8x8 TU block according to Table 3.

Similarly, a 16x16 TU block is divided into 16 4x4 blocks. The 16 4x4 blocks may be coded along either zigzag, horizontal, or vertical scan pattern, as shown in Fig. 5, depending upon the intra prediction direction associated with the 16x16 TU block according to Table 3.

A 32x32 TU block can be divided into 64 4x4 blocks. The 64 4x4 blocks may be coded along either zigzag, horizontal, or vertical scan pattern, as shown in Fig. 6, depending upon the intra prediction direction associated with the 32x32 TU block according to Table 3.

Fig. 4. The coding order for 4x4 blocks within a 8x8 TU block. Left: zigzag scan. Center: horizontal scan. Right: vertical scan. Here, each box represents a 4x4 block.

Figure 5. The coding order for 4x4 blocks within a 16x16 TU block. Left: block zigzag scan. Center: horizontal scan. Right: vertical scan. Here, each box represents a 4x4 block.

Figure 6. The coding order for 4x4 blocks for a 32x32 TU block. Left: zigzag scan. Center: horizontal scan. Right: vertical scan. Here, each box represents a 4x4 block.

# The Simulation Results

The proposed method was implemented in the HM-3.0 and tested to evaluate its coding performance and complexity. The binaries were compiled using Microsoft Visual Studio 2010 64 bits. The coding conditions used for the simulations are specified in [3]. YUV file output during the decoding was disabled when collecting the decoding time statistics. Experimental results for intra coding are given in Table 4.

From Table 4, we can see that average coding gains of 0.3% and 0.2% can be achieved for low complexity and high efficiency cases, respectively.

Table 4: Coding performance and complexity of the proposed method

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | -0.5 | -0.3 | -0.4 | -1.0 | -0.8 |
| Class B | -0.2 | -0.6 | -0.6 | -0.3 | -0.4 | -0.4 |
| Class C | -0.2 | -0.4 | -0.4 | -0.1 | -0.1 | -0.1 |
| Class D | -0.2 | -0.4 | -0.4 | -0.1 | -0.2 | -0.1 |
| Class E | -0.5 | -1.0 | -1.1 | -0.4 | -0.6 | -0.3 |
| All | -0.2 | -0.6 | -0.5 | -0.3 | -0.5 | -0.3 |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 96% |  |  | 101% |  |  |

# Conclusions

# In this contribution, an adaptive scan method for large blocks is proposed. Without increasing complexity at both encoder and decoder, average coding gains of 0.3% and 0.2% are achieved for low complexity and high efficiency intra coding cases, respectively.

# References

[1] “HEVC Specification Working Draft 2,” JCT-VC document, JCTVC-D503, Jan. 2011.

[2] Y. Zheng, M. Coban, X. Wang, J. Sole, R. Joshi, M. Karczewicz, “CE11: Mode Dependent Coefficient Scanning,” JCT-VC document, JCTVC-D393, Jan. 2011.

[3] Frank Bossen, “Common test conditions and software reference configurations,” JCT-VC document, JCTVC-D600, Jan. 2011.

# Patent rights declaration(s)

Motorola Mobility Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).