| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F573-r2 |
| --- | --- |

| Title: | The Construction of Combined List for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shuy Fang Yue Yu Limin Wang | Tel: Email: | +1-858-404-2723 x.fang@motorola.com |
| Source: | Motorola Mobility |  |  |

_____________________________

# Abstract

This contribution proposes the construction of combined list to the new video coding standard, HEVC (High Efficiency Video Coding). Without increasing any complexity, it is more efficient if there are multiple consecutive non-reference pictures.

# Introduction

In the HEVC [1], there are two possible prediction modes for inter partition in the generalized B slices. One is bi-directional prediction,  Pred_BI, with two reference lists of list 0 and list 1 and the other is uni-directional prediction, Pred_LC, with a single combined reference list. The syntax element, inter_pred_flag at the PU level, indicates which mode is used. If inter_pred_flag is set to 1, bi-directional prediction with two reference lists (Pred_BI) is used. If inter_pred_flag is set to 0, uni-directional prediction (Pred_LC) with a single combined list is used.

In the JCTVC-D421 [2], a construction of combined list is proposed. It is done by interleaving the entries of list 0 and list 1 in ascending order of the indices, beginning with the smallest index of list 0.  Indices pointing to the reference pictures that have already been included in the combined list will be skipped.

Fig. 1 illustrates a current picture and its reference pictures in the past (ref1 and ref 2) and in the future (ref 4 and ref 5). Fig. 2 shows the mapping between two reference lists and one combined reference list

Figure 1: A current picture and its reference pictures in output picture order

Figure 2: Mapping between the combined reference list and two reference lists, list 0 and list 1.

The combined list in the JCTVC-D421 may work fine if there is only one non-reference B between two reference pictures. However, if there are more than one non-reference B pictures between two reference pictures, the combined reference list proposed in JCT-VC-D421 may not be optimal.

# The Proposed Default Combined List

In this contribution, we propose a generalized construction of combined list to further improve the coding efficiency, especially for several consecutive non-reference pictures case.

It is proposed that given a current picture,

- the reference picture with the shortest temporal distance from the current picture is assigned reference index 0, with the second shortest temporal distance from the current picture is assigned reference index 1, and so on. In other words, the reference pictures with shorter temporal distances from the current picture are assigned smaller reference indices. The rational for this index assignment is that it is likely that the blocks in a current picture refer to close reference pictures more than distant reference pictures.

- if two reference pictures in the combined reference list have the same temporal distance from the current picture, the one with a smaller average QP will be assigned a smaller reference index.

- if two reference pictures have the same temporal distance from the current picture, and the same average QPs, the one in the past will be assigned a smaller reference index.

Fig. 3. A sequence of pictures where two non-reference pictures between reference pictures. Yellow pictures are non-reference and gray pictures are reference pictures.

Fig. 3 shows an example of a sequence of pictures in display order, where the gray pictures are reference pictures and yellow pictures are non-reference pictures. In the example of Fig. 3, both Current 0 and Current 1 have four reference pictures, ref0, ref1, ref2 and ref3.

If two reference lists are used, the reference indices for list 0 and list 1 are the same for Current 0 and Current 1, as shown in Fig. 4 and 5, respectively.

list0[0]=ref0list0[1]=ref1list0[2]=ref2list0[3]=ref3

Fig. 4. Reference indices for ref0, ref1, ref2 and ref3 in list 0.

list1[0]=ref2list1[1]=ref3list1[2]=ref0list1[3]=ref1

Fig. 5. Reference indices for ref0, ref1, ref2 and ref3 in list 1.

If the combined list is used, Current 0 and Current 1 will have different lists because the combined list is constructed according to the temporal distances between a current picture and its reference pictures.

For current picture, Current 0, ref0 is assigned index 0 as it is closest to Current 0. ref2 is assigned index 1 as it is the second closest picture to Current 0. ref1 and ref3 are assigned indices 2 and 3 because ref1 is closer to Current 0 than ref3. The proposed construction of combined list for Current 0 picture is shown in  Fig. 6.

listcurrent0[0]=ref0listcurrent0[1]=ref2listcurrent0[2]=ref1listcurrent0[3]=ref3

Fig. 6. The proposed construction of combined list for Current 0

For current picture, Current 1, ref2 is assigned index 0 as it is closest to Current 1. ref0 is assigned index 1 as it is the second closest picture to Current 0. ref3 and ref1 are assigned indices 2 and 3 because ref3 is closer to Current 0 than ref1. The proposed construction of combined list for Current 1 picture is shown in  Fig. 7.

listcurrent1[0]=ref2listcurrent1[1]=ref0listcurrent1[2]=ref3listcurrent1[3]=ref1

Fig. 7. The proposed construction of combined list for Current 1

Fig. 8. A sequence of pictures of random access case.

Fig. 8 shows another example of a sequence of pictures. It is in a GOP structure used for random access common test condition in HM3.0. The GOP shown in Fig. 8 is of a hierarchical structure, where there are four coding levels. The red and the gray pictures are in the first coding level; the red one may be a I picture. The green picture is in the second coding level. The blue pictures are in the third coding level and the purple pictures are in the fourth coding level. The QPs for pictures in this GOP structure may use a hierarchical structure as well, which means that the higher the coding level is, the bigger the QP value is. The QP values for pictures in the same coding level may maintain the same.

Let QP[i] be the average QP value for picture i, where i is in the display order. The QP values for the pictures in the GOP in the example of Fig 8 may be as follows,

QP[0] is for picture 0, which may be I picture

QP[8] = QP[0] +1, if QP[0] is a I picture

QP[4] = QP[8] +1

QP[2] = QP[6] = QP[4] +1

QP[1] = QP[3] = QP[5] = QP[7] = QP[2] +1

Fig. 9. An example of assignment of QP values for random access test condition in the HM3.0.

In the example of Fig. 8, picture 2 and picture 4 are both references pictures for picture 3. The distance between picture 2 and picture 3 is the same as between picture 4 and picture 3. Since the average QP of picture 4 is smaller than that of picture 2, picture 4 will be assigned a smaller reference index than picture 2 in the combined reference list. For example, “0” is for picture 4 and “1” for picture 2.

Similarly, pictures 0 and 6 may also be reference pictures for picture 3. The distances from picture 0 and picture 6 to picture 3 are the same. Since the average QP of picture 0 is smaller than that of picture 6, picture 0 will be assigned a smaller reference index than picture 6 in the combined reference list. For example, “2” may be assigned to picture 0 and “3” to picture 6.

# The Simulation Results

The proposed method was implemented in the HM-3.0 and tested to evaluate its coding performance and complexity. The binaries were compiled using Microsoft Visual Studio 2010 64 bits. However, the coding conditions specified in [3] only can generate one non-reference picture between two reference pictures for random access case. And the current code cannot support to generate two non-reference pictures. Therefore, we make some changes to the code so that it can generate three consecutive non-reference pictures as shown in the Figure 9.

Fig. 10. A sequence of pictures where three non-reference pictures between reference pictures. Yellow pictures are non-reference and gray pictures are reference pictures.

In order to have a fair comparison, the QP for all reference pictures except I frame will be same. The QP for all non-reference pictures are also set as the same. The relationship among QP for I frame, reference pictures and non-reference pictures are as follows.

QP_reference = QP_I + 1

QP_non_reference = QP_I + 2

According to JCTVC-D421, the combined list for all three non-reference pictures, Current 0, Current 1, Current 2, will be same. It is {Ref0, Ref2, Ref1, Ref 3}. According to our proposed method, the combined list for Current 0, Current 1 still are{Ref0, Ref2, Ref1, Ref 3}. However, the combined list for Current 2 will be {Ref2, Ref0, Ref3, Ref1}. For this example, the construction of combined list for 1/3 non-reference pictures will be impacted.

We will compare our proposed construction of combined list to JCTVC-D421 for Fig. 10 configuration. Experimental results for random access cases are given in Table 1. From Table 1, we can see that average coding gains of 0.1% and 0.0% can be achieved for low complexity and high efficiency cases, respectively. More gains are expected if there are two non-reference pictures case.

Table 1: Coding performance and complexity of the proposed method

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | -0.1 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class C | -0.1 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class D | -0.1 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | -0.1 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 89% |  |  |
| Dec Time[%] | 99% |  |  | 90% |  |  |

# Conclusions

In this contribution, a new construction of combined list is proposed. Without increasing any complexity, it is more efficient for the proposed method.

# References

[1] “HEVC Specification Working Draft 2,” JCT-VC document, JCTVC-D503, Jan. 2011.

[2]Yoshinori Suzuki, TK Tan, Wei Jung Chien, Ying Chen, Peisong Chen and Muhammed Coban, “Extension of uni-prediction simplification in B slices”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D421, 4rd Meeting: Daegu, Korea, 20-28 January, 2011.

[3] Frank Bossen, “Common test conditions and software reference configurations,” JCT-VC document, JCTVC-D600, Jan. 2011.

# Patent rights declaration(s)

Motorola Mobility Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).