| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F575 |
| --- | --- |

| Title: | Simplification of MVP Design for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Yu Krit Panusopone Limin Wang | Tel: Email: | +1-858-404-2251 yyu@motorola.com |
| Source: | Motorola Mobility |  |  |

_____________________________

# Abstract

This contribution proposes a simplification of MVP design for HEVC (High Efficiency Video Coding). Simulation results show that there is very minor loss or no loss compared to original AMVP while the complexity of the proposed method is reduced by half as compared to the MVP selection procedure of AMVP.

# Introduction

In the HEVC [1], motion vectors (MV) are predictive coded. For a current PU of one MV and the associated reference index, MV predictor (MVP) is derived from MVs of the spatial neighboring or temporal collocated PUs of the current PU. The difference between a current MV and its MVP is then be coded. In the JCTVC-A124 . AMVP proposed to select the best MVP for a current MV from a candidate set of MVPs. In [2], the candidate set of MVPs for MV of a current PU is composed of MVs of spatial neighboring and collocated PUs. The candidate set of MVPs is reduced by removing the duplicated MVP candidates. An index indicating the selected MVP candidate, mvpIdx, may be transmitted in the bitstream, if necessary.

In bi-predictive mode, there are two MVs per PU, MV0 for reference list 0 and MV1 for reference list 1. The corresponding reference pictures pointed by two motion vectors, MV0 and MV1, are indexed by ref_list0 and ref_list1, respectively. To code the two MVs, two MVP candidate sets are first formed. One is for List 0 MV and the other for List 1 MV. The same procedure applies to find the best MVPs for MV0 and MV1, namely MVP_list0 and MVP_list1, from the corresponding MPV candidate sets. Up to two MPV indices, mvpIdx_listX where X can be either 0 or 1, may be transmitted.

In this contribution, a simplification of MVP design for bi-predictive mode is proposed. Simulation results show that there is very minor loss or no loss compared to original AMVP while the complexity of the proposed method is reduced by half as compared to the MVP selection procedure of AMVP.

# The Proposed Simplification of MVP

For bi-predictive mode, given a PU of two MVs, MV0 and MV1, pointing two reference pictures indexed by ref_list0 and ref_list1. MVP_list0 for MV0 is first obtained by using the same procedure as in AMVP. MVP_list1 is then calculated from either MV0 or MPV_list0.

First, the MVP candidate set for MV0 is constructed as in AMVP [2]. The best MVP candidate, MVP_list0, for MV0 is then selected from the MVP candidate set. The corresponding index for the selected MVP candidate, mvpIdx_list0, is coded in the bitstream, if it is necessary.

Then, MVP_List1 for MV1 is calculated from either MV0 or MVP_list0, instead of selecting from the candidate set as in AMVP.

Let TR1 be the temporal distance between the current picture and the reference picture pointed by MV0, and TR2 are the temporal distance between the current picture and the reference picture pointed by MV1.

We propose the MVP for MV1 is calculated from MV0. A specific example is as follows,

MVP_list1 = (sign) (TR2/TR1) *MV0                                                	(1)

where if MV0 and MV1 both point to the past or the future, sign is positive (+), otherwise, sign is negative (-).

Fig. 1 shows an example where both MV0 and MV1 point to the past reference pictures, and TR1 is an interval of 4 pictures and TR2 is an interval of 8 pictures. The MVP for MV1 is therefore equal to

MVP_list1 = (TR2/TR1)*MV0 = (8/4)*MV0 = MV0 << 1              		(2)

Figure 1. An example of proposed scaled MVP

Fig. 2 shows an example, where MV0 points to a past reference picture and MV1 to a future reference picture, and TR1 is an interval of 4 pictures and TR2 is an interval of 8 pictures. The MVP for MV1 is therefore equal to

MVP_list1 =  - (TR2/TR1)*MV0 = -(8/4) *MV0 = -(MV0 << 1)        (3)

Figure 2. Another example of proposed scaled MVP

Since MVP_list0 is available at both encoder and decoder, MVP_list0, instead of MV0, can also be used in calculating MVP_list1. In case MVP_list0 is used, the MVP for MV1 is calculated as

MVP_list1 = (sign) (TR2/TR1) *MVP_list0                                       (4)

where TR1 is the temporal distance between the current picture and the reference picture pointed by MV0, and TR2 are the temporal distance between the current picture and the reference picture pointed by MV1, and sign is positive (+) if MVP_list0 and MV1 both point to the past or the future, otherwise, sign is negative (-).

In this contribution, MVP_list1 is calculated from either MV0 or MVP_list0, instead of from a MVP candidate set as using in AMVP. Therefore, there is no overhead required for coding and sending any MVP index, mvpIdx_list1, as in AMVP, resulting in a saving in bits. In addition, there is no need to implement the MVP selection procedure used in AMVP to find MVP_list1. As a result, the complexity of the proposed method is reduced by half as compared to the MVP selection procedure of AMVP.

# The Simulation Results

The proposed method was implemented in the HM-3.0 and tested to evaluate its coding performance and complexity. The binaries were compiled using Microsoft Visual Studio 2010 64 bits. The coding conditions used for the simulations are specified in [3]. YUV file output during the decoding was disabled when collecting the decoding time statistics. Experimental results for Low delay low complexity is given in Table 1 and Table 2.

From Table 1, we can see that there is very minor loss if MV0 is used to calculate MVP_list1 and there is no any loss if MVP_list0 is used from Table 2. However, the proposed method only takes half of complexity of finding MVP comparing to original AMVP.

Table 1: Coding performance and complexity of the proposed method if MV0 is used to drive MVP_list1

|  | Low delay LC |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.2 | 0.2 | 0.0 |
| Class C | 0.1 | 0.3 | 0.0 |
| Class D | 0.0 | 0.1 | -0.2 |
| Class E | 0.1 | -0.5 | 0.4 |
| All | 0.1 | 0.1 | 0.1 |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 99% |  |  |

Table 2: Coding performance and complexity of the proposed method if MVP_list0 is used to drive MVP_list1

|  | Low delay LC |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.1 | 0.2 | -0.4 |
| Class C | 0.0 | 0.0 | -0.2 |
| Class D | 0.0 | -0.2 | -0.3 |
| Class E | 0.0 | -0.1 | -0.4 |
| All | 0.0 | 0.0 | -0.3 |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 99% |  |  |

# Conclusions

In this contribution, a simplification of MVP design for HEVC is proposed. Simulation results show that there is very minor loss while only half of complexity of original AVMP is required.

# References

[1] “HEVC Specification Working Draft 2,” JCT-VC document, JCTVC-D503, Jan. 2011.

[2] Ken McCann, Woo-Jin Han, Il-Koo Kim, Jung-Hye Min, Elena Alshina, Alexander Alshin, Tammy Lee, Jianle Chen, Vadim Seregin, Sunil Lee, Yoon-Mi Hong, Min-Su Cheon, Nikolay Shlyakhov, “Samsung’s Response to the Call for Proposals on Video Compression Technology,” JCTVC-A124, April, 2010.

[3] Frank Bossen, “Common test conditions and software reference configurations,” JCT-VC document, JCTVC-D600, Jan. 2011.

# Patent rights declaration(s)

Motorola Mobility Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).