| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F576 |
| --- | --- |

| Title: | CE3: Slice-type based adaptive interpolation filter tests by Motorola Mobility |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian Lou Koohyar Minoo David Baylon Krit Panusopone Limin Wang Ajay Luthra 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-3427 lou@motorola.com +1-858-404-2521 kminoo@motorola.com +1-858-404-2285 david.baylon@motorola.com +1-858-404-3618 krit@motorola.com +1-858-404-3893 limin.wang@motorola.com +1-858-404-3470 aluthra@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

This document reports the results of Motorola Mobility’s slice-type based adaptive interpolation filters for HEVC. The simulations were conducted using HM3.0 software with Motorola Mobility’s modifications. 8-tap quarter-pel filter is used for B-slice interpolation and 7-tap quarter-pel filter is used for P-slice interpolation. The same 8-tap half-pel filter as the HM3.0 is used both for B-slice and P-slice. Compared with the current interpolation filter in HM3.0, the proposed scheme achieves -0.1%, -0.3%, 0.0%, -0.3%, -0.9% and -1.4% bitrate differences in RAHE, RALC, LBHE, LBLC, LPHE and LPLC settings. Cross-check will be provided by Samsung. The attached spreadsheet contains detailed data of the results.

# Introduction

The target of this document is to verify the performance of Motorola Mobility’s 6-bit slice-type adaptive interpolation filter for HEVC. The simulations were conducted using HM3.0 software [1] with Motorola Mobility’s modifications. Cross-check will be provided by Samsung [2].

Fig. 1 illustrates the positions of the resulting half-pel and quarter-pel pixels between full-pel pixels along a pixel line within an image, where L3, L2, L1, L0, R0, R1, R2, and R3 are full-pel pixels, H is a half-pel pixel between L0 and R0, FL is a sub-pel pixel between L0 and H, and FR is a sub-pel pixel between H and R0.

Fig. 1 Sub-Pel Positions

# Proposed sub-pel interpolation filters

## 8-tap half-pel with 8-tap quarter-pel filter for B-slice

The half-pel interpolation filter is the same as the anchor. 3 multiplications and 7 additions, or 9 additions and 4 shifts are needed for each half-pel interpolation. An 8-tap FIR filter with 6-bit coefficients is proposed for the quarter-pel interpolation. 4 multiplications and 8 additions, or 10 additions and 3 shifts are needed for each quarter-pel interpolation.

FL = (-1*L3 + 3*L2 - 8*L1 + 60*L0 + 14*R0 - 6*R1 + 3*R2 - 1*R3 + 32) >> 6;

H = (-1*L3 + 4*L2 - 11*L1 + 40*L0 + 40*R0 - 11*R1 + 4*R2 - 1*R3 + 32) >> 6;

FR = (-1*L3 + 3*L2 - 6*L1 + 14*L0 + 60*R0 - 8*R1 + 3*R2 - 1*R3 + 32) >> 6;

A mathematically equivalent representation of the FL calculation using only addition and shift operations is

T1 = L1+R2;

T2 = L1-R0;

T3 = R1-R3;

T4 = T1-(T3<<1);

FL = ((-L3+((T4-L0)<<2)-T4-(T2<<3)-R3+32)>>6)+L0

## 8-tap Half-pel with 7-tap Quarter-pel filter for P-slice

The half-pel interpolation filter is the same as the anchor. 3 multiplications and 7 additions, or 9 additions and 4 shifts are needed for each half-pel interpolation. A 7-tap FIR filter with 6-bit coefficients is proposed for the quarter-pel interpolation. 5 multiplications and 6 additions, or 9 additions and 3 shifts are needed for each quarter-pel interpolation.

FL = (-1*L3 + 3*L2 - 8*L1 + 60*L0 + 13*R0 - 4*R1 + 1*R2 - 0*R3 + 32) >> 6;

H = (-1*L3 + 4*L2 - 11*L1 + 40*L0 + 40*R0 - 11*R1 + 4*R2 - 1*R3 + 32) >> 6;

FR = (0*L3 + 1*L2 - 4*L1 + 13*L0 + 60*R0 - 8*R1 + 3*R2 - 1*R3 + 32) >> 6;

A mathematically equivalent representation of the H calculation using only addition and shift operations is

T1 = L2+R2;

T2 = L1+R1;

T3 = L0+R0;

FL = ((T1<<1)+((T3-T2)<<3)-T2-T3+(T3<<5)+32)>>6;

A mathematically equivalent representation of the FL calculation using only addition and shift operations is

T1 = L1-R0;

T2 = L0+R1;

FL = ((-L3+((T1-T2)<<2)-T1-(L1<<3)+(R0<<4)+R2+32)>>6)+L0

Table 1 summarizes the filter coefficients.

Table 1 Sub-pel pixel interpolation filter coefficients

|  | Position | Coefficients |
| --- | --- | --- |
| B-slice | FL | { -1, 3, -8, 60, 14, -6, 3, -1,} |
|  | H | { -1, 4, -11, 40, 40, -11, 4, -1,} |
|  | FR | { -1, 3, -6, 14, 60, -8, 3, -1,} |
| P-slice | FL | { -1, 3, -8, 60, 13, -4, 1, 0,} |
|  | H | { -1, 4, -11, 40, 40, -11, 4, -1,} |
|  | FR | { 0, 1, -4, 13, 60, -8, 3, -1,} |

# Simulation environment

## Software

The software used to run the simulations is HM3.0. The binaries were compiled using Microsoft Visual Studio 2008 64bit. The coding conditions used for the simulations are specified in [3]. YUV file output during the decoding was disabled when collecting the decoding time statistics.

## Hardware

All the simulation experiments were conducted on the PCs with identical settings, i.e., Microsoft Windows 7 64bits, Intel Xeon 3.2GHz with 12G memory. Therefore, both the encoding time and decoding time comparisons are reliable.

# Simulation Results

The encoder reconstructed YUV files match the decoder reconstructed YUV files. The experimental results of the rate-distortion and speed comparison are summarized in Table 2. Detailed results are included in the attachment (JCTVC-F576.xls).

Table 2 Rate-distortion and speed performance comparison of slice-type adaptive interpolation filter

|  | Random access |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | 0.3 | -0.1 | 0.3 | 0.6 |
| Class B | -0.1 | 0.1 | 0.1 | -0.3 | 0.1 | 0.1 |
| Class C | -0.2 | 0.2 | 0.1 | -0.4 | 0.3 | 0.3 |
| Class D | -0.2 | 0.1 | 0.0 | -0.5 | 0.1 | 0.1 |
| Class E |  |  |  |  |  |  |
| All | -0.1 | 0.1 | 0.1 | -0.3 | 0.2 | 0.2 |
| Enc Time[%] | 98% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay |  |  | Low delayLC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.1 | 0.9 | 0.8 | -0.2 | 0.3 | -0.1 |
| Class C | 0.0 | 1.4 | 1.4 | -0.5 | 1.4 | 1.3 |
| Class D | -0.2 | 3.0 | 3.7 | -0.7 | 1.6 | 1.7 |
| Class E | 0.3 | 0.2 | 1.0 | 0.2 | -0.8 | -0.5 |
| All | 0.0 | 1.4 | 1.7 | -0.3 | 0.7 | 0.6 |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay P |  |  | Low delay LCP |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.7 | 0.1 | -0.3 | -1.5 | -0.2 | -0.3 |
| Class C | -0.9 | 0.7 | 0.9 | -0.7 | 0.9 | 0.9 |
| Class D | -0.7 | 2.5 | 2.5 | -0.6 | 1.5 | 1.3 |
| Class E | -1.3 | -0.6 | -0.8 | -3.4 | -2.8 | -1.9 |
| All | -0.9 | 0.7 | 0.6 | -1.4 | 0.0 | 0.1 |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |

# Complexity

Table 3 and 4 give the complexity analysis of the proposed filters for B-slice and P-slice.

Table 3 Complexity analysis of 8H+8Q interpolation filter for B-slice

| Method |  |  |  | DCTIF 8-tap | Proposal |  | DCTIF 8-tap | Proposal |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| add/shift or multiple |  |  |  | add/shift | add/shift |  | multiple | multiple |  |
| Sheet |  |  |  | Anchor | Proposal_B | Ratio (%) | Anchor | Proposal_B | Ratio (%) |
| Single | 4x4 | Operations | Worst case / pixel | 60.00 | 48.75 | 81% | 48.75 | 45.00 | 92% |
|  |  |  | Average of operations / pixel | 39.75 | 34.45 | 87% | 31.80 | 30.03 | 94% |
|  |  | Bandwith | Worst case / pixel | 7.56 | 7.56 | 100% | 7.56 | 7.56 | 100% |
|  |  |  | Average of bytes / pixel | 5.35 | 5.35 | 100% | 5.35 | 5.35 | 100% |
| Single | 8x8 | Operations | Worst case / pixel | 46.00 | 37.38 | 81% | 37.38 | 34.50 | 92% |
|  |  |  | Average of operations / pixel | 31.88 | 27.63 | 87% | 25.50 | 24.08 | 94% |
|  |  | Bandwith | Worst case / pixel | 3.52 | 3.52 | 100% | 3.52 | 3.52 | 100% |
|  |  |  | Average of bytes / pixel | 2.74 | 2.74 | 100% | 2.74 | 2.74 | 100% |
| Single | 16x16 | Operations | Worst case / pixel | 39.00 | 31.69 | 81% | 31.69 | 29.25 | 92% |
|  |  |  | Average of operations / pixel | 27.94 | 24.21 | 87% | 22.35 | 21.11 | 94% |
|  |  | Bandwith | Worst case / pixel | 2.07 | 2.07 | 100% | 2.07 | 2.07 | 100% |
|  |  |  | Average of bytes / pixel | 1.76 | 1.76 | 100% | 1.76 | 1.76 | 100% |
| Single | 32x32 | Operations | Worst case / pixel | 35.50 | 28.84 | 81% | 28.84 | 26.63 | 92% |
|  |  |  | Average of operations / pixel | 25.97 | 22.51 | 87% | 20.78 | 19.62 | 94% |
|  |  | Bandwith | Worst case / pixel | 1.49 | 1.49 | 100% | 1.49 | 1.49 | 100% |
|  |  |  | Average of bytes / pixel | 1.36 | 1.36 | 100% | 1.36 | 1.36 | 100% |
| Single | 64x64 | Operations | Worst case / pixel | 33.75 | 27.42 | 81% | 27.42 | 25.31 | 92% |
|  |  |  | Average of operations / pixel | 24.98 | 21.65 | 87% | 19.99 | 18.88 | 94% |
|  |  | Bandwith | Worst case / pixel | 1.23 | 1.23 | 100% | 1.23 | 1.23 | 100% |
|  |  |  | Average of bytes / pixel | 1.17 | 1.17 | 100% | 1.17 | 1.17 | 100% |
| Bi | 4x4 | Operations | Worst case / pixel | 120.00 | 97.50 | 81% | 97.50 | 90.00 | 92% |
|  |  |  | Average of operations / pixel | 79.50 | 68.90 | 87% | 63.60 | 60.07 | 94% |
|  |  | Bandwith | Worst case / pixel | 15.13 | 15.13 | 100% | 15.13 | 15.13 | 100% |
|  |  |  | Average of bytes / pixel | 10.70 | 10.70 | 100% | 10.70 | 10.70 | 100% |
| Bi | 8x8 | Operations | Worst case / pixel | 92.00 | 74.75 | 81% | 74.75 | 69.00 | 92% |
|  |  |  | Average of operations / pixel | 63.75 | 55.25 | 87% | 51.00 | 48.17 | 94% |
|  |  | Bandwith | Worst case / pixel | 7.03 | 7.03 | 100% | 7.03 | 7.03 | 100% |
|  |  |  | Average of bytes / pixel | 5.49 | 5.49 | 100% | 5.49 | 5.49 | 100% |
| Bi | 16x16 | Operations | Worst case / pixel | 78.00 | 63.38 | 81% | 63.38 | 58.50 | 92% |
|  |  |  | Average of operations / pixel | 55.88 | 48.43 | 87% | 44.70 | 42.22 | 94% |
|  |  | Bandwith | Worst case / pixel | 4.13 | 4.13 | 100% | 4.13 | 4.13 | 100% |
|  |  |  | Average of bytes / pixel | 3.53 | 3.53 | 100% | 3.53 | 3.53 | 100% |
| Bi | 32x32 | Operations | Worst case / pixel | 71.00 | 57.69 | 81% | 57.69 | 53.25 | 92% |
|  |  |  | Average of operations / pixel | 51.94 | 45.01 | 87% | 41.55 | 39.24 | 94% |
|  |  | Bandwith | Worst case / pixel | 2.97 | 2.97 | 100% | 2.97 | 2.97 | 100% |
|  |  |  | Average of bytes / pixel | 2.71 | 2.71 | 100% | 2.71 | 2.71 | 100% |
| Bi | 64x64 | Operations | Worst case / pixel | 67.50 | 54.84 | 81% | 54.84 | 50.63 | 92% |
|  |  |  | Average of operations / pixel | 49.97 | 43.31 | 87% | 39.98 | 37.75 | 94% |
|  |  | Bandwith | Worst case / pixel | 2.46 | 2.46 | 100% | 2.46 | 2.46 | 100% |
|  |  |  | Average of bytes / pixel | 2.34 | 2.34 | 100% | 2.34 | 2.34 | 100% |
| Single |  | Operations | Worst case / pixel | 60.00 | 48.75 | 81% | 48.75 | 45.00 | 92% |
|  |  |  | Average of operations / pixel | 30.10 | 26.09 | 87% | 24.08 | 22.74 | 94% |
|  |  | Bandwith | Worst case / pixel | 7.56 | 7.56 | 100% | 7.56 | 7.56 | 100% |
|  |  |  | Average of bytes / pixel | 2.48 | 2.48 | 100% | 2.48 | 2.48 | 100% |
| Bi |  | Operations | Worst case / pixel | 120.00 | 97.50 | 81% | 97.50 | 90.00 | 92% |
|  |  |  | Average of operations / pixel | 60.21 | 52.18 | 87% | 48.17 | 45.49 | 94% |
|  |  | Bandwith | Worst case / pixel | 15.13 | 15.13 | 100% | 15.13 | 15.13 | 100% |
|  |  |  | Average of bytes / pixel | 4.95 | 4.95 | 100% | 4.95 | 4.95 | 100% |
| Total |  | Operations | Worst case / pixel | 120.00 | 97.50 | 81% | 97.50 | 90.00 | 92% |
|  |  |  | Average of operations / pixel | 45.15 | 39.13 | 87% | 36.12 | 34.12 | 94% |
|  |  | Bandwith | Worst case / pixel | 15.13 | 15.13 | 100% | 15.13 | 15.13 | 100% |
|  |  |  | Average of bytes / pixel | 3.71 | 3.71 | 100% | 3.71 | 3.71 | 100% |

Table 4 Complexity analysis of 8H+7Q interpolation filter for P-slice

| Method |  |  |  | DCTIF 8-tap | Proposal |  | DCTIF 8-tap | Proposal |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| add/shift or multiple |  |  |  | add/shift | add/shift |  | multiple | multiple |  |
| Sheet |  |  |  | Anchor | Proposal_P | Ratio (%) | Anchor | Proposal_P | Ratio (%) |
| Single | 4x4 | Operations | Worst case / pixel | 60.00 | 45.50 | 76% | 48.75 | 38.50 | 79% |
|  |  |  | Average of operations / pixel | 39.75 | 30.83 | 78% | 31.80 | 26.67 | 84% |
|  |  | Bandwith | Worst case / pixel | 7.56 | 7.56 | 100% | 7.56 | 7.56 | 100% |
|  |  |  | Average of bytes / pixel | 5.35 | 4.79 | 89% | 5.35 | 4.79 | 89% |
| Single | 8x8 | Operations | Worst case / pixel | 46.00 | 35.75 | 78% | 37.38 | 30.25 | 81% |
|  |  |  | Average of operations / pixel | 31.88 | 25.28 | 79% | 25.50 | 21.87 | 86% |
|  |  | Bandwith | Worst case / pixel | 3.52 | 3.52 | 100% | 3.52 | 3.52 | 100% |
|  |  |  | Average of bytes / pixel | 2.74 | 2.54 | 93% | 2.74 | 2.54 | 93% |
| Single | 16x16 | Operations | Worst case / pixel | 39.00 | 30.88 | 79% | 31.69 | 26.13 | 82% |
|  |  |  | Average of operations / pixel | 27.94 | 22.51 | 81% | 22.35 | 19.47 | 87% |
|  |  | Bandwith | Worst case / pixel | 2.07 | 2.07 | 100% | 2.07 | 2.07 | 100% |
|  |  |  | Average of bytes / pixel | 1.76 | 1.68 | 95% | 1.76 | 1.68 | 95% |
| Single | 32x32 | Operations | Worst case / pixel | 35.50 | 28.44 | 80% | 28.84 | 24.06 | 83% |
|  |  |  | Average of operations / pixel | 25.97 | 21.12 | 81% | 20.78 | 18.27 | 88% |
|  |  | Bandwith | Worst case / pixel | 1.49 | 1.49 | 100% | 1.49 | 1.49 | 100% |
|  |  |  | Average of bytes / pixel | 1.36 | 1.32 | 97% | 1.36 | 1.32 | 97% |
| Single | 64x64 | Operations | Worst case / pixel | 33.75 | 27.22 | 81% | 27.42 | 23.03 | 84% |
|  |  |  | Average of operations / pixel | 24.98 | 20.43 | 82% | 19.99 | 17.67 | 88% |
|  |  | Bandwith | Worst case / pixel | 1.23 | 1.23 | 100% | 1.23 | 1.23 | 100% |
|  |  |  | Average of bytes / pixel | 1.17 | 1.15 | 99% | 1.17 | 1.15 | 99% |
| Single |  | Operations | Worst case / pixel | 60.00 | 45.50 | 76% | 48.75 | 38.50 | 79% |
|  |  |  | Average of operations / pixel | 30.10 | 24.03 | 80% | 24.08 | 20.79 | 86% |
|  |  | Bandwith | Worst case / pixel | 7.56 | 7.56 | 100% | 7.56 | 7.56 | 100% |
|  |  |  | Average of bytes / pixel | 2.48 | 2.30 | 93% | 2.48 | 2.30 | 93% |
| Total |  | Operations | Worst case / pixel | 60.00 | 45.50 | 76% | 48.75 | 38.50 | 79% |
|  |  |  | Average of operations / pixel | 30.10 | 24.03 | 80% | 24.08 | 20.79 | 86% |
|  |  | Bandwith | Worst case / pixel | 7.56 | 7.56 | 100% | 7.56 | 7.56 | 100% |
|  |  |  | Average of bytes / pixel | 2.48 | 2.30 | 93% | 2.48 | 2.30 | 93% |

# References

[1] https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-3.0

[2] E. Alshina, “CE3: Cross-check by Samsung for Motorola Mobility tests on interpolation filters,” JCT-VC document, JCTVC-F443, July 2011

[3] JCT-VC, “Common test conditions and software reference configurations”, JCTVC-E700, March 2011

Motorola Mobility Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).