| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F581 |
| --- | --- |

| Title: | Cross-check report for BBC’s proposal on Transform skip mode (JCTVC-F077) by Motorola Mobility |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Krit Panusopone Xue Fang Vivian Kung Limin Wang | Tel: Email: | +1-858-404-3618 krit@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

This contribution document contains cross-check results of BBC’s transform skip mode. The cross-check has been completed successfully and the results match those provided by BBC. Attached spreadsheet contains detailed data of the results.

# Introduction

The target of this report is to verify the performance of BBC’s transform skip mode [1]. The simulations were conducted using HM3.2 software with BBC’s modifications. The anchor results are obtained with HM3.2 [2] and the tested results are obtained with the proposed transform skip mode.

# Simulation environment

## Software

The software used to run the simulations is HM3.2. The binaries were compiled using Microsoft Visual Studio 2010 64bit. The coding conditions used for the simulations are specified in [3], except for QuadtreeTUMaxDepthInter which was set to 1. Another anchor with QuadtreeTUMaxDepthInter which was set to 3 was also tested. YUV file output during the decoding was disabled when collecting the decoding time statistics.

## Hardware

The simulation experiments were conducted on the PCs with multiple settings, i.e., Microsoft Windows 7 64bits, Intel Xeon 3.2GHz with 12G memory and Microsoft Windows 7 64bits, Intel Xeon 2.8GHz with 8G memory. Therefore, both the encoding time and decoding time comparisons are not reliable.

# Simulation Results

The encoder reconstructed YUV files match the decoder reconstructed YUV files. The experimental results of the rate-distortion and speed comparison are summarized in Table 1. Detailed results are included in the attachment (JCTVC-F581-anchorHM3.2d13.xls and JCTVC-F581-anchorHM3.2d3.xls).

Table 1 Rate-distortion and speed performance comparison of

BBC’s transform skip mode (Inter RQT depth for anchor = 1)

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | 0.0 | -0.2 | 0.3 | 0.0 | 0.2 |
| Class B | -0.6 | -0.7 | -0.3 | -0.5 | -0.4 | 0.1 |
| Class C | -0.8 | -0.2 | 0.0 | -0.7 | -0.1 | 0.3 |
| Class D | -1.1 | 0.3 | 0.3 | -1.1 | 0.4 | 0.6 |
| Class E |  |  |  |  |  |  |
| All | -0.6 | -0.2 | -0.1 | -0.5 | 0.0 | 0.3 |
| Enc Time[%] | 121% |  |  | 118% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Low delay |  |  | Low delay LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.8 | -1.1 | -0.8 | -0.7 | -0.9 | -0.1 |
| Class C | -1.5 | -0.7 | -0.1 | -1.4 | -0.4 | 0.6 |
| Class D | -2.2 | 0.6 | 1.0 | -2.1 | 1.2 | 1.2 |
| Class E | -0.8 | 1.0 | 0.3 | -0.8 | 0.9 | 0.4 |
| All | -1.3 | -0.2 | 0.0 | -1.2 | 0.1 | 0.5 |
| Enc Time[%] | 117% |  |  | 114% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |

Table 2 Rate-distortion and speed performance comparison of

BBC’s transform skip mode (Inter RQT depth for anchor = 3)

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.5 | 0.6 | 0.3 | 1.1 | 0.1 | 0.4 |
| Class B | 0.2 | -0.2 | 0.2 | 0.5 | -0.9 | 0.1 |
| Class C | -0.1 | 0.1 | 0.4 | 0.2 | -0.3 | 0.4 |
| Class D | -0.4 | 0.4 | 0.5 | 0.0 | 0.2 | 0.5 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.2 | 0.3 | 0.4 | -0.3 | 0.3 |
| Enc Time[%] | 93% |  |  | 95% |  |  |
| Dec Time[%] | 90% |  |  | 90% |  |  |

|  | Low delay |  |  | Low delay LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | -0.2 | 0.9 | 0.6 | -2.4 | 0.0 |
| Class C | -0.5 | 0.0 | 0.5 | -0.1 | -0.6 | 1.0 |
| Class D | -1.2 | 1.4 | 1.6 | -0.5 | 0.9 | 1.4 |
| Class E | -0.5 | 0.5 | 0.0 | 0.0 | -0.5 | -0.9 |
| All | -0.5 | 0.5 | 0.8 | 0.0 | -0.7 | 0.4 |
| Enc Time[%] | 93% |  |  | 93% |  |  |
| Dec Time[%] | 91% |  |  | 90% |  |  |

# References

[1] M. Mrak, A. Gabriellini, N. Sprljan, D. Flynn, “Transform skip mode,” JCT-VC document, JCTVC-F077, July 2011

[2] https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-3.2

[3] JCT-VC, “Common test conditions and software reference configurations”, JCTVC-E700, March 2011

Motorola Mobility Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).