| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F584 |
| --- | --- |

| Title: | Modification to DC prediction in SDIP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongbing Lin Changcai Lai | Email: | yblin@huawei.com laichangcai@huawei.com |
| Source: | HiSilicon Technologies |  |  |

_____________________________

# Abstract

A simple change on DC prediction of SDIP PU is present with only shifting to avoid division operation. Almost no gain lost can be found according to the test results.

# Introduction

DC prediction in HM3.0 uses two sides of the reference samples when above and left reference samples are available. The prediction values are derived as following.

DCVal = x'=0nS−1p[x', −1]+y'=0nS−1p[−1, y']+nS>>(k+1), with x, y = 0..nS-1	
where k=log2(nS)

Since the sum of width and height of a SDIP block is not a form of power of 2, the above formula can’t be directly applied to SDIP blocks. It is noted that the existing calculation of DC value in HM3.0 and HM3.0-SDIP branch is based on division instead of shifting [1]. Thus the following modified DC prediction in SDIP is used, which exploits only one side of the reference samples corresponding to the longer side of current SDIP block.

DCVal = x'=0width−1p[x', −1]+width2>>m,  if width > height
DCVal = y'=0height−1p[−1, y']+height2>>n,  if width < height
where m = log2(width), n = log2(height)

# Results

The proposed modification on DC prediction in SDIP has been integrated into HM3.0-SDIP branch software. As shown in Table 1, the modification achieves almost the same R-D performance as HM3.0-SDIP [2-3].

Table-1 Performance of the simplified DC prediction in SDIP

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.2 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.1 | 0.0 | 0.1 | 0.1 |
| All | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table 2 shows the performance of all the harmonization of SDIP as described in JCTVC-F532 [4]. Table 3 further shows the performance of all the harmonization plus the proposed modification on DC prediction in SDIP.

Table-2 performance of all harmonization for SDIP as described in JCTVC-F532

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.04 | -0.53 | -0.68 | -0.41 | -1.26 | -1.25 |
| Class B | -0.19 | -0.50 | -0.49 | -0.65 | -0.98 | -1.00 |
| Class C | -0.03 | -0.15 | -0.14 | -0.35 | -0.80 | -0.81 |
| Class D | -0.04 | -0.05 | -0.09 | -0.29 | -0.56 | -0.49 |
| Class E | -0.47 | -1.59 | -1.40 | -1.20 | -2.84 | -2.37 |
| All | -0.14 | -0.51 | -0.52 | -0.55 | -1.20 | -1.12 |
| Enc Time[%] | 101% |  |  | 104% |  |  |
| Dec Time[%] | 99% |  |  | 103% |  |  |

Table-3 performance of all the harmonization plus the proposed modification

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.04 | -0.59 | -0.73 | -0.41 | -1.25 | -1.26 |
| Class B | -0.20 | -0.52 | -0.51 | -0.66 | -0.98 | -1.00 |
| Class C | -0.03 | -0.14 | -0.15 | -0.37 | -0.80 | -0.83 |
| Class D | -0.06 | -0.06 | -0.13 | -0.30 | -0.59 | -0.58 |
| Class E | -0.47 | -1.56 | -1.42 | -1.21 | -2.83 | -2.38 |
| All | -0.15 | -0.52 | -0.54 | -0.56 | -1.20 | -1.14 |
| Enc Time[%] | 101% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  |

# Conclusion

A simple change on DC prediction of SDIP PU is present with only shifting to avoid division operation. Almost no gain lost in HM-SDIP software, and a small gain can be found based on HM3.0-SDIP with all harmonization according to the test results. We suggest adopting this change into HM for purpose of avoiding division operation.

# References

- X. Cao, X. Peng, C. Lai (HiSilicon), et.al., “CE6.b1 Report on Short Distance Intra Prediction Method,” ITU-T JCTVC-E278, Geneva, CH, March 2011.

- F. Bossen, “Common test conditions and software reference configurations,” ITU-T JCTVC-E700, Geneva, CH, March 2011.

- Ali Tabatabai, Keiichi Chono, Muhammed Coban, et al. "Core Experiment 6: Intra Prediction Improvement", Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E706, 5th Meeting, Geneva, Switzerland, Mar., 2011.

- X. Cao, X. Peng, C. Lai, et al. "CE6.b Test Summary and spec text of SDIP ", Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F532, 6th Meeting, Italy, Torino, Jul., 2011.

# Patent rights declaration(s)

Huawei and Hisilicon may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).