| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F585 |
| --- | --- |

| Title: | Luma/chroma interpolation precision |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Peisong Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | +1 858 658 3973 mcoban@qualcomm.com peisongc@qualcomm.com martak@qualcomm.com |
| Source: | Qualcomm Inc |  |  |

_____________________________

# Abstract

This contribution presents reduced precision luma/chroma interpolation process with minimal impact on coding efficiency. In the current design, the intermediate values for calculating the fractional values have 17-bit range. When the dynamic range of the intermediate values are reduced to 16-bit, there is no drop in performance. Simulation results show 0.0%, 0.0%, and 0.0% BD rate change for RA_HE, LB_HE and LP_HE cases, respectively.

# Introduction

In the current WD3, the luma interpolation process for fractional pixels is described as below:

Integer samples (shaded blocks with upper-case letters) and fractional sample positions (un-shaded blocks with lower-case letters) for quarter sample luma interpolation

Fractional pel pixels labelled e0,0, f0,0, g0,0, i0,0, j0,0, k0,0, p0,0, q0,0 and r0 are derived by first calculating intermediate values:

d1i,0 =  -Ai,-3 + 4*Ai,-2 - 10*Ai,-1 + 57*Ai,0 + 19*Ai,1 - 7*Ai,2 + 3*Ai,3 - Ai,4

h1i,0 =  -Ai,-3 + 4*Ai,-2 - 11*Ai,-1 + 40*Ai,0 +	 40*Ai,1 - 11*Ai,2 + 4*Ai,3 - Ai,4

n1i,0 =  -Ai,-3 + 3*Ai,-2 - 7*Ai,-1 + 19*Ai,0 +	 57*Ai,1 - 10*Ai,2 + 4*Ai,3 - Ai,4

For 10-bit input values, the range of the intermediate values is [-24552, 90024]. If these intermediate values are shifted down by 1-bit with rounding for 10-bit input values, the filtered values have 16-bit range [-12277, 45012] that can be represented by 16-bit values if a fixed offset is applied to these values. This can be represented by

d1i,0 = ( -Ai,-3 + 4*Ai,-2 - 10*Ai,-1 + 57*Ai,0 + 19*Ai,1 - 7*Ai,2 + 3*Ai,3 - Ai,4 + offset0)  >>  shift0

h1i,0 =  (-Ai,-3 + 4*Ai,-2 - 11*Ai,-1 + 40*Ai,0 +	 40*Ai,1 - 11*Ai,2 + 4*Ai,3 - Ai,4 + offset0)  >>  shift0

n1i,0 =  (-Ai,-3 + 3*Ai,-2 - 7*Ai,-1 + 19*Ai,0 + 57*Ai,1 - 10*Ai,2 + 4*Ai,3 - Ai,4 + offset0)  >>  shift0

where shift0 = (BitDepthY – 8) ? BitDepthY – 8 – 1 : 0. The rounding offset0 = shift0 ? 1 << (shift0 -1) : 0.  Final prediction values are obtained by

e0,0 = ( -d1-3,0 + 4*d1-2,0 - 10*d1-1,0 + 57*d10,0 +	 19*d11,0 - 7*d12,0 + 3*d13,0 - d14,0 + offset2 ) >> shift2

f0,0 = ( -d1-3,0 + 4*d1-2,0 - 11*d1-1,0 + 40*d10,0 +	 40*d11,0 - 11*d12,0 + 4*d13,0 - d14,0 + offset2 ) >> shift2

g0,0 = ( -d1-3,0 + 3*d1-2,0 - 7*d1-1,0 + 19*d10,0 +	 57*d11,0 - 10*d12,0 + 4*d13,0 - d14,0 + offset2 ) >> shift2

i0,0 = ( -h1-3,0 + 4*h1-2,0 - 10*h1-1,0 + 57*h10,0 +	 19*h11,0 - 7*h12,0 + 3*h13,0 - h14,0 + offset2 ) >> shift2

j0,0 = ( -h1-3,0 + 4*h1-2,0 - 11*h1-1,0 + 40*h10,0 +	 40*h11,0 - 11*h12,0 + 4*h13,0 - h14,0 + offset2 ) >> shift2

k0,0 = ( -h1-3,0 + 3*h1-2,0 - 7*h1-1,0 + 19*h10,0 +	 57*h11,0 - 10*h12,0 + 4*h13,0 - h14,0 + offset2 ) >> shift2

p0,0 = ( -n1-3,0 + 4*n1-2,0 - 10*n1-1,0 + 57*n10,0 +	 19*n11,0 - 7*n12,0 + 3*n13,0 - n14,0 + offset2 ) >> shift2

q0,0 = ( -n1-3,0 + 4*n1-2,0 - 11*n1-1,0 + 40*n10,0 +	 40*n11,0 - 11*n12,0 + 4*n13,0 - n14,0 + offset2 ) >> shift2

r0,0 = ( -n1-3,0 + 3*n1-2,0 - 7*n1-1,0 + 19*n10,0 +	 57*n11,0 - 10*n12,0 + 4*n13,0 - n14,0 + offset2 ) >> shift2

where shift2 = (BitDepthY – 2) – shift0  and offset2 = shift2 ? 1 << (shift2-1) : 0. In the current HM3.2 software for uni-directional prediction, the shift2 value would be  shift2 = 12 – shift0 and offset2 = shift2 ? 1 << (shift2-1) : 0. In WD3[2] final stage of fractional pel unidirectional interpolation case for these samples is descibed as cascaded 2 stage down shift with rounding.

Similar process can be applied to the interpolation process for chroma pixels.  In the current WD3, the chroma interpolation process for fractional pixels is described as below:

Integer samples (shaded blocks with upper-case letters) and fractional sample positions (un-shaded blocks with lower-case letters) for eighth sample chroma interpolation

bai,0 =  -3*B0,-1 + 60*B0,0 + 8*B0,1 – B0,2

cai,0 =  -4*B0,-1 + 54*B0,0 + 16*B0,1 – 2*B0,2

dai,0 =  -5*B0,-1 + 46*B0,0 + 27*B0,1 – 4*B0,2

eai,0 =  -4*B0,-1 + 36*B0,0 + 36*B0,1 – 4*B0,2

fai,0 =  -4*B0,-1 + 27*B0,0 + 46*B0,1 – 5*B0,2

gai,0 =  -2*B0,-1 + 16*B0,0 + 54*B0,1 – 4*B0,2

hai,0 =  -B0,-1 + 8*B0,0 + 60*B0,1 – 3*B0,2

For 10-bit input values, the range of the intermediate values is [-9207, 74679]. If these intermediate values are shifted down by 1-bit with rounding for 10-bit input values, the filtered values have 16-bit range [-4603, 37340] that can be represented by 16-bit values if a fixed offset is applied to these values. This can be represented by

bai,0 = ( -3*B0,-1 + 60*B0,0 + 8*B0,1 – B0,2 + offset0)  >>  shift0

cai,0 =  (-4*B0,-1 + 54*B0,0 + 16*B0,1 – 2*B0,2 + offset0)  >>  shift0

dai,0 =  (-5*B0,-1 + 46*B0,0 + 27*B0,1 – 4*B0,2 + offset0)  >>  shift0

eai,0 =  (-4*B0,-1 + 36*B0,0 + 36*B0,1 – 4*B0,2 + offset0)  >>  shift0

fai,0 =  (-4*B0,-1 + 27*B0,0 + 46*B0,1 – 5*B0,2 + offset0)  >>  shift0

gai,0 =  (-2*B0,-1 + 16*B0,0 + 54*B0,1 – 4*B0,2 + offset0)  >>  shift0

hai,0 = ( -B0,-1 + 8*B0,0 + 60*B0,1 – 3*B0,2 + offset0)  >>  shift0

where shift0 = (BitDepthY – 8) ? BitDepthY – 8 – 1 : 0. The rounding offset0 = shift0 ? 1 << (shift0 -1) : 0.  Final prediction values are obtained by

Xb0,0 = ( -3*Xa-1,0 + 60*Xa0,0 + 8*Xa1,0 – Xa2,0 + offset2 ) >> shift2

Xc0,0 = ( -4*Xa-1,0 + 54*Xa0,0 + 16*Xa1,0 – 2*Xa2,0 + offset2 ) >> shift2

Xd0,0 = ( -5*Xa-1,0 + 46*Xa0,0 + 27*Xa1,0 – 4*Xa2,0 + offset2 ) >> shift2

Xe0,0 = ( -4*Xa-1,0 + 36*Xa0,0 + 36*Xa1,0 – 4*Xa2,0 + offset2 ) >> shift2

Xf0,0 = ( -4*Xa-1,0 + 27*Xa0,0 + 46*Xa1,0 – 5*Xa2,0 + offset2 ) >> shift2

Xg0,0 = ( -2*Xa-1,0 + 16*Xa0,0 + 54*Xa1,0 – 4*Xa2,0 + offset2 ) >> shift2

Xh0,0 = ( -Xa-1,0 + 8*Xa0,0 + 60*Xa1,0 – 3*Xa2,0 + offset2 ) >> shift2

where shift2 = (BitDepthY – 2) – shift0  and offset2 = shift2 ? 1 << (shift2-1) : 0.

# Results

The proposed luma/chroma interpolation precision reduction algorithm is implemented in HM3.0. The simulations are conducted based on the JCT-VC common test condition defined in [1]. Proposed changes only affect RA_HE, LB_HE and LP_HE configurations. Table 1 shows summary of simulation results. As can be from the results on average there is 0.0% BD rate loss.

Table 1: Summary of simulation results (BD-rate), HM30 anchor

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | -0.3 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 89% |  |  | 105% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.3 | 0.2 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.1 | 0.3 | 0.0 | 0.0 | 0.0 |
| Class E | 0.1 | 0.4 | 0.0 | 0.0 | 0.0 | 0.0 |
| Overall | 0.0 | 0.2 | 0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 89% |  |  | 108% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | -0.5 | -0.2 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.1 | -0.2 | 0.0 | 0.0 | 0.0 |
| Overall | 0.0 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 85% |  |  | 97% |  |  |

# Conclusions

This contribution has presented a reduced precision fractional luma/chroma interpolation process. The method guarantees 16-bit range for storage of intermediate values for fractional luma/chroma interpolation process without loss in coding efficiency.

# References

- F. Bossen, “Common test conditions and software reference configurations,” JCT-VC document JCTVC-E700, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 5th meeting, Geneva, March 2011.

- T. Wiegand, et al., “WD3: Working Draft 3 of High-Efficiency Video Coding,” JCT-VC document JCTVC-E603, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 5th meeting, Geneva, March 2011.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix A: Proposed modification of HEVC Working Draft 3

###### Luma sample interpolation process

Inputs to this process are:

–	a luma location in full-sample units ( xIntL, yIntL ),

–	a luma location in fractional-sample units ( xFracL, yFracL ),

–	the luma reference sample array refPicLXL.

Output of this process is a predicted luma sample value predSampleLXL[ xL, yL ]

Figure 8-1 – Integer samples (shaded blocks with upper-case letters) and fractional sample positions (un-shaded blocks with lower-case letters) for quarter sample luma interpolation

In Figure 8-1, the positions labelled with upper-case letters Ai, j within shaded blocks represent luma samples at full-sample locations inside the given two-dimensional array refPicLXL of luma samples. These samples may be used for generating the predicted luma sample value predSampleLXL[ xL, yL ]. The locations ( xAi, j, yAi, j ) for each of the corresponding luma samples Ai, j inside the given array refPicLXL of luma samples are derived as follows:

xAi, j = Clip3( 0, PicWidthInSamplesL – 1, xIntL +i )		(8-1)
yAi, j = Clip3( 0, PicHeightInSamplesL – 1, yIntL +j )		(8-2)

Variables shift0, shift1, shift2, shift3, offset0, offset1 and offset2 are derived as follows.

–	The variable shift0 is set equal to BitDepthY – 8– 1, if the value of shift0 is less than 0, the variable shift0 is set to 0, the variable shift1 is set equal to BitDepthY – 8 – shift0, the variable shift2 is set equal to BitDepthY – 2– shift0, and the variable shift3 is set equal to 14 – BitDepthY.

–	If the variable shift0 is equal to 0, the variable offset0 is set equal to 0, otherwise, the variable offset0  is set equal to 1 << ( shift0 – 1 ).

.–	If the variable shift1 is equal to 0, the variable offset1 is set equal to 0, otherwise, the variable offset1 is set equal to 1 << ( shift1 – 1 ).

–	If the variable shift2 is equal to 0, the variable offset2 is set equal to 0, otherwise, the variable offset2 is set equal to 1 << ( shift2 – 1 ).

Given the luma samples Ai, j at full-sample locations ( xAi, j, yAi, j ), the luma samples ‘a0,0’ to ‘r0,0’ at fractional sample positions are derived by the following rules.

–	The samples labelled a0,0, b0,0, c0,0, d0,0, h0,0, and n0,0 shall be derived by applying the 8-tap filter to the nearest integer position samples and clipping the filtered value:

a0,0 = ( -A-3,0 + 4*A-2,0 - 10*A-1,0 + 57*A0,0 +	 19*A1,0 - 7*A2,0 + 3*A3,0 - A4,0 + offset1 ) >> shift1	(8-3)

b0,0 = ( -A-3,0 + 4*A-2,0 - 11*A-1,0 + 40*A0,0 +	 40*A1,0 - 11*A2,0 + 4*A3,0 - A4,0 + offset1 ) >> shift1	(8-4)

c0,0 = ( -A-3,0 + 3*A-2,0 - 7*A-1,0 + 19*A0,0 +	 57*A1,0 - 10*A2,0 + 4*A3,0 - A4,0 + offset1 ) >> shift1	(8-5)

d0,0 = ( -A0,-3 + 4*A0,-2 - 10*A0,-1 + 57*A0,0 +	 19*A0,1 - 7*A0,2 + 3*A0,3 - A0,4 + offset1 ) >> shift1	(8-6)

h0,0 = ( -A0,-3 + 4*A0,-2 - 11*A0,-1 + 40*A0,0 +	 40*A0,1 - 11*A0,2 + 4*A0,3 - A0,4 + offset1 ) >> shift1	(8-7)

n0,0 = ( -A0,-3 + 3*A0,-2 - 7*A0,-1 + 19*A0,0 +	 57*A0,1 - 10*A0,2 + 4*A0,3 - A0,4 + offset1 ) >> shift1	(8-8)

–	The samples labelled e0,0, f0,0, g0,0, i0,0, j0,0, k0,0, p0,0, q0,0 and r0,0 shall be derived by first calculating intermediate values denoted as d1i,0, h1i,0 and n1i,0 where i = -3..4 by applying the 8-tap filter to the nearest integer position samples in vertical direction: [Ed: (WJ) horizontal first yields the same result]

d1i,0 =  (-Ai,-3 + 4*Ai,-2 - 10*Ai,-1 + 57*Ai,0 + 19*Ai,1 - 7*Ai,2 + 3*Ai,3 - Ai,4 + offset0)  >>  shift0	(8-9)

h1i,0 =  (-Ai,-3 + 4*Ai,-2 - 11*Ai,-1 + 40*Ai,0 +	 40*Ai,1 - 11*Ai,2 + 4*Ai,3 - Ai, 4 + offset0)  >>  shift0	(8-10)

n1i,0 =  (-Ai,-3 + 3*Ai,-2 - 7*Ai,-1 + 19*Ai,0 +	 57*Ai,1 - 10*Ai,2 + 4*Ai,3 - Ai, 4 + offset0)  >>  shift0	(8-11)

–	The final prediction values e0,0, f0,0, g0,0, i0,0, j0,0, k0,0, p0,0, q0,0 and r0,0 shall be derived by applying the 8-tap filter to the intermediate values d1i,0, h1i,0 and n1i,0 where i = -3..4 in horizontal direction:

e0,0 = ( -d1-3,0 + 4*d1-2,0 - 10*d1-1,0 + 57*d10,0 +	 19*d11,0 - 7*d12,0 + 3*d13,0 - d14,0 + offset2 ) >> shift2	(8-12)

f0,0 = ( -d1-3,0 + 4*d1-2,0 - 11*d1-1,0 + 40*d10,0 +	 40*d11,0 - 11*d12,0 + 4*d13,0 - d14,0 + offset2 ) >> shift2	(8-13)

g0,0 = ( -d1-3,0 + 3*d1-2,0 - 7*d1-1,0 + 19*d10,0 +	 57*d11,0 - 10*d12,0 + 4*d13,0 - d14,0 + offset2 ) >> shift2	(8-14)

i0,0 = ( -h1-3,0 + 4*h1-2,0 - 10*h1-1,0 + 57*h10,0 +	 19*h11,0 - 7*h12,0 + 3*h13,0 - h14,0 + offset2 ) >> shift2	(8-15)

j0,0 = ( -h1-3,0 + 4*h1-2,0 - 11*h1-1,0 + 40*h10,0 +	 40*h11,0 - 11*h12,0 + 4*h13,0 - h14,0 + offset2 ) >> shift2	(8-16)

k0,0 = ( -h1-3,0 + 3*h1-2,0 - 7*h1-1,0 + 19*h10,0 +	 57*h11,0 - 10*h12,0 + 4*h13,0 - h14,0 + offset2 ) >> shift2	(8-17)

p0,0 = ( -n1-3,0 + 4*n1-2,0 - 10*n1-1,0 + 57*n10,0 +	 19*n11,0 - 7*n12,0 + 3*n13,0 - n14,0 + offset2 ) >> shift2	(8-18)

q0,0 = ( -n1-3,0 + 4*n1-2,0 - 11*n1-1,0 + 40*n10,0 +	 40*n11,0 - 11*n12,0 + 4*n13,0 - n14,0 + offset2 ) >> shift2	(8-19)

r0,0 = ( -n1-3,0 + 3*n1-2,0 - 7*n1-1,0 + 19*n10,0 +	 57*n11,0 - 10*n12,0 + 4*n13,0 - n14,0 + offset2 ) >> shift2	(8-20)

The positions labelled with lower-case letters within un-shaded blocks represent luma samples at quarter-pel sample fractional locations. The luma location offset in fractional-sample units ( xFracL, yFracL ) specifies which of the generated luma samples at full-sample and fractional-sample locations is assigned to the predicted luma sample value predSampleLXL[ xL, yL ]. This assignment is done according to Table 8. The value of predSampleLXL[ xL, yL ] shall be the output.

Table 8-11 – Assignment of the luma prediction sample predSampleLXL[ xL, yL ]

| xFracL | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| yFracL | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 |
| predSampleLXL[ xL, yL ] | A << shift3 | d | h | n | a | e | i | p | b | f | j | q | c | g | k | r |

###### Chroma sample interpolation process

Inputs to this process are:

–	a chroma location in full-sample units ( xIntC, yIntC ),

–	a chroma location in fractional-sample units ( xFracC, yFracC ),

–	the chroma reference sample array refPicLXC.

Output of this process is a predicted chroma sample value predSampleLXC[ xC, yC ]

Figure 8-2 – Integer samples (shaded blocks with upper-case letters) and fractional sample positions (un-shaded blocks with lower-case letters) for eighth sample chroma interpolation

In Figure 8-1, the positions labelled with upper-case letters Bi, j within shaded blocks represent chroma samples at full-sample locations inside the given two-dimensional array refPicLXC of chroma samples. These samples may be used for generating the predicted chroma sample value predSampleLXC[ xC, yC ]. The locations ( xBi, j, yBi, j ) for each of the corresponding chroma samples Bi, j inside the given array refPicLXC of chroma samples are derived as follows:

xBi, j = Clip3( 0, PicWidthInSamplesC – 1, xIntC +i )		(8-21)
yBi, j = Clip3( 0, PicHeightInSamplesC – 1, yIntC +j )		(8-22)

Variables shift0, shift1, shift2, shift3, offset0, offset1 and offset2 are derived as follows.

–	The variable shift0 is set equal to BitDepthC – 8– 1, if the value of shift0 is less than 0, the variable shift0 is set to 0, the variable shift1 is set equal to BitDepthC – 8– shift0, the variable shift2 is set equal to BitDepthC – 2– shift0, and the variable shift3 is set equal to 14 – BitDepthC.

–	If the variable shift0 is equal to 0, the variable offset0 is set equal to 0, otherwise, the variable offset0  is set equal to 1 << ( shift0 – 1 ).

–	If the variable shift1 is equal to 0, the variable offset1 is set equal to 0, otherwise, the variable offset1 is set equal to 1 << ( shift1 – 1 ).

–	If the variable shift2 is equal to 0, the variable offset2 is set equal to 0, otherwise, the variable offset2 is set equal to 1 << ( shift2 – 1 ).

Given the chroma samples Bi, j at full-sample locations ( xBi, j, yBi, j ), the chroma samples ‘ab0,0’ to ‘hh0,0’ at fractional sample positions are derived by the following rules.

–	The samples labelled ab0,0, ac0,0, ad0,0, ae0,0, af0,0, ag0,0, and ah0,0 shall be derived by applying the 4-tap filter to the nearest integer position samples and clipping the filtered value:

ab0,0 = ( -3*B-1,0 + 60*B0,0 + 8*B1,0 – B2,0 + offset1 ) >> shift1	(8-23)

ac0,0 = ( -4*B-1,0 + 54*B0,0 + 16*B1,0 – 2*B2,0 + offset1 ) >> shift1	(8-24)

ad0,0 = ( -5*B-1,0 + 46*B0,0 + 27*B1,0 – 4*B2,0 + offset1 ) >> shift1	(8-25)

ae0,0 = ( -4*B-1,0 + 36*B0,0 + 36*B1,0 – 4*B2,0 + offset1 ) >> shift1	(8-26)

af0,0 = ( -4*B-1,0 + 27*B0,0 + 46*B1,0 – 5*B2,0 + offset1 ) >> shift1	(8-27)

ag0,0 = ( -2*B-1,0 + 16*B0,0 + 54*B1,0 – 4*B2,0 + offset1 ) >> shift1	(8-28)

ah0,0 = ( -B-1,0 + 8*B0,0 + 60*B1,0 – 3*B2,0 + offset1 ) >> shift1	(8-29)

–	The samples labelled ba0,0, ca0,0, da0,0, ea0,0, fa0,0, ga0,0, and ha0,0 shall be derived by applying the 4-tap filter to the nearest integer position samples and clipping the filtered value:

ba0,0 = ( -3*B0,-1 + 60*B0,0 + 8*B0,1 – B0,2 + offset1 ) >> shift1	(8-30)

ca0,0 = ( -4*B0,-1 + 54*B0,0 + 16*B0,1 – 2*B0,2 + offset1 ) >> shift1	(8-31)

da0,0 = ( -5*B0,-1 + 46*B0,0 + 27*B0,1 – 4*B0,2 + offset1 ) >> shift1	(8-32)

ea0,0 = ( -4*B0,-1 + 36*B0,0 + 36*B0,1 – 4*B0,2 + offset1 ) >> shift1	(8-33)

fa0,0 = ( -4*B0,-1 + 27*B0,0 + 46*B0,1 – 5*B0,2 + offset1 ) >> shift1	(8-34)

ga0,0 = ( -2*B0,-1 + 16*B0,0 + 54*B0,1 – 4*B0,2 + offset1 ) >> shift1	(8-35)

ha0,0 = ( -B0,-1 + 8*B0,0 + 60*B0,1 – 3*B0,2 + offset1 ) >> shift1	(8-36)

–	The samples labelled Xb0,0, Xc0,0, Xd0,0, Xe0,0, Xf0,0, Xg0,0 and Xh0,0 for X being replaced by b, c, d, e, f, g and h, respectively, shall be derived by first calculating intermediate values denoted as bai,0, cai,0, dai,0, eai,0, fai,0, gai,0 and hai,0 where i = -1..2 by applying the 4-tap filter to the nearest integer position samples in vertical direction: [Ed: (WJ) horizontal first yields the same result]

bai,0 =  (-3*B0,-1 + 60*B0,0 + 8*B0,1 – B0, 2 + offset0)  >>  shift0		(8-37)

cai,0 =  (-4*B0,-1 + 54*B0,0 + 16*B0,1 – 2*B0, 2 + offset0)  >>  shift0 		(8-38)

dai,0 =  (-5*B0,-1 + 46*B0,0 + 27*B0,1 – 4*B0,2 + offset0)  >>  shift0 		(8-39)

eai,0 =  (-4*B0,-1 + 36*B0,0 + 36*B0,1 – 4*B0,2 + offset0)  >>  shift0 		(8-40)

fai,0 =  (-4*B0,-1 + 27*B0,0 + 46*B0,1 – 5*B0,2 + offset0)  >>  shift0		(8-41)

gai,0 =  (-2*B0,-1 + 16*B0,0 + 54*B0,1 – 4*B0,2 + offset0)  >>  shift0 		(8-42)

hai,0 =  (-B0,-1 + 8*B0,0 + 60*B0,1 – 3*B0,2 + offset0)  >>  shift0 		(8-43)

–	The final prediction values Xb0,0, Xc0,0, Xd0,0, Xe0,0, Xf0,0, Xg0,0 and Xh0,0 for X being replaced by b, c, d, e, f, g and h, respectively, shall be derived by applying the 4-tap filter to the intermediate values Xai,0 where i = -1..2 in horizontal direction:

Xb0,0 = ( -3*Xa-1,0 + 60*Xa0,0 + 8*Xa1,0 – Xa2,0 + offset2 ) >> shift2	(8-44)

Xc0,0 = ( -4*Xa-1,0 + 54*Xa0,0 + 16*Xa1,0 – 2*Xa2,0 + offset2 ) >> shift2	(8-45)

Xd0,0 = ( -5*Xa-1,0 + 46*Xa0,0 + 27*Xa1,0 – 4*Xa2,0 + offset2 ) >> shift2	(8-46)

Xe0,0 = ( -4*Xa-1,0 + 36*Xa0,0 + 36*Xa1,0 – 4*Xa2,0 + offset2 ) >> shift2	(8-47)

Xf0,0 = ( -4*Xa-1,0 + 27*Xa0,0 + 46*Xa1,0 – 5*Xa2,0 + offset2 ) >> shift2	(8-48)

Xg0,0 = ( -2*Xa-1,0 + 16*Xa0,0 + 54*Xa1,0 – 4*Xa2,0 + offset2 ) >> shift2	(8-49)

Xh0,0 = ( -Xa-1,0 + 8*Xa0,0 + 60*Xa1,0 – 3*Xa2,0 + offset2 ) >> shift2	(8-50)

The positions labelled with lower-case letters within un-shaded blocks represent chroma samples at eighth-pel sample fractional locations. The chroma location offset in fractional-sample units ( xFracC, yFracC ) specifies which of the generated chroma samples at full-sample and fractional-sample locations is assigned to the predicted chroma sample value predSampleLXC[ xC, yC ]. This assignment is done according to Table 8. The value of predSampleLXC[ xC, yC ] shall be the output.

Table 8-12 – Assignment of the chroma prediction sample predSampleLXC[ xC, yC ] for ( X, Y ) being replaced by ( 1, b ), ( 2, c ), ( 3, d ), ( 4, e ), ( 5, f ), ( 6, g ), and ( 7, h ), respectively

| xFracC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | X | X | X | X | X | X | X | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| yFracC | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| predSampleLXC[ xC, yC ] | B << shift3 | ba | ca | da | ea | fa | ga | ha | aY | bY | cY | dY | eY | fY | gY | hY |

##### Weighted sample prediction process

[Ed.: (KU) only the default weighted prediction part is added for bi-directional averaging]

Inputs to this process are:

–	a location ( xB, yB ) specifying the top-left sample of the current prediction unit relative to the top left sample of the current coding unit,

–	the width and height of this prediction unit, nPSW and nPSH,

–	two (nPSW)x(nPSH) arrays predSamplesL0 and predSamplesL1,

–	prediction list utilization flags, predFlagL0 and predFlagL1,

–	the bit-depth of the chroma component, bitDepth.

Outputs of this process are:

–	the (nPSW)x(nPSH) array predSamples of prediction sample values.

Variables shift1, shift2, offset1 and offset2 are derived as follows.

–	The variable shift1 is set equal to 14 – bitDepth and the variable shift2 is set equal to 15 – bitDepth,

–	The variable offset1 is set equal to 1 << ( shift1 – 1 ) and the variable offset2 is set equal to 1 << ( shift2 – 1 ).

Depending on the value of predFlagL0 and predFlagL1, the prediction samples predSamples[ x, y ] with x = 0..(nPSW)-1 and y = 0..(nPSH)-1 are derived as follows.

–	If predFlagL0 is equal to 1 and predFlagL1 is equal to 0,

predSamples[ x, y ] = ( predSamplesL0[ x, y ] + offset1 ) >> shift1	(8-51)

–	Otherwise, if predFlagL0 is equal to 0 and predFlagL1 is equal to 1,

predSamples[ x, y ] = ( predSamplesL1[ x, y ] + offset1 ) >> shift1	(8-52)

–	Otherwise (both predFlagL0 and predFlagL1 are equal to 1),

predSamples[ x, y ] = Clip3( 0, ( 1 << bitDepth ) – 1 , 
								 predSamplesL0[ x, y ] + predSamplesL1[ x, y ] + offset2 ) >> shift2 )	(8-53)