| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F586 |
| --- | --- |

| Title: | Adaptive sampling for Intra coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jae Hoon Kim Jian Lou Xue Fang Limin Wang 6450 Sequence Dr. San Diego, CA 92121 USA | Tel: Email: | +1-858-404-2369 jaekim@motorola.com +1-858-404-3427 lou@motorola.com +1-858-404-2723 X.Fang@motorola.com +1-858-404-3893 limin.wang@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

This contribution presents adaptive sampling technique exploiting spatial correlation in CUs at specified depth. Different sampling directions are defined and tested to determine the best sampling mode in CU at specified depth. Multiple sampling modes are proposed to reflect different spatial correlation in a picture. Experimental results based on HM2.0 for Intra prediction show average 1.4% and 2.5% of bit rate savings in Luma for class B and class E test sequences.

# Introduction

To achieve higher coding efficiency in video compression, signal is predicted spatially and/or temporally. Spatial correlation within a picture is exploited by Intra prediction. In Intra prediction, neighboring pixel information is used to predict current pixel information thus, as the number of prediction modes increases, the coding efficiency increases with also increased complexity. In this contribution, we propose to adaptively sample a target CU within a picture so that the less number of pixels needs to be sent.

# Adaptive sampling

Because in HEVC, CU is used as the basic coding unit, basic sampling unit is defined as CU, and adaptive sampling depth (AS depth) is used to specify the CUs to be coded through adaptive sampling. We define 4 sampling modes; first, the original CU without sampling (ORG), second, vertical sub-sampling by 2 (HOR), horizontal sub-sampling by 2 (VER) and both vertical and horizontal sub-sampling by 2 (QTR). For example, if LCU size is 64x64 (width and height in pixels) and AS depth is 1, adaptive sampling is applied to 32x32 CUs. After sub-sampling, a block in HOR mode is 32x16, a block in VER mode is 16x32 and a block in QTR mode is 16x16.

After sub-sampling of 32x32 CU and split, only two 16x16 CUs need to be coded for HOR and VER modes and only one 16x16 CU needs to be coded for QTR mode as shown in Fig. 1 and 2.

To sub-sample a CU, DCT transform is used as a sampling method. For example, to resize a 32x32 ORG CU into 32x16 HOR mode, it is first DCT transformed into the frequency domain and the high frequency components size of 32x16 are cropped. The remaining 32x16 block in frequency domain is DCT transformed back to the spatial domain with proper scaling.

Figure 1. Example of 32x32 CU as a sampling unit (ORG) and 32x16 (HOR), 16x32 (VER) and 16x16 (QTR) as sampled block in HEVC.

Figure 2. Example of 64x64 LCU split with AS depth equal to 1. The first 32x32 CU is coded as ORG, the second 32x32 CU is coded as HOR, the third 32x32 CU is coded as VER and the last 32x32 CU is coded as QTR. Note that HOR-partIdx0 can be further split into four 16x16 CUs. HOR – partIdx2 and partIdx3, VER-partIdx1 and partIdx3 and QTR-partIdx1, partIdx2 and partIdx3 are not available after sub-sampling.

# Experimental results

The proposed technique is implemented in HM2.0 software [1] and tested under the common test conditions. The performance is compared to HM2.0 anchor in terms of Bjøntegaard Delta (BD) Bit Rate [2]. Table 1 summarizes the experimental results for Intra and IntraLC with AS applied at CU size 32x32. Results have been cross-checked by Sharp [3].

Table 1. Experimental results of adaptive sampling on HM2.0 with AS applied at CU32x32

| AS @ CU32x32 | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.2 | -1.3 | -1.8 | -3.9 | -2.9 | -3.4 |
| Class B | -1.4 | -2.6 | -3.0 | -2.6 | -4.3 | -4.7 |
| Class C | -0.8 | -1.3 | -1.3 | -1.1 | -2.0 | -2.0 |
| Class D | -0.3 | -0.7 | -0.6 | -0.6 | -1.4 | -1.2 |
| Class E | -2.5 | -3.4 | -3.7 | -4.3 | -6.6 | -6.6 |
| All | -1.2 | -1.8 | -2.0 | -2.4 | -3.3 | -3.5 |

# References

- https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-2.0/

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-D600, JCT-VC 4th Meeting: Daegu, KR, 20-28 January, 2011.

- Kiran Misra, Andrew Segall, “A Cross-check report for JCTVC-F586 proposal on adaptive sampling for intra coding”, JCTVC-F743, JCT-VC 6th Meeting: Turin, IT, 14-22 July, 2011

# Patent rights declaration(s)

Motorola Mobility Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).