| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F587 |
| --- | --- |

| Title: | Reduction of reference picture list checking for temporal motion vector prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Il-Koo Kim Woo-Jin Han JeongHoon Park | Email: | ilkoo.kim@samsung.com wjhan.han@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In this contribution, reduction of reference picture list checking for temporal motion vector prediction is proposed. Current HM-3.0 always checks two reference picture lists to select the best picture list of collocated PU for temporal motion vector prediction. In this contribution, only one picture list is tested for collocated PU, but only when the first picture list doesn’t exist, the other list is examined instead of checking two picture lists. This simplification shows no loss without encoding and decoding time changes. It is recommended to adopt this simplification in the next HM.

# Introduction

Current HM-3.0 always checks two reference picture lists to select the best picture list of collocated PU for temporal motion vector prediction. In this proposal, reduction of reference picture list checking for temporal motion vector prediction is proposed. Only one picture list is tested, but only when the first picture list doesn’t exist, the other list is examined instead of checking two picture lists always.

# Reduction of reference picture list checking for temporal motion vector prediction

Figure 1 illustrates the temporal motion vector prediction scheme in HM-3.0. In order to derive temporal motion vector candidate, POC numbers of reference pictures of collocated PU are needed. This is because temporal motion vector candidate is constructed by POC-based scaling of motion vector of collocated PU. In this figure, motion information for list0 and list1 of collocated PUs are examined to determine the best motion information. Then, the motion vector which is crossing the current picture is selected for the temporal motion vector scaling. The crossing or not is determined based on POCs of reference picture of collocated PU and current picture. In Figure 1, list0 is selected as the best since it crosses the current picture and list1 does not.

Figure 1 Reference picture list selection of collocated PU in HM-3.0

However, in HM-3.0 design, two reference picture lists have to be checked in order to determine which list’s motion vector is crossing the current picture for every PU. In this contribution, it is proposed to check motion information in only one picture list, but only when motion information of the first picture list doesn’t exist (not available or intra PU), the other list is examined instead of checking two picture lists always. Figure 2 illustrates this proposal. As in Figure 2, motion information of one reference list (list0 or list1) is checked and if the motion information is available, the other list is not checked. The motion information of the other list is only checked when the motion information of the first list is not available.

Figure 2 Proposed reference picture list selection of coolocated PU

In our design, we choose the first picture list by reverse the colDir as to set (1-colDir) in order to increase the possibility of crossing the current picture. However, by this simplification, 0.3% loss is obtained for low delay configurations (please see Table 1 )

In order to compensate the loss, in low delay configuration, one condition is added and the list which has same list of current picture is selected as the first list checked. This is because, in low delay configuration, there is no such case that the motion of collocated PU is crossing current picture since POCs of collocated picture’s references are always smaller than current picture and in GPB case, all reference pictures are identical for both list0 and list1. In this case, giving priority to crossing is not beneficial.

At first, UseLDC() function in HM-3.0 reference software is used to check this condition in our implementation. This approach is simple but has a problem because LDC flag is not in current WD and will be removed in software anyway after implementing MMCO-like picture buffer management. Thus, another detection method is proposed, which detects low delay condition by checking POCs of reference lists. If all POCs of reference lists are smaller than POC of current slice, low delay condition is assumed. (slice level detection)

# Experimental results

Proposed method is implemented in HM-3.0 reference software [1] and tested under common test condition [2]. Error: Reference source not found shows results of proposed method choosing the first picture list as (1-colDir). The average loss from this unification is average 0.2% without encoding and decoding time changes. Whereas average loss of random access configuration is 0.0%, average 0.3% loss is observed in low delay configuration.

Table 1 Test results with the first picture list as (1-colDir).

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.3 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 |
| Class E |  |  |  |  |  |  |
| Overall | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2 | 0.4 | 0.6 | 0.6 | 0.4 | 0.2 |
| Class C | 0.2 | 0.3 | 0.2 | 0.3 | 0.2 | 0.0 |
| Class D | 0.4 | 0.4 | 0.8 | 0.3 | -0.1 | 0.2 |
| Class E | 0.3 | 0.3 | 0.2 | 0.6 | 0.8 | 0.2 |
| Overall | 0.3 | 0.3 | 0.5 | 0.4 | 0.3 | 0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

Table 2 shows results of proposed method using low delay condition check. The average loss from this modification is average 0.0% without encoding and decoding time changes. The results of low delay cases are exactly same with those of HM-3.0.

Table 2 Test result with low delay flag check by UseLDC() function

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.3 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 |
| Class E |  |  |  |  |  |  |
| Overall | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 3shows results of proposed method using low delay condition detection. The average loss from this modification is average 0.0% without encoding and decoding time changes. The results of low delay cases are exactly same with those of HM-3.0.

Table 3 Test result with low delay condition detection

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.1 | 0.0 | 0.0 | -0.1 | 0.1 |
| Class B | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | -0.1 | 0.0 | -0.2 | 0.0 |
| Class E |  |  |  |  |  |  |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusions

In this proposal, reduction of reference picture list checking for temporal motion vector prediction was proposed. Instead of checking two picture lists always, only one picture list is tested, but only when the first picture list doesn’t exist, the other list is examined. This simplification shows no loss without encoding and decoding time changes. It is recommended to adopt this simplification in the next HM.

# References

- https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-3.0/

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, 5th Meeting: Geneva, CH, 16-23 March, 2011

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)

# Proposed draft text

## With the first picture list as (1-colDir)

- Derivation process for temporal luma motion vector prediction

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are

- the motion vector prediction mvLXCol,

- the availability flag availableFlagLXCol.

The function RefPicOrderCnt( pic, refidx, LX ) is specified by the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of pic with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-exisiting.”

Depending on the values of slice_type and collocated_from_l0_flag, the variable colPic, specifying the picture that contains the co-located partition, is derived as follows.

- If slice_type is equal to B and collocated_from_l0_flag is equal to 0, the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList1[ 0 ].

- Otherwise (slice_type is equal to B and collocated_from_l0_flag is equal to 1 or slice_type is equal to P) , the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList0[ 0 ].

Variable colPu and its position ( xPCol, yPCol ) are derived in the following ordered steps:

- Right-bottom luma position of the current prediction unit is defined by

xPRb = xP + nPSW (8-1)

yPRb = yP + nPSH (8-2)

- The variable colPu is set as the prediction unit covering the modified position given by ( ( xPRb >> 4 ) << 4, ( yPRb >> 4 ) << 4 ) inside the colPic.

- If colPu is coded in an intra prediction mode or colPu is unavailable, the following applies.

  - Central luma position of the current prediction unit is defined by

xPCtr = ( xP + ( nPSW >> 1 ) – 1 (8-3)

yPCtr = ( yP + ( nPSH >> 1 ) – 1 (8-4)

  - The variable colPu is set as the prediction unit covering the modified position given by ( ( xPCtr >> 4 ) << 4, ( yPCtr >> 4 ) << 4 ) inside the colPic.

- ( xPCol, yPCol ) is set equal to the top-left luma sample of the colPu relative to the top-left luma sample of the colPic.

The variables mvLXCol and availableFlagLXCol are derived as follows.

- If colPu is coded in an intra prediction mode or colPu is unavailable, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise (colPu is not coded in an intra prediction mode and colPu is available), the variables mvCol and refIdxCol are derived as follows,

  - If PredFlagL0[ xPCol ][ yPCol ] is equal to 0, the motion vector mvCol and the reference index refIdxCol are set equal to MvL1[ xPCol ][ yPCol ] and RefIdxL1[ xPCol ][ yPCol ], respectively.

  - Otherwise (PredFlagL0[ xPCol ][ yPCol ] is equal to 1), the following applies.

  - If PredFlagL1[ xPCol ][ yPCol ] is equal to 0, the motion vector mvCol and the reference index refIdxCol are set equal to MvL0[ xPCol ][ yPCol ] and RefIdxL0[ xPCol ][ yPCol ], respectively.

  - Otherwise (PredFlagL1[ xPCol ][ yPCol ] is equal to 1), the motion vector mvCol, the reference index refIdxCol and ListCol are set equal to MvLX[ xPCol ][ yPCol ], RefIdxColLX and LX, respectively, with X being value of collocated_from_l0_flag.

    - The following asignments are made with X being 0 or 1.

      - RefIdxColLX = RefIdxLX[ xPCol ][ yPCol ]

      - If PicOrderCnt( colPic ) is less than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxColLX, LX ) is greater than PicOrderCnt( currPic ) or PicOrderCnt( colPic ) is greater than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxColLX, LX ) is less than PicOrderCnt( currPic ), the variable MvXCross is derived as follows.

        - MvXCross = 1

      - Otherwise (PicOrderCnt( colPic ) is less than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxColLX, LX ) is less than or equal to PicOrderCnt( currPic ) or PicOrderCnt( colPic ) is greater than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxColLX, LX ) is greater than or equal to PicOrderCnt( currPic )), the variable MvXCross is derived as follows.

        - MvXCross = 0

    - If one of the following conditions is true, the motion vector mvCol, the reference index refIdxCol and ListCol are set equal to MvL1[ xPCol ][ yPCol ], RefIdxColL1 and L1, respectively.

      - Mv0Cross is equal to 0 and Mv1Cross is equal to 1

      - Mv0Cross is equal to Mv1Cross and reference index list is equal to L1

    - Otherwise, the motion vector mvCol, the reference index refIdxCol and ListCol are set equal to MvL0[ xPCol ][ yPCol ], RefIdxColL0 and L0, respectively.

and the variable availableFlagLXCol is set equal to 1 and the following applies.

  - If PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) is equal to PicOrderCnt( currPic ) - RefPicOrderCnt( currPic, refIdxLX, LX ),

mvLXCol = mvCol (8-5)

  - Otherwise, mvLXCol is derived as scaled version of the motion vector mvCol as specified below

tx = ( 16384 + Abs( td / 2 ) ) / td (8-6)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 ) (8-7)

mvLXCol = ClipMv( ( DistScaleFactor * mvCol + 128 ) >> 8 ) (8-8)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) ) (8-9)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) ) (8-10)

## With low delay flag check

- Derivation process for temporal luma motion vector prediction

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are

- the motion vector prediction mvLXCol,

- the availability flag availableFlagLXCol.

The function RefPicOrderCnt( pic, refidx, LX ) is specified by the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of pic with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-exisiting.”

Depending on the values of slice_type and collocated_from_l0_flag, the variable colPic, specifying the picture that contains the co-located partition, is derived as follows.

- If slice_type is equal to B and collocated_from_l0_flag is equal to 0, the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList1[ 0 ].

- Otherwise (slice_type is equal to B and collocated_from_l0_flag is equal to 1 or slice_type is equal to P) , the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList0[ 0 ].

Variable colPu and its position ( xPCol, yPCol ) are derived in the following ordered steps:

- Right-bottom luma position of the current prediction unit is defined by

xPRb = xP + nPSW (8-11)

yPRb = yP + nPSH (8-12)

- The variable colPu is set as the prediction unit covering the modified position given by ( ( xPRb >> 4 ) << 4, ( yPRb >> 4 ) << 4 ) inside the colPic.

- If colPu is coded in an intra prediction mode or colPu is unavailable, the following applies.

  - Central luma position of the current prediction unit is defined by

xPCtr = ( xP + ( nPSW >> 1 ) – 1 (8-13)

yPCtr = ( yP + ( nPSH >> 1 ) – 1 (8-14)

  - The variable colPu is set as the prediction unit covering the modified position given by ( ( xPCtr >> 4 ) << 4, ( yPCtr >> 4 ) << 4 ) inside the colPic.

- ( xPCol, yPCol ) is set equal to the top-left luma sample of the colPu relative to the top-left luma sample of the colPic.

The variables mvLXCol and availableFlagLXCol are derived as follows.

- If colPu is coded in an intra prediction mode or colPu is unavailable, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise (colPu is not coded in an intra prediction mode and colPu is available), the variables mvCol and refIdxCol are derived as follows,

  - If PredFlagL0[ xPCol ][ yPCol ] is equal to 0, the motion vector mvCol and the reference index refIdxCol are set equal to MvL1[ xPCol ][ yPCol ] and RefIdxL1[ xPCol ][ yPCol ], respectively.

  - Otherwise (PredFlagL0[ xPCol ][ yPCol ] is equal to 1), the following applies.

  - If PredFlagL1[ xPCol ][ yPCol ] is equal to 0, the motion vector mvCol and the reference index refIdxCol are set equal to MvL0[ xPCol ][ yPCol ] and RefIdxL0[ xPCol ][ yPCol ], respectively.

  - Otherwise (PredFlagL1[ xPCol ][ yPCol ] is equal to 1), the following assignment are made.

  - If getUseLDC() is equal to 1, the motion vector mvCol, the reference index refIdxCol and ListCol are set equal to MvLN[ xPCol ][ yPCol ], RefIdxColLN and LN, respectively, with N being value of RefIdxLX.

  - Otherwise ( getUseLDC() is equal to 0 ), the motion vector mvCol, the reference index refIdxCol and ListCol are set equal to MvLN[ xPCol ][ yPCol ], RefIdxColLN and LN, respectively, with N being value of collocated_from_l0_flag.

    - The following asignments are made with X being 0 or 1.

      - RefIdxColLX = RefIdxLX[ xPCol ][ yPCol ]

      - If PicOrderCnt( colPic ) is less than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxColLX, LX ) is greater than PicOrderCnt( currPic ) or PicOrderCnt( colPic ) is greater than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxColLX, LX ) is less than PicOrderCnt( currPic ), the variable MvXCross is derived as follows.

        - MvXCross = 1

      - Otherwise (PicOrderCnt( colPic ) is less than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxColLX, LX ) is less than or equal to PicOrderCnt( currPic ) or PicOrderCnt( colPic ) is greater than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxColLX, LX ) is greater than or equal to PicOrderCnt( currPic )), the variable MvXCross is derived as follows.

        - MvXCross = 0

    - If one of the following conditions is true, the motion vector mvCol, the reference index refIdxCol and ListCol are set equal to MvL1[ xPCol ][ yPCol ], RefIdxColL1 and L1, respectively.

      - Mv0Cross is equal to 0 and Mv1Cross is equal to 1

      - Mv0Cross is equal to Mv1Cross and reference index list is equal to L1

    - Otherwise, the motion vector mvCol, the reference index refIdxCol and ListCol are set equal to MvL0[ xPCol ][ yPCol ], RefIdxColL0 and L0, respectively.

and the variable availableFlagLXCol is set equal to 1 and the following applies.

  - If PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) is equal to PicOrderCnt( currPic ) - RefPicOrderCnt( currPic, refIdxLX, LX ),

mvLXCol = mvCol (8-15)

  - Otherwise, mvLXCol is derived as scaled version of the motion vector mvCol as specified below

tx = ( 16384 + Abs( td / 2 ) ) / td (8-16)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 ) (8-17)

mvLXCol = ClipMv( ( DistScaleFactor * mvCol + 128 ) >> 8 ) (8-18)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) ) (8-19)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) ) (8-20)

## With low delay condition detection

- Derivation process for temporal luma motion vector prediction

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are

- the motion vector prediction mvLXCol,

- the availability flag availableFlagLXCol.

The function RefPicOrderCnt( pic, refidx, LX ) is specified by the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of pic with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-exisiting.”

Depending on the values of slice_type and collocated_from_l0_flag, the variable colPic, specifying the picture that contains the co-located partition, is derived as follows.

- If slice_type is equal to B and collocated_from_l0_flag is equal to 0, the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList1[ 0 ].

- Otherwise (slice_type is equal to B and collocated_from_l0_flag is equal to 1 or slice_type is equal to P) , the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList0[ 0 ].

Variable colPu and its position ( xPCol, yPCol ) are derived in the following ordered steps:

- Right-bottom luma position of the current prediction unit is defined by

xPRb = xP + nPSW (8-21)

yPRb = yP + nPSH (8-22)

- The variable colPu is set as the prediction unit covering the modified position given by ( ( xPRb >> 4 ) << 4, ( yPRb >> 4 ) << 4 ) inside the colPic.

- If colPu is coded in an intra prediction mode or colPu is unavailable, the following applies.

  - Central luma position of the current prediction unit is defined by

xPCtr = ( xP + ( nPSW >> 1 ) – 1 (8-23)

yPCtr = ( yP + ( nPSH >> 1 ) – 1 (8-24)

  - The variable colPu is set as the prediction unit covering the modified position given by ( ( xPCtr >> 4 ) << 4, ( yPCtr >> 4 ) << 4 ) inside the colPic.

- ( xPCol, yPCol ) is set equal to the top-left luma sample of the colPu relative to the top-left luma sample of the colPic.

The variables mvLXCol and availableFlagLXCol are derived as follows.

- If colPu is coded in an intra prediction mode or colPu is unavailable, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise (colPu is not coded in an intra prediction mode and colPu is available), the variables mvCol and refIdxCol are derived as follows,

  - If PredFlagL0[ xPCol ][ yPCol ] is equal to 0, the motion vector mvCol and the reference index refIdxCol are set equal to MvL1[ xPCol ][ yPCol ] and RefIdxL1[ xPCol ][ yPCol ], respectively.

  - Otherwise (PredFlagL0[ xPCol ][ yPCol ] is equal to 1), the following applies.

  - If PredFlagL1[ xPCol ][ yPCol ] is equal to 0, the motion vector mvCol and the reference index refIdxCol are set equal to MvL0[ xPCol ][ yPCol ] and RefIdxL0[ xPCol ][ yPCol ], respectively.

  - Otherwise (PredFlagL1[ xPCol ][ yPCol ] is equal to 1), the following assignment are made.

  - If all PicOrderCnts of pictures in all reference picture lists are smaller than PicOrderCnt( currPic ), the motion vector mvCol, the reference index refIdxCol and ListCol are set equal to MvLN[ xPCol ][ yPCol ], RefIdxColLN and LN, respectively, with N being value of RefIdxLX.

  - Otherwise (one of PicOrderCnt of pictures in all reference picture lists are larger than PicOrderCnt( currPic ) ), the motion vector mvCol, the reference index refIdxCol and ListCol are set equal to MvLN[ xPCol ][ yPCol ], RefIdxColLN and LN, respectively, with N being value of collocated_from_l0_flag.

    - The following asignments are made with X being 0 or 1.

      - RefIdxColLX = RefIdxLX[ xPCol ][ yPCol ]

      - If PicOrderCnt( colPic ) is less than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxColLX, LX ) is greater than PicOrderCnt( currPic ) or PicOrderCnt( colPic ) is greater than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxColLX, LX ) is less than PicOrderCnt( currPic ), the variable MvXCross is derived as follows.

        - MvXCross = 1

      - Otherwise (PicOrderCnt( colPic ) is less than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxColLX, LX ) is less than or equal to PicOrderCnt( currPic ) or PicOrderCnt( colPic ) is greater than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxColLX, LX ) is greater than or equal to PicOrderCnt( currPic )), the variable MvXCross is derived as follows.

        - MvXCross = 0

    - If one of the following conditions is true, the motion vector mvCol, the reference index refIdxCol and ListCol are set equal to MvL1[ xPCol ][ yPCol ], RefIdxColL1 and L1, respectively.

      - Mv0Cross is equal to 0 and Mv1Cross is equal to 1

      - Mv0Cross is equal to Mv1Cross and reference index list is equal to L1

    - Otherwise, the motion vector mvCol, the reference index refIdxCol and ListCol are set equal to MvL0[ xPCol ][ yPCol ], RefIdxColL0 and L0, respectively.

and the variable availableFlagLXCol is set equal to 1 and the following applies.

  - If PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) is equal to PicOrderCnt( currPic ) - RefPicOrderCnt( currPic, refIdxLX, LX ),

mvLXCol = mvCol (8-25)

  - Otherwise, mvLXCol is derived as scaled version of the motion vector mvCol as specified below

tx = ( 16384 + Abs( td / 2 ) ) / td (8-26)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 ) (8-27)

mvLXCol = ClipMv( ( DistScaleFactor * mvCol + 128 ) >> 8 ) (8-28)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) ) (8-29)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) ) (8-30)