| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F589 |
| --- | --- |

| Title: | Cross check report of JCTVC-F335 Tiles |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Muhammed Coban 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | +1 858 658 3973 mcoban@qualcomm.com |
| Source: | Qualcomm Inc |  |  |

_____________________________

# Abstract

This contribution provides a summary of a cross-check performed by Qualcomm of Tiles (JCTVC-F335) proposal. The results of the crosscheck matched those reported by Cisco. The encoding and decoding times also matched closely.

# Introduction

Tiles partition a picture into columns and rows respectively defined by their vertical and horizontal boundaries. In Experiment 2 of Tiles proposal[2], tiles are compared with multi slices where high-level parallel processing is considered without MTU size matching.

# Results

Results for all configurations, AI-HE, AI-LC, RA-HE, RA-LC, LB-HE and LB-LC, were crosschecked, as specified in common test conditions[1]. Table 1 shows a summary of the BD-rate results. More detailed coding results can be found in the Excel attachment. No YUV files were written when the decoding times were measured. Encoding as well as decoding was done on a LINUX cluster

Table 1: BD-rate results for the proposed Tiles Experiment 2

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.5 | -4.5 | -3.2 | -1.3 | -5.0 | -3.1 |
| Class B | -2.2 | -2.7 | -1.9 | -2.2 | -3.4 | -2.8 |
| Class C | -0.8 | -0.9 | -0.9 | -0.9 | -1.1 | -1.2 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | -4.8 | -5.9 | -5.6 | -4.9 | -8.3 | -7.1 |
| Overall | -1.7 | -2.6 | -2.1 | -1.7 | -3.3 | -2.6 |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 96% |  |  | 97% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -4.3 | -8.4 | -6.3 | -4.4 | -8.7 | -6.5 |
| Class B | -5.4 | -6.3 | -5.2 | -5.5 | -6.5 | -5.7 |
| Class C | -2.1 | -2.6 | -2.4 | -2.1 | -2.4 | -2.5 |
| Class D | 0.0 | -0.3 | -0.3 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| Overall | -3.1 | -4.5 | -3.7 | -3.1 | -4.5 | -3.8 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 94% |  |  | 96% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -4.7 | -5.4 | -5.3 | -4.7 | -5.3 | -4.8 |
| Class C | -1.8 | -2.2 | -2.1 | -1.7 | -2.1 | -2.0 |
| Class D | 0.0 | -0.3 | -0.4 | 0.0 | 0.0 | 0.0 |
| Class E | -15.3 | -16.4 | -16.2 | -15.0 | -15.3 | -15.8 |
| Overall | -4.8 | -5.4 | -5.3 | -4.7 | -5.0 | -5.0 |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 97% |  |  | 96% |  |  |

# Conclusions

We have verified results reported in Experiment 2 of JCTVC-F335 Tiles proposal. The results of the crosscheck matched those reported by Cisco. The encoding and decoding times also matched closely.

# References

- F. Bossen, “Common test conditions and software reference configurations,” JCT-VC document JCTVC-E700, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 5th meeting, Geneva, March 2011.

- A. Fuldseth et al., “Tiles,” JCT-VC document JCTVC-F335, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.