| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F592 |
| --- | --- |

| Title: | Recursive factorization for 16 and 32-point transforms using 4 and 8-point HM 3.0 core transforms |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | R. Joshi, Y. Reznik, J. Sole, M. Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: +1-858-658-4511 Email: rajanj@qualcomm.com |  |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In this contribution, 16 and 32-point transforms are proposed based on the recursive factorization proposed in JCTVC-F352. However the 4 and 8-point transforms in the factorization are replaced by the 4 and 8-point core transforms from HM3.0. This represents a trade-off between the number of multiplications and the number of sequential operations in the transform. As an added benefit, the amount of memory required for storing the quantization and dequantization matrices decreases compared with the full butterfly factorization in JCTVC-F352.

- Introduction

The partial butterfly form of HM 3.0 core transforms of size N, as implemented in HM3.0, can be described as a combination of decomposition into a butterfly stage (sum and difference) followed by transform of size (N/2) for the even (top) part of the transform and an (N/2)×(N/2) matrix multiplication for the odd (lower) part of the transform. As transform size grows, the number of multiplications in the lower part of the transform grows as a square of the transform size. On the other hand, a desirable property of the HM3.0 core transforms is that they have only one cascaded multiplier. The partial butterfly form of the 8 point Cisco/TI transform only needs a 4×4 matrix multiply, which is not very costly, even when using general purpose multipliers. Hence we propose reusing the HM 3.0 core 4 and 8 point transforms in the butterfly structures for the 16 and 32 point transforms.

# Transform specification

Figure 1 shows the 16-point transform proposed in JCTVC-F352. The factors on the right hand side are normalization / scale factors (not including). Thus, the transform is a scaled transform. In order for the transform to be orthogonal, the butterfly factors have to satisfy:

Z4=A42+B42, Z8=A82+B82=C82+D82, and

Z16=A162+B162=C162+D162=E162+F162=G162+H162.

In this factorization, the solid rectangle represents an 8-point transform and dotted rectangle represents a 4 point transform. If we replace the 8-point transform by the HM3.0 core 8-point transform and the 4-point transforms in the lower part by the 4-point core transforms, we arrive at the proposed transform as shown in Figure 2.

Figure 1: The 16-point transform proposed in JCTVC-F352

Figure 2: Proposed 16-point transform

It should be noted that the normalization / scale factors shown on the right hand side in figure 2 are no longer accurate. Instead there are only 3 different normalization / scale factors for the proposed 16-point transform, one for the upper half and two for the lower half. Similarly for the proposed 32-point transform, there are 5 different normalization / scale factors. In comparison, 32-point transform proposed in JCTVC needs 12 normalization / scale factors. Thus one benefit of the proposed transform is that the size of quantization and dequantization matrices is reduced.

- Complexity

## Arithmetic operations

Table 1 compares the number of multiplications for the proposed 16 and 32 point transforms with HM 3.0 core transforms and the transforms proposed in JCTVC-F352. The numbers are corresponding to a one-dimensional transform.

|  | HM 3.0 core transforms | Proposed transforms | Transforms proposed in JCTVC-352 |
| --- | --- | --- | --- |
| 16 point | 86 | 50 | 36 |
| 32 point | 342 | 126 | 92 |

Table 1: Comparison of number of multiplications for a 1-D transform

# Conclusions

In this contribution, 16 and 32-point transforms are proposed based on the recursive factorization proposed in JCTVC-F352. However the 4 and 8-point transforms in the factorization are replaced by the 4 and 8-point core transforms from HM3.0. This represents a trade-off between the number of multiplications and the number of sequential operations in the transform. As an added benefit, the amount of memory required for storing the quantization and dequantization matrices decreases compared with the full butterfly factorization in JCTVC-F352.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).