| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F593 (Supplement) |
| --- | --- |

| Title: | Improved CABAC Context Initialization - Supplement |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kiran Misra, Andrew Segall 5750 Pacific Rim Blvd Camas, WA 98607 USA | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com asegall@sharplabs.com |
| Source: | Sharp Laboratories of America, Inc. |  |  |

_____________________________

# Abstract

This contribution reports the performance of HM-3.1 using improved/re-designed context initialization tables.  These improved tables consist of 47 significant changes to the current table.  The modified tables are then used for testing the proposal in JCT-F593, and it is observed that the gains of the improved tables and the method in JCTVC-F593 are additive (and in some cases more than additive).  The coding efficiency improvement is reported to be LD-HE: Y:-0.6%, U:-2.0%, V:-2.0%; RA-HE: Y:-0.2%, U:-0.9%, V:-0.9%, and LD(P)-HE: Y: -0.1%, U:-0.1%, V:0.1%.

# Introduction

Changing the CABAC [4] initialization parameters within the context tables better reflects the statistical properties of the tools in HM [1] and helps achieve higher compression efficiency.  We report improved tables in Annex A of this document.  The parameters that are changed are highlighted in yellow.

# Results

The proposed CABAC context initialization changes results in further coding improvement summarized below:

|  | Random access HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0 | 0.0 | 0.0 |
| Class B | -0.1 | -0.1 | -0.1 |
| Class C | -0.1 | -0.1 | -0.1 |
| Class D | -0.1 | -0.2 | -0.1 |
| Class E |  |  |  |
| Overall | -0.1 | -0.1 | -0.1 |
|  | Low delay (B) HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.2 | -0.1 | 0.2 |
| Class C | -0.2 | -0.1 | -0.1 |
| Class D | -0.2 | 0.3 | 0.1 |
| Class E | -0.3 | 0.2 | -0.3 |
| Overall | -0.2 | 0.1 | 0.0 |
|  | Low delay (P) HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0 | 0.0 | -0.2 |
| Class C | -0.1 | -0.2 | 0.0 |
| Class D | 0.0 | -0.4 | 0.4 |
| Class E | -0.1 | 0.3 | 0.3 |
| Overall | -0.1 | -0.1 | 0.1 |

We then use the method for context initialization method proposed in JCTVC-F593.  This uses the P slice initialization process for forward predicted B slices.  The results of this experiment is reported below:

|  |  | Low delay (B) HE |  |  |  |  | Random access HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V |  |  | Y | U | V |
| Class A |  |  |  |  |  | Class A | -0.1 | -1.0 | -1.2 |
| Class B |  | -0.4 | -2.2 | -2.3 |  | Class B | -0.2 | -0.8 | -0.9 |
| Class C |  | -0.5 | -1.6 | -1.6 |  | Class C | -0.2 | -0.7 | -0.7 |
| Class D |  | -0.7 | -2.3 | -2.0 |  | Class D | -0.3 | -1.0 | -0.7 |
| Class E |  | -0.7 | -1.8 | -1.9 |  | Class E |  |  |  |
| Overall |  | -0.6 | -2.0 | -2.0 |  | Overall | -0.2 | -0.9 | -0.8 |
|  |  | Low delay (P) HE |  |  |  |  |  |  |  |
|  |  | Y | U | V |  |  |  |  |  |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B |  | 0.0 | 0.0 | -0.2 |  |  |  |  |  |
| Class C |  | -0.1 | -0.2 | 0.0 |  |  |  |  |  |
| Class D |  | 0.0 | -0.4 | 0.4 |  |  |  |  |  |
| Class E |  | -0.1 | 0.3 | 0.3 |  |  |  |  |  |
| Overall |  | -0.1 | -0.1 | 0.1 |  |  |  |  |  |

# Conclusion

We report CABAC context initialization changes results in further coding improvement and is found to be: ld_he: Y:-0.6%, U:-2.0%, V:-2.0%; ra_he: Y:-0.2%, U:-0.9%, V:-0.8%; ld_p_he: Y: -0.1%, U:-0.1%, V:0.1%.

# References

[1] High efficiency test model (HM) version 3.1 URL: https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-3.1/

[2] Frank Bossen, "Common test conditions and software reference configurations," JCTVC-E700, 5th Meeting: Geneva, CH, 16-23 March, 2011.

[3] Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

[4] Detlev Marpe, Heiko Schwarz, Thomas Weigand, “Context-Based Adaptive Binary Arithmethic Coding in the H.264/AVC Video Compression Standard,” IEEE Transactions on Circuits and Systems for Video Technology, Vol. 13, No. 7, pp. 620-636, July 2003.

# Appendix A

The CABAC context initialization parameters and their updated values (when compared to their original values in HM-3.1) are listed below. The corresponding table numbers in JCTVC-E603 working draft 8 is listed in round brackets.

- Table 1 (Table 9-20 – Values of variables m and n for skip_flag ctxIdx)

| Initialization variables | skip_flag ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| M | 20 | 0 | 22 | 16 | 0 | 28 |
| n | 14 | 64 | 41 | 25 | 64 | 35 |
| P Slice : (0..2) B Slice : (3..5) |  |  |  |  |  |  |

- Table 2 (Table 9-27 – Values of variables m and n for merge_idx ctxIdx)

| Initialization variables | merge_idx ctxIdx |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| m | 0 | -3 | -11 | -4 | 1 | 6 | -7 | -4 |
| n | 68 | 58 | 83 | 72 | 65 | 42 | 75 | 72 |
| P Slice : (0..3) B Slice : (4..7) |  |  |  |  |  |  |  |  |

- Table 3

| Initialization variables | SYNTAX: PART_SIZE ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| m | 0 | 0 | 0 | 0 | 0 | -9 | -4 | 6 | -7 | -11 | -1 | -3 | 10 | -3 | -9 |
| n | 73 | 64 | 64 | 64 | 64 | 87 | 61 | 67 | 62 | 68 | 68 | 59 | 61 | 55 | 64 |
| I Slice : (0..4) P Slice : (5..9) B Slice : (10..14) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- Table 4

| Initialization variables | SYNTAX: PRED_MODE ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| m | 0 | 0 | 0 | 0 | 0 | 0 |
| n | 64 | 64 | 64 | 64 | 64 | 64 |
| I Slice : (0..1) P Slice : (2..3) B Slice : (4..5) |  |  |  |  |  |  |

- Table 5

| Initialization variables | SYNTAX: INTER_DIR ctxIdx |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| m | -4 | -5 | -4 | 0 | -2 | -5 | -9 | 1 |
| n | 51 | 54 | 56 | 64 | 58 | 70 | 85 | 61 |
| P Slice : (0..3) B Slice : (4..7) |  |  |  |  |  |  |  |  |

- Table 6 (Table 9-30 – Values of variables m and n for mvd_lc, mvd_l0, mvd_l1 ctxIdx)

- mvd_l0[ ][ ][ 0 ] P:0..6,B:14..20;

- mvd_lc[ ][ ][ 0 ], mvd_l1[ ][ ][ 0 ] B:14..20;

- mvd_l0[ ][ ][ 1 ] P:7..13, B:21..27;

- mvd_lc[ ][ ][ 1 ], mvd_l1[ ][ ][ 1 ] B:21..27;

- mvd_lc[ ][ ][ 0 ], 0..1	2..3 P:0..1,B:2..3;

| Initialization variables | mvd_lc, mvd_l0, mvd_l1 ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| m | -6 | -6 | -9 | 16 | 13 | 2 | 8 | -6 | -7 | -9 | 15 | 10 | 4 | 7 | -4 |
| n | 80 | 84 | 90 | 32 | 55 | 70 | 74 | 77 | 84 | 89 | 33 | 62 | 68 | 75 | 75 |
|  | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 |  |  |
| m | -5 | -12 | 7 | 11 | 6 | 8 | -2 | -5 | -21 | 6 | 10 | 5 | 10 |  |  |
| n | 82 | 94 | 55 | 59 | 63 | 71 | 71 | 81 | 111 | 58 | 60 | 64 | 67 |  |  |
| P Slice : (0..13) B Slice : (14..27) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- Table 7

| Initialization variables | SYNTAX: REF_PIC ctxIdx |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| m | -6 | -10 | -8 | -17 | 1 | 0 | -9 | -9 | -9 | -12 | -35 | 0 |
| n | 59 | 75 | 75 | 96 | 59 | 64 | 55 | 71 | 76 | 86 | 109 | 64 |
| P Slice : (0..5) B Slice : (6..11) |  |  |  |  |  |  |  |  |  |  |  |  |

- Table 8

| Initialization variables | SYNTAX: QT_CBF ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| m | -22 | -5 | -16 | -16 | -32 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| n | 116 | 75 | 112 | 111 | 165 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 |
|  | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| m | -35 | -12 | -9 | -10 | -14 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| n | 116 | 61 | 73 | 75 | 96 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 |
|  | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 |
| m | -29 | -12 | -5 | -6 | -11 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| n | 104 | 59 | 65 | 67 | 90 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 |
|  | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 |
| m | -18 | -41 | -29 | -23 | -35 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| n | 98 | 120 | 117 | 108 | 143 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 |
|  | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 |
| m | -46 | -42 | -11 | -19 | -42 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| n | 114 | 119 | 74 | 90 | 139 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 |
|  | 75 | 76 | 77 | 78 | 79 | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 |
| m | -43 | -41 | -17 | -25 | -14 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| n | 107 | 118 | 86 | 101 | 91 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 |
|  | 90 | 91 | 92 | 93 | 94 | 95 | 96 | 97 | 98 | 99 | 100 | 101 | 102 | 103 | 104 |
| m | -11 | -32 | -19 | -26 | -38 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| n | 80 | 83 | 89 | 112 | 149 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 |
|  | 105 | 106 | 107 | 108 | 109 | 110 | 111 | 112 | 113 | 114 | 115 | 116 | 117 | 118 | 119 |
| m | -22 | -48 | -7 | -37 | -58 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| n | 52 | 123 | 68 | 121 | 164 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 |
|  | 120 | 121 | 122 | 123 | 124 | 125 | 126 | 127 | 128 | 129 | 130 | 131 | 132 | 133 | 134 |
| m | -19 | -48 | -21 | -9 | -42 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| n | 45 | 123 | 94 | 73 | 138 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 |
| I Slice : (0..44) P Slice : (45..89) B Slice : (90..134) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- Table 9

| Initialization variables | SYNTAX: SIGMAP ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| m | -3 | -17 | -7 | -12 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| n | 102 | 114 | 97 | 96 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 |
|  | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| m | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| n | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 |
|  | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 |
| m | 0 | 0 | -5 | -9 | -5 | -7 | -3 | -1 | -5 | -2 | 1 | -6 | 0 | 1 | 0 |
| n | 64 | 64 | 99 | 92 | 90 | 83 | 37 | 62 | 81 | 41 | 64 | 82 | 12 | 43 | 57 |
|  | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 |
| m | -1 | -4 | 0 | 23 | 0 | 10 | 0 | -1 | 4 | -3 | -12 | -4 | -2 | -18 | 5 |
| n | 66 | 79 | 64 | 51 | 69 | 54 | 65 | 42 | 51 | 70 | 61 | 50 | 68 | 57 | 38 |
|  | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 |
| m | 1 | -3 | -9 | 0 | -10 | -9 | -3 | 1 | -13 | -13 | -2 | -5 | -11 | -10 | -3 |
| n | 54 | 66 | 80 | 64 | 91 | 76 | 61 | 46 | 84 | 81 | 60 | 61 | 77 | 76 | 65 |
|  | 75 | 76 | 77 | 78 | 79 | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 |
| m | -3 | -4 | -14 | 1 | -33 | 19 | 8 | 2 | -3 | 8 | 16 | -10 | -51 | 2 | -9 |
| n | 63 | 59 | 79 | 61 | 126 | 45 | 48 | 40 | 40 | 46 | 31 | 73 | 114 | 52 | 71 |
|  | 90 | 91 | 92 | 93 | 94 | 95 | 96 | 97 | 98 | 99 | 100 | 101 | 102 | 103 | 104 |
| m | -37 | -8 | -10 | -47 | -105 | -32 | -4 | -2 | -4 | 16 | 0 | -1 | -4 | -16 | -9 |
| n | 118 | 38 | 64 | 113 | 234 | 123 | 94 | 84 | 83 | 39 | 83 | 81 | 82 | 91 | 86 |
|  | 105 | 106 | 107 | 108 | 109 | 110 | 111 | 112 | 113 | 114 | 115 | 116 | 117 | 118 | 119 |
| m | -8 | -2 | -14 | -16 | 13 | -12 | 0 | 29 | 1 | -6 | 23 | 13 | -6 | -8 | 35 |
| n | 89 | 82 | 92 | 91 | 44 | 91 | 64 | 42 | 74 | 77 | 41 | 63 | 88 | 80 | 17 |
|  | 120 | 121 | 122 | 123 | 124 | 125 | 126 | 127 | 128 | 129 | 130 | 131 | 132 | 133 | 134 |
| m | 0 | -17 | -25 | -39 | 26 | -36 | -41 | 0 | -7 | 5 | -4 | 5 | 0 | 0 | 0 |
| n | 70 | 100 | 111 | 122 | 29 | 114 | 130 | 64 | 88 | 52 | 74 | 56 | 64 | 64 | 64 |
|  | 135 | 136 | 137 | 138 | 139 | 140 | 141 | 142 | 143 | 144 | 145 | 146 | 147 | 148 | 149 |
| m | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| n | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 |
|  | 150 | 151 | 152 | 153 | 154 | 155 | 156 | 157 | 158 | 159 | 160 | 161 | 162 | 163 | 164 |
| m | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | -1 | -3 | 6 |
| n | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 75 | 59 | 67 | 67 | 29 |
|  | 165 | 166 | 167 | 168 | 169 | 170 | 171 | 172 | 173 | 174 | 175 | 176 | 177 | 178 | 179 |
| m | 2 | -5 | -5 | -4 | -7 | -9 | 2 | 2 | 2 | -8 | 0 | 19 | 11 | 9 | -24 |
| n | 57 | 77 | 44 | 64 | 78 | 34 | 43 | 53 | 60 | 83 | 64 | 22 | 48 | 51 | 95 |
|  | 180 | 181 | 182 | 183 | 184 | 185 | 186 | 187 | 188 | 189 | 190 | 191 | 192 | 193 | 194 |
| m | -27 | -22 | -20 | -25 | -15 | 24 | -31 | -19 | -21 | -23 | -35 | 0 | 2 | -10 | -1 |
| n | 82 | 92 | 89 | 82 | 77 | 32 | 76 | 77 | 84 | 95 | 112 | 64 | 69 | 82 | 57 |
|  | 195 | 196 | 197 | 198 | 199 | 200 | 201 | 202 | 203 | 204 | 205 | 206 | 207 | 208 | 209 |
| m | -7 | 8 | 3 | -28 | -29 | -3 | -11 | -6 | -48 | -35 | -10 | 4 | 5 | -3 | -35 |
| n | 59 | 38 | 47 | 107 | 100 | 49 | 67 | 71 | 136 | 111 | 74 | 56 | 56 | 84 | 122 |
|  | 210 | 211 | 212 | 213 | 214 | 215 | 216 | 217 | 218 | 219 | 220 | 221 | 222 | 223 | 224 |
| m | -42 | 101 | -14 | -70 | 6 | 0 | 8 | -64 | 0 | 0 | -65 | 0 | 0 | 0 | 6 |
| n | 111 | -147 | 87 | 179 | 30 | 64 | 32 | 156 | 64 | 64 | 144 | 64 | 64 | 64 | 67 |
|  | 225 | 226 | 227 | 228 | 229 | 230 | 231 | 232 | 233 | 234 | 235 | 236 | 237 | 238 | 239 |
| m | -1 | -9 | -29 | 4 | 0 | -7 | -34 | -11 | -14 | -15 | -34 | 26 | -34 | -35 | 0 |
| n | 72 | 84 | 100 | 65 | 70 | 80 | 117 | 79 | 91 | 93 | 119 | 26 | 111 | 126 | 64 |
|  | 240 | 241 | 242 | 243 | 244 | 245 | 246 | 247 | 248 | 249 | 250 | 251 | 252 | 253 | 254 |
| m | -15 | 25 | 23 | -65 | -21 | 27 | -34 | -71 | -30 | 34 | -144 | -168 | 43 | -125 | -129 |
| n | 103 | 19 | 29 | 142 | 108 | 19 | 111 | 174 | 104 | 12 | 285 | 304 | 6 | 248 | 252 |
|  | 255 | 256 | 257 | 258 | 259 | 260 | 261 | 262 | 263 | 264 | 265 | 266 | 267 | 268 | 269 |
| m | 0 | -7 | 5 | -4 | 5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| n | 64 | 88 | 52 | 74 | 56 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 |
|  | 270 | 271 | 272 | 273 | 274 | 275 | 276 | 277 | 278 | 279 | 280 | 281 | 282 | 283 | 284 |
| m | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| n | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 |
|  | 285 | 286 | 287 | 288 | 289 | 290 | 291 | 292 | 293 | 294 | 295 | 296 | 297 | 298 | 299 |
| m | 0 | 0 | 0 | 0 | 1 | -1 | -3 | 6 | 2 | -5 | -5 | -4 | -7 | -9 | 2 |
| n | 64 | 64 | 64 | 75 | 59 | 67 | 67 | 29 | 57 | 77 | 44 | 64 | 78 | 34 | 43 |
|  | 300 | 301 | 302 | 303 | 304 | 305 | 306 | 307 | 308 | 309 | 310 | 311 | 312 | 313 | 314 |
| m | 2 | 2 | -8 | 0 | 19 | 11 | 9 | -24 | -27 | -22 | -20 | -25 | -15 | 24 | -31 |
| n | 53 | 60 | 83 | 64 | 22 | 48 | 51 | 95 | 82 | 92 | 89 | 82 | 77 | 32 | 76 |
|  | 315 | 316 | 317 | 318 | 319 | 320 | 321 | 322 | 323 | 324 | 325 | 326 | 327 | 328 | 329 |
| m | -19 | -21 | -23 | -35 | 0 | 2 | -10 | -1 | -7 | 6 | -13 | -28 | -29 | -21 | -28 |
| n | 77 | 84 | 95 | 112 | 64 | 69 | 82 | 57 | 59 | 43 | 85 | 107 | 100 | 93 | 108 |
|  | 330 | 331 | 332 | 333 | 334 | 335 | 336 | 337 | 338 | 339 | 340 | 341 | 342 | 343 | 344 |
| m | -6 | -48 | -35 | -10 | 4 | 5 | -3 | -35 | -42 | 101 | -14 | -70 | 6 | 0 | 8 |
| n | 71 | 136 | 111 | 74 | 56 | 56 | 84 | 122 | 111 | -147 | 87 | 179 | 30 | 64 | 32 |
|  | 345 | 346 | 347 | 348 | 349 | 350 | 351 | 352 | 353 | 354 | 355 | 356 | 357 | 358 | 359 |
| m | -64 | 0 | 0 | -65 | 0 | 0 | 0 | 6 | -1 | -9 | -29 | 4 | 0 | -7 | -34 |
| n | 156 | 64 | 64 | 144 | 64 | 64 | 64 | 67 | 72 | 84 | 100 | 65 | 70 | 80 | 117 |
|  | 360 | 361 | 362 | 363 | 364 | 365 | 366 | 367 | 368 | 369 | 370 | 371 | 372 | 373 | 374 |
| m | -11 | -14 | -15 | -34 | 26 | -34 | -35 | 0 | -15 | 25 | 23 | -65 | -21 | 27 | -34 |
| n | 79 | 91 | 93 | 119 | 26 | 111 | 126 | 64 | 103 | 19 | 29 | 142 | 108 | 19 | 111 |
|  | 375 | 376 | 377 | 378 | 379 | 380 | 381 | 382 | 383 |  |  |  |  |  |  |
| m | -71 | -30 | 34 | -144 | -168 | 43 | -125 | -129 | 0 |  |  |  |  |  |  |
| n | 174 | 104 | 12 | 285 | 304 | 6 | 248 | 252 | 64 |  |  |  |  |  |  |
| I Slice : (0..127) P Slice : (128..255) B Slice : (256..383) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- Table 10

| Initialization variables | SYNTAX: last_significant_coeff_x ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| m | 8 | 8 | 8 | 12 | 8 | 6 | 3 | -1 | 18 | 14 | 15 | 16 | 16 | 12 | -4 |
| n | 31 | 39 | 48 | 31 | 38 | 45 | 46 | 56 | 16 | 22 | 22 | 17 | 16 | 24 | 59 |
|  | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| m | 33 | 18 | 20 | 22 | 17 | 11 | 31 | 38 | 12 | -4 | -13 | 32 | 25 | 50 | 32 |
| n | -26 | 1 | 2 | -1 | 14 | 21 | -24 | -38 | 11 | 47 | 69 | 11 | 27 | -1 | 20 |
|  | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 |
| m | 12 | 18 | 12 | 5 | 40 | 14 | 17 | 7 | 15 | 15 | 9 | 0 | 0 | 0 | 0 |
| n | 38 | 32 | 41 | 70 | -6 | 29 | 26 | 43 | 26 | 27 | 51 | 64 | 64 | 64 | 64 |
|  | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 |
| m | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 9 | 9 | 10 | 17 | 21 | 20 | 14 | 8 |
| n | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 40 | 44 | 52 | 24 | 15 | 20 | 29 | 46 |
|  | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 |
| m | 7 | 18 | 26 | 26 | 25 | 15 | 9 | 23 | 19 | 27 | 33 | 38 | 41 | 40 | 26 |
| n | 46 | 18 | 3 | 0 | 2 | 18 | 27 | 20 | 16 | -1 | -19 | -30 | -39 | -36 | -12 |
|  | 75 | 76 | 77 | 78 | 79 | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 |
| m | 3 | -7 | -4 | 16 | 16 | 49 | 6 | 17 | 24 | 24 | 61 | -11 | 6 | 14 | 17 |
| n | 20 | 33 | 20 | 42 | 45 | 7 | 61 | 36 | 24 | 21 | -22 | 101 | 53 | 40 | 30 |
|  | 90 | 91 | 92 | 93 | 94 | 95 | 96 | 97 | 98 | 99 | 100 | 101 | 102 | 103 | 104 |
| m | 20 | -10 | -6 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 9 |
| n | 22 | 67 | 68 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 40 |
|  | 105 | 106 | 107 | 108 | 109 | 110 | 111 | 112 | 113 | 114 | 115 | 116 | 117 | 118 | 119 |
| m | 9 | -8 | 17 | 21 | 20 | 14 | 8 | 7 | 18 | 26 | 26 | 25 | 15 | 9 | 23 |
| n | 44 | 96 | 24 | 15 | 20 | 29 | 46 | 46 | 18 | 3 | 0 | 2 | 18 | 27 | 20 |
|  | 120 | 121 | 122 | 123 | 124 | 125 | 126 | 127 | 128 | 129 | 130 | 131 | 132 | 133 | 134 |
| m | 19 | 27 | 33 | 38 | 41 | 40 | 26 | 3 | -7 | -4 | 16 | 16 | 49 | 6 | 17 |
| n | 16 | -1 | -19 | -30 | -39 | -36 | -12 | 20 | 33 | 20 | 42 | 45 | 7 | 61 | 36 |
|  | 135 | 136 | 137 | 138 | 139 | 140 | 141 | 142 | 143 | 144 | 145 | 146 | 147 | 148 | 149 |
| m | 24 | 24 | 61 | -11 | 6 | 14 | 17 | 20 | -10 | -6 | 0 | 0 | 0 | 0 | 0 |
| n | 24 | 21 | -22 | 101 | 53 | 40 | 30 | 22 | 67 | 68 | 64 | 64 | 64 | 64 | 64 |
|  | 150 | 151 | 152 | 153 | 154 | 155 |  |  |  |  |  |  |  |  |  |
| m | 0 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |  |  |  |  |
| n | 64 | 64 | 64 | 64 | 64 | 64 |  |  |  |  |  |  |  |  |  |
| I Slice : (0..51) P Slice : (52..103) B Slice : (104..155) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- Table 11 (Table 9-42 – Values of variables m and n for coeff_abs_level_greater1_flag ctxIdx)

| Initialization variables | coeff_abs_level_greater1_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| m | -11 | -20 | -16 | -13 | -10 | -5 | -8 | -8 | -3 | -9 | 0 | -5 | -12 | -9 | -1 |
| n | 87 | 64 | 68 | 71 | 73 | 67 | 26 | 37 | 36 | 56 | 63 | 39 | 56 | 57 | 52 |
|  | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| m | -4 | -19 | -28 | -23 | -3 | -2 | -27 | -22 | -14 | -1 | -10 | -22 | -13 | -6 | -5 |
| n | 72 | 73 | 88 | 85 | 59 | 72 | 97 | 89 | 77 | 58 | 86 | 74 | 63 | 57 | 63 |
|  | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 |
| m | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | -4 | 16 | 9 | 5 | -7 |
| n | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 70 | -2 | 23 | 41 | 67 |
|  | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 |
| m | -6 | 13 | -12 | -18 | -14 | -5 | -8 | -22 | -35 | -3 | -12 | -20 | -51 | -44 | -23 |
| n | 63 | -5 | 42 | 53 | 59 | 65 | 36 | 67 | 89 | 47 | 77 | 66 | 113 | 109 | 84 |
|  | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 |
| m | -1 | -58 | -71 | -67 | -91 | -1 | 20 | 13 | 2 | 5 | 0 | 0 | 0 | 0 | 0 |
| n | 64 | 127 | 143 | 134 | 187 | 64 | -3 | 21 | 46 | 48 | 64 | 64 | 64 | 64 | 64 |
|  | 75 | 76 | 77 | 78 | 79 | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 |
| m | 0 | 0 | 0 | 0 | 0 | -4 | 29 | 1 | -6 | -9 | -6 | 23 | 3 | -5 | 5 |
| n | 64 | 64 | 64 | 64 | 64 | 71 | -5 | 45 | 58 | 67 | 66 | -7 | 26 | 42 | 31 |
|  | 90 | 91 | 92 | 93 | 94 | 95 | 96 | 97 | 98 | 99 | 100 | 101 | 102 | 103 | 104 |
| m | -12 | -2 | -15 | -11 | -3 | -4 | -13 | -31 | -26 | -18 | -9 | -30 | -24 | -43 | -6 |
| n | 79 | 45 | 67 | 62 | 54 | 70 | 68 | 100 | 91 | 84 | 83 | 103 | 90 | 122 | 66 |
|  | 105 | 106 | 107 | 108 | 109 | 110 | 111 | 112 | 113 | 114 | 115 | 116 | 117 | 118 | 119 |
| m | 4 | 34 | 21 | 14 | 5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| n | 59 | -11 | 10 | 25 | 44 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 |
|  | 120 | 121 | 122 | 123 | 124 | 125 | 126 | 127 | 128 | 129 | 130 | 131 | 132 | 133 | 134 |
| m | -4 | 40 | 19 | -11 | -6 | -11 | 12 | 17 | -6 | 11 | -11 | 1 | -26 | -28 | -2 |
| n | 69 | -33 | 8 | 65 | 59 | 68 | -2 | -10 | 34 | 14 | 70 | 27 | 71 | 79 | 47 |
|  | 135 | 136 | 137 | 138 | 139 | 140 | 141 | 142 | 143 | 144 | 145 | 146 | 147 | 148 | 149 |
| m | 6 | -23 | -47 | -55 | 25 | -70 | -38 | -34 | 106 | 3 | 9 | 41 | 32 | 14 | 17 |
| n | 47 | 67 | 107 | 117 | 12 | 163 | 82 | 77 | -160 | 51 | 46 | -31 | -17 | 19 | 18 |
|  | 150 | 151 | 152 | 153 | 154 | 155 | 156 | 157 | 158 | 159 | 160 | 161 | 162 | 163 | 164 |
| m | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | -2 | 18 | 10 | -2 | -7 |
| n | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 65 | 22 | 33 | 55 | 64 |
|  | 165 | 166 | 167 | 168 | 169 | 170 | 171 | 172 | 173 | 174 | 175 | 176 | 177 | 178 | 179 |
| m | -6 | 19 | -5 | -7 | -4 | -23 | -3 | 2 | -32 | -16 | -8 | -21 | -26 | -33 | -4 |
| n | 67 | 11 | 50 | 53 | 54 | 99 | 51 | 41 | 102 | 79 | 77 | 84 | 91 | 104 | 61 |
|  | 180 | 181 | 182 | 183 | 184 | 185 | 186 | 187 | 188 | 189 | 190 | 191 | 192 | 193 | 194 |
| m | -31 | -34 | -25 | -43 | -6 | 3 | 23 | 12 | 11 | 8 | 0 | 0 | 0 | 0 | 0 |
| n | 122 | 110 | 96 | 124 | 70 | 60 | 12 | 30 | 33 | 40 | 64 | 64 | 64 | 64 | 64 |
|  | 195 | 196 | 197 | 198 | 199 | 200 | 201 | 202 | 203 | 204 | 205 | 206 | 207 | 208 | 209 |
| m | 0 | 0 | 0 | 0 | 0 | -2 | 40 | 0 | -5 | 7 | -39 | 31 | 1 | -35 | -32 |
| n | 64 | 64 | 64 | 64 | 64 | 66 | -20 | 46 | 54 | 37 | 116 | -27 | 22 | 82 | 85 |
|  | 210 | 211 | 212 | 213 | 214 | 215 | 216 | 217 | 218 | 219 | 220 | 221 | 222 | 223 | 224 |
| m | -15 | 16 | -43 | -75 | -8 | -9 | -12 | -84 | -93 | 25 | -104 | -40 | -51 | 110 | 3 |
| n | 72 | 0 | 102 | 152 | 55 | 68 | 54 | 171 | 186 | 12 | 222 | 92 | 93 | -169 | 52 |
|  | 225 | 226 | 227 | 228 | 229 | 230 | 231 | 232 | 233 | 234 | 235 | 236 | 237 | 238 | 239 |
| m | 17 | 55 | 28 | -5 | 29 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| n | 33 | -45 | 1 | 55 | -4 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 |
| I Slice : (0..79) P Slice : (80..159) B Slice : (160..239) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- Table 12 (Table 9-43 – Values of variables m and n for coeff_abs_level_greater2_flag ctxIdx)

| Initialization variables | coeff_abs_level_greater2_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| m | -12 | -10 | -11 | -14 | -35 | -10 | -1 | -17 | -5 | -22 | -13 | -2 | -13 | -21 | -3 |
| n | 72 | 79 | 87 | 94 | 136 | 58 | 54 | 86 | 70 | 105 | 70 | 59 | 81 | 96 | 73 |
|  | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| m | -24 | -19 | -1 | 1 | -17 | -7 | -20 | -11 | -27 | -12 | -9 | -8 | -4 | -3 | -9 |
| n | 90 | 88 | 63 | 60 | 97 | 69 | 95 | 84 | 110 | 96 | 72 | 76 | 75 | 76 | 94 |
|  | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 |
| m | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | -11 | -4 | -9 | -24 | -24 |
| n | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 65 | 66 | 82 | 107 | 111 |
|  | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 |
| m | -18 | -10 | -19 | -28 | 6 | -12 | -17 | -8 | -44 | -17 | -26 | -4 | -43 | -58 | -51 |
| n | 58 | 61 | 82 | 97 | 47 | 50 | 73 | 66 | 115 | 86 | 74 | 48 | 115 | 141 | 137 |
|  | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 |
| m | -51 | -85 | -47 | -95 | -65 | -2 | 7 | -2 | 3 | 4 | 0 | 0 | 0 | 0 | 0 |
| n | 117 | 176 | 120 | 202 | 159 | 53 | 43 | 67 | 62 | 67 | 64 | 64 | 64 | 64 | 64 |
|  | 75 | 76 | 77 | 78 | 79 | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 |
| m | 0 | 0 | 0 | 0 | 0 | -2 | -13 | -11 | -13 | -24 | -4 | -17 | -16 | -19 | -28 |
| n | 64 | 64 | 64 | 64 | 64 | 49 | 81 | 82 | 88 | 112 | 44 | 77 | 82 | 89 | 110 |
|  | 90 | 91 | 92 | 93 | 94 | 95 | 96 | 97 | 98 | 99 | 100 | 101 | 102 | 103 | 104 |
| m | -12 | -11 | -18 | -6 | -19 | -17 | -27 | -18 | -13 | -17 | -11 | -15 | -10 | -10 | -12 |
| n | 64 | 70 | 88 | 67 | 97 | 76 | 100 | 88 | 84 | 94 | 73 | 83 | 77 | 80 | 91 |
|  | 105 | 106 | 107 | 108 | 109 | 110 | 111 | 112 | 113 | 114 | 115 | 116 | 117 | 118 | 119 |
| m | -8 | -7 | -6 | -8 | -9 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| n | 63 | 71 | 73 | 80 | 90 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 |
|  | 120 | 121 | 122 | 123 | 124 | 125 | 126 | 127 | 128 | 129 | 130 | 131 | 132 | 133 | 134 |
| m | 14 | -7 | -4 | -8 | -50 | 2 | 11 | -4 | -41 | 26 | -12 | -6 | 1 | -5 | -12 |
| n | 20 | 68 | 66 | 76 | 148 | 22 | 15 | 49 | 117 | 3 | 49 | 55 | 44 | 58 | 73 |
|  | 135 | 136 | 137 | 138 | 139 | 140 | 141 | 142 | 143 | 144 | 145 | 146 | 147 | 148 | 149 |
| m | 13 | 5 | -29 | 0 | 31 | -12 | 6 | -28 | -26 | 22 | 24 | 11 | 2 | -5 | 4 |
| n | 9 | 26 | 101 | 46 | 12 | 57 | 30 | 66 | 60 | 24 | 0 | 34 | 61 | 75 | 62 |
|  | 150 | 151 | 152 | 153 | 154 | 155 | 156 | 157 | 158 | 159 | 160 | 161 | 162 | 163 | 164 |
| m | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | -6 | -7 | -31 | -25 |
| n | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 43 | 65 | 71 | 117 | 109 |
|  | 165 | 166 | 167 | 168 | 169 | 170 | 171 | 172 | 173 | 174 | 175 | 176 | 177 | 178 | 179 |
| m | -20 | -14 | -20 | -21 | -6 | -19 | -34 | -27 | -7 | -7 | -18 | -7 | -20 | -9 | -7 |
| n | 76 | 73 | 88 | 92 | 71 | 73 | 108 | 101 | 69 | 75 | 77 | 64 | 91 | 75 | 78 |
|  | 180 | 181 | 182 | 183 | 184 | 185 | 186 | 187 | 188 | 189 | 190 | 191 | 192 | 193 | 194 |
| m | -26 | -13 | -6 | -12 | -2 | -3 | -6 | -7 | -6 | -11 | 0 | 0 | 0 | 0 | 0 |
| n | 98 | 81 | 69 | 83 | 70 | 50 | 66 | 73 | 75 | 91 | 64 | 64 | 64 | 64 | 64 |
|  | 195 | 196 | 197 | 198 | 199 | 200 | 201 | 202 | 203 | 204 | 205 | 206 | 207 | 208 | 209 |
| m | 0 | 0 | 0 | 0 | 0 | 19 | 2 | 4 | -4 | -32 | 13 | -29 | -64 | -90 | 26 |
| n | 64 | 64 | 64 | 64 | 64 | 10 | 49 | 52 | 69 | 114 | -1 | 85 | 143 | 186 | 4 |
|  | 210 | 211 | 212 | 213 | 214 | 215 | 216 | 217 | 218 | 219 | 220 | 221 | 222 | 223 | 224 |
| m | -24 | 8 | -10 | -16 | -27 | 31 | -12 | -15 | -68 | 31 | -8 | -32 | -36 | 0 | 0 |
| n | 68 | 30 | 61 | 78 | 96 | -16 | 54 | 70 | 158 | 12 | 44 | 63 | 81 | 64 | 64 |
|  | 225 | 226 | 227 | 228 | 229 | 230 | 231 | 232 | 233 | 234 | 235 | 236 | 237 | 238 | 239 |
| m | 26 | 19 | -22 | 5 | 8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| n | -5 | 17 | 101 | 54 | 53 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 |
| I Slice : (0..79) P Slice : (80..159) B Slice : (160..239) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- Table 13

| Initialization variables | SYNTAX: ALF_FLAG |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 |  |  |
| m | 50 | -15 | -19 |  |  |
| n | -48 | 91 | 98 |  |  |
| I Slice : (0..0) P Slice : (1..1) B Slice : (2..2) |  |  |  |  |  |

- Table 14

| Initialization variables | SYNTAX: ALF_SVLC |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| m | 11 | -1 | 0 | 6 | -1 | -12 | 1 | 2 | -10 |
| n | 57 | 62 | 64 | 66 | 64 | 98 | 73 | 61 | 91 |
| I Slice : (0..2) P Slice : (3..5) B Slice : (6..8) |  |  |  |  |  |  |  |  |  |

- Table 15

| Initialization variables | SYNTAX: TRANS_SUBDIV_FLAG |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| m | 0 | 12 | 22 | -2 | 4 | 0 | 0 | 0 | 0 | 0 | 0 | -28 | -30 | -34 | 0 |
| n | 0 | 12 | 4 | 49 | 46 | 64 | 64 | 64 | 64 | 64 | 64 | 89 | 99 | 106 | 64 |
|  | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| m | 0 | 0 | 0 | 0 | 0 | 0 | -31 | -42 | -47 | 0 | 0 | 0 | 0 | 0 | 0 |
| n | 64 | 64 | 64 | 64 | 64 | 64 | 88 | 118 | 130 | 64 | 64 | 64 | 64 | 64 | 64 |
| I Slice : (0..9) P Slice : (10..19) B Slice : (20..29) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- Table 16

| Initialization variables | SYNTAX: PLANARFLAG |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| m | 0 | 0 | -7 | 0 | -10 | 0 |
| n | 64 | 64 | 85 | 64 | 92 | 64 |
| I Slice : (0..1) P Slice : (2..3) B Slice : (4..5) |  |  |  |  |  |  |

- Table 17

| Initialization variables | SYNTAX: AO_FLAG |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 |  |  |  |
| m | 50 | -22 | -12 |  |  |  |
| n | -48 | 96 | 68 |  |  |  |
| I Slice : (0..0) P Slice : (1..1) B Slice : (2..2) |  |  |  |  |  |  |

- Table 18

| Initialization variables | SYNTAX: AO_UVLC |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| m | 1 | -3 | -5 | -14 | -24 | -30 |
| n | 66 | 77 | 75 | 94 | 116 | 122 |
| I Slice : (0..1) P Slice : (2..3) B Slice : (4..5) |  |  |  |  |  |  |

- Table 19

| Initialization variables | SYNTAX: AO_SVLC |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| m | 11 | -1 | 0 | 14 | -1 | 28 | 1 | 2 | 0 |
| n | 57 | 62 | 64 | 40 | 64 | -1 | 73 | 61 | 64 |
| I Slice : (0..2) P Slice : (3..5) B Slice : (6..8) |  |  |  |  |  |  |  |  |  |