| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F594 |
| --- | --- |

| Title: | New results for parallel decoding for Tiles |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra, Andrew Segall 5750 Pacific Rim Blvd Camas, WA 98607 USA | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com asegall@sharplabs.com |
| Source: | Sharp Laboratories of America, Inc. |  |  |

_____________________________

# Abstract

This contribution presents new results for JCTVC-E412, which extended “Tiles” (JCTVC-E408) to support decoder parallelization. Decoder parallelization is realized by signaling tile entry points in the bit-stream. A decoder is then able to enter the bit-stream at the beginning of a tile.  The signaling is necessary as tiles does not use a header and thus cannot be located in the bit-stream by a decoder (without completely parsing the bit-stream). The approach supports sending tile entry points explicitly in the slice header or using markers in the bit-stream. The bitrate overhead for signaling locations explicitly in slice headers is I_HE: 0.0%, I_LC: 0.0%, RA_HE: 0.1%, RA_LC: 0.2%,  LD_HE: 0.5%, LD_LC: 0.5%. The bitrate overhead for signaling using markers is I_HE: 0.1%, I_LC: 0.1%, RA_HE: 0.6%, RA_LC: 0.7%,  LD_HE: 1.8%, LD_LC: 1.7%.

# Introduction

Tiles provide a convenient mechanism to enable parallelization at an encoder.  Unfortunately, while it is also desirable to use tiles for parallelization at the decoder, it is not currently possible.  This is because a decoder does not know the location of tiles within a bit-stream.  Thus, a decoder can not begin decoding bit-stream without completely parsing the previous tile.  To be clear, after decoding a slice header, a decoder knows the location of Tiles within the image frame.  However, it is the location of the bits that correspond to each tile that is unknown.

If the decoder was able to find the start of a tile within a bit-stream, the decoder could then begin decoding the tile without referencing previously decoded data.  This would directly lead to parallel processing, as multiple decoders could then decode multiple portions of an image frame without reference to other decoding processes.

# Proposal

To enable the parallel decoding of tiles, we propose to transmit the locations of tile entry points in the bit-stream.  We propose two methods for the signaling.  The first signals the location data in the slice header.  This has the advantage of high coding efficiency, as our technique is designed to adjust the signaling overhead based on the number of bits in a tile.  (Thus, small rate frames will have lower overhead than higher rate frames.)  The drawback to the approach is latency at the encoder, as the encoder process must buffer the bit-stream in order to store the location data.  For applications sensitive to this latency issue, we provide a second approach that transmits a tile marker in the bit-stream.  These markers signal the entry point in the bit-stream.

For explicit signaling within the slice header the syntax and semantics are as follows:

Table 1 Slice header

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| tile_locations_idc | 1 | u(1) |
| if (tile_locations_idc == 1) { |  |  |
| tile_ locations () |  |  |
| } |  |  |
| } |  |  |

tile_locations_idc indicates the presence of tile information in the bit-stream  When not present, tile_locations_flag shall be 0.

Table 2 Tile location information

| tile_ locations() | C | Descriptor |
| --- | --- | --- |
| { |  |  |
| num_of_tiles_minus1_slice | 2 | u(5) |
| num_of_bits_minus1_for_tile_offset | 2 | u(5) |
| if( num_of_tiles_minus1_slice > 0 ) { |  |  |
| tile_offset[0] | 2 | ue(v) |
| for (i=1; i<num_of_tiles_minus1_slice; i++) { |  |  |
| tile_offset[i] | 2 | ue(v) |
| tile_offset[i] = tile_offset[i] + tile_offset[i-1] |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

num_of_tiles_minus1_slice defines the number of tile entry points in the slice

num_of_bits_minus1_for_tile_offset specifies the number of bits used to signal tile_offset [i] entries.

tile_offset [i] is the number of bytes between the first byte of tile i+1 and the fist byte of tile i.

For applications sensitive to latency issues, we provide a second approach that transmits markers in the bit-stream.  These markers signal the entry point in the bit-stream. Additionally, they include a tile header. We choose 0x000002 as a marker, but any reserved word would be acceptable.  The marker and tile header is not transmitted for the first tile within a slice.

Table 3 Tile header

| tile_header( ) { | C | Descriptor |
| --- | --- | --- |
| tile_idx_minus_1 | 1 | u(v) |
| } |  |  |

tile_idx_minus_1 is the order of the tile in raster scan order.  The first tile in the slice shall have a tile order index of 0.

# Experimental Results

For reference we use the following Tiles configuration for experiment 2 outlined in JCTVC-E408 []. In addition the encoder configuration is set to common test coditions specified in JCTVC-E700 [2].

|  | Reference |  |  |  |
| --- | --- | --- | --- | --- |
| Class | Number of tiles |  | Tile dimensions (in units of LCUs) |  |
|  | horizontally | vertically | horizontally | vertically |
| A | 5 | 5 | 8 | 5 |
| B | 6 | 3 | 5 | 6 |
| C | 2 | 2 | 7 | 4 |
| D | 1 | 1 | 7 | 4 |
| E | 4 | 3 | 5 | 4 |

The software used to generate anchor data was the HM-3.0 based tiles implementation released on the JCT-VC e-mail reflector [1]. It was further modified to incorporate tile location information within the bitstream.

## Explicitly signaling Tile start location within slice header

|  | All intra HE |  |  | All intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 | 0.1 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | Random access HE |  |  | Random access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1 | 0.4 | 0.4 | 0.3 | 0.2 | 0.3 |
| Class B | 0.2 | 0.5 | 0.5 | 0.4 | 0.4 | 0.3 |
| Class C | 0.1 | 0.3 | 0.2 | 0.2 | 0.2 | 0.2 |
| Class D | 0.0 | 0.3 | 0.4 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| Overall | 0.1 | 0.4 | 0.4 | 0.2 | 0.2 | 0.2 |
|  | Low delay (B) HE |  |  | Low delay (B) LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.4 | 0.7 | 1.2 | 0.4 | 0.4 | 0.4 |
| Class C | 0.2 | 0.3 | 0.5 | 0.2 | 0.2 | 0.2 |
| Class D | 0.1 | 0.3 | 0.4 | 0.0 | 0.0 | 0.0 |
| Class E | 1.7 | 2.4 | 2.1 | 1.8 | 1.7 | 1.8 |
| Overall | 0.5 | 0.8 | 1.0 | 0.5 | 0.5 | 0.5 |

## Implicitly signaling Tile start location using markers

|  | All intra HE |  |  | All intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 |
| Class B | 0.1 | 0.1 | 0.1 | 0.2 | 0.1 | 0.1 |
| Class C | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 |
| Overall | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 |
|  | Random access HE |  |  | Random access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.8 | 1.1 | 1.0 | 0.9 | 0.9 | 0.9 |
| Class B | 1.2 | 1.5 | 1.4 | 1.3 | 1.2 | 1.2 |
| Class C | 0.5 | 0.7 | 0.6 | 0.5 | 0.5 | 0.5 |
| Class D | 0.0 | 0.3 | 0.3 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| Overall | 0.6 | 0.9 | 0.9 | 0.7 | 0.7 | 0.7 |
|  | Low delay (B) HE |  |  | Low delay (B) LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 1.3 | 1.7 | 2.1 | 1.3 | 1.2 | 1.2 |
| Class C | 0.5 | 0.6 | 0.9 | 0.5 | 0.5 | 0.5 |
| Class D | 0.0 | 0.3 | 0.4 | 0.0 | 0.0 | 0.0 |
| Class E | 6.5 | 7.4 | 6.9 | 6.5 | 6.2 | 6.3 |
| Overall | 1.8 | 2.1 | 2.3 | 1.7 | 1.7 | 1.7 |

## Implicitly signaling Four Tile start locations

|  | All intra HE |  |  | All intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Random access HE |  |  | Random access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.4 | 0.7 | 0.6 | 0.6 | 0.5 | 0.5 |
| Class B | 0.1 | 0.4 | 0.4 | 0.3 | 0.3 | 0.3 |
| Class C | 0.4 | 0.6 | 0.5 | 0.4 | 0.4 | 0.4 |
| Class D | 0.0 | 0.3 | 0.3 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| Overall | 0.2 | 0.5 | 0.5 | 0.3 | 0.3 | 0.3 |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay (B) HE |  |  | Low delay (B) LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.3 | 0.6 | 1.1 | 0.3 | 0.3 | 0.3 |
| Class C | 0.4 | 0.5 | 0.8 | 0.4 | 0.4 | 0.4 |
| Class D | 0.0 | 0.3 | 0.4 | 0.0 | 0.0 | 0.0 |
| Class E | 1.7 | 2.4 | 2.1 | 1.9 | 1.8 | 1.9 |
| Overall | 0.5 | 0.8 | 1.0 | 0.5 | 0.5 | 0.5 |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

## Implicitly signaling Two Tile start location

|  | All intra HE |  |  | All intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Random access HE |  |  | Random access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.4 | 0.7 | 0.6 | 0.5 | 0.5 | 0.5 |
| Class B | 0.0 | 0.3 | 0.3 | 0.2 | 0.2 | 0.2 |
| Class C | 0.1 | 0.3 | 0.2 | 0.2 | 0.2 | 0.2 |
| Class D | 0.0 | 0.3 | 0.3 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| Overall | 0.1 | 0.4 | 0.4 | 0.2 | 0.2 | 0.2 |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay (B) HE |  |  | Low delay (B) LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1 | 0.5 | 0.9 | 0.2 | 0.2 | 0.2 |
| Class C | 0.2 | 0.3 | 0.5 | 0.2 | 0.2 | 0.2 |
| Class D | 0.0 | 0.3 | 0.4 | 0.0 | 0.0 | 0.0 |
| Class E | 0.8 | 1.4 | 1.1 | 1.0 | 1.0 | 1.0 |
| Overall | 0.2 | 0.6 | 0.7 | 0.3 | 0.3 | 0.3 |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

# Conclusion

We propose a method where the start of tile locations are communicated to the decoder within the bitstream. The primary goal of this proposal is to enable parallel tile decoding operations. Two signaling approaches are presented corresponding to two different encoder capabilities. Signaling based on location information within slice header (explicit) has lower bitrate overhead but requires bitstream buffering.

# References

[1] Arild Fuldseth , Michael Horowitz , Minhua Zhou, "Tiles," JCTVC-E408, 5th Meeting: Geneva, CH, 16-23 March, 2011.

[2] “Tiles software based on HM 3.0”, https://s3.amazonaws.com/ebrisk_ftp/tiles+related+files/Jul+2011+meeting+tiles+software/released+version/HM3.0-Tiles_v0.zip

[3] Frank Bossen, "Common test conditions and software reference configurations," JCTVC-E700, 5th Meeting: Geneva, CH, 16-23 March, 2011.

[4] Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

[5] K. Misra, A. Segall, “Tiles for Parallel Decoding”, JCTVC-E412, 5th Meeting: Geneva, CH, 16-23 March, 2011.

SHARP may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).