| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F595 |
| --- | --- |

| Title: | New results for guaranteeing 16-bit transform dynamic range |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra, Louie Kerofsky, Andrew Segall 5750 Pacific Rim Blvd Camas, WA 98607 USA | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com lkerofsky@sharplabs.com asegall@sharplabs.com |
| Source: | Sharp Laboratories of America, Inc. |  |  |

_____________________________

# Abstract

This document reports new results for limiting the dynamic range within an inverse transform.  It is asserted that the method guarantees a 16-bit dynamic range limit within the transpose buffer.  Additionally, it is asserted that the current HM3.0 design does not guarantee this 16-bit limit.  The proposed technique was previously proposed in JCTVC-D071/JCTVC-E411.  Here, the technique is adapted to HM3.1 which includes as default the transform proposed in JCTVC-E243.  The proposed approach shows a BD-rate impact of 0.0% on coding efficiency for common test conditions. Further evaluation for CE10 configuration settings on HM3.0 demonstrates BD bitrate increase of 0.0%.

# Introduction

It is often desirable to have a guaranteed small dynamic range, say 16-bits, between the two 1-D inverse transforms being used in a video codec. In JCTVC-D071 [1], JCTVC-E411 [2] a method consisting of QP-period based scaling, clipping and transform size dependent bit shifts was used to achieve the desired 16-bit dynamic range. In this proposal, we modify the approach for HM3.0 [3], HM 3.1 [4] that includes the transform proposed in JCTVC-E243 [5].  The method of JCTVC-E243 enables decoding with 16-bit intermediates for ‘typical’ operation based on forward transform and reasonable quantization.  We note that 16-bit coefficient data can result in intermediate data exceeding 16-bits in the primary design of JCTVC-E243 in the sense that valid coefficient data can generate these high dynamic range intermediate values.  We note a remark in JCTVC-E243 about the potential to include intermediate clipping to allow slightly higher precision in some calculations of the inverse transform.

The parameters used in the rest of this document are:

QP			: Quantization Parameter

Remainder	: (QP + 6*BitIncrement) % 6

Period 		: (QP + 6*BitIncrement) / 6

BitIncrement: Bit depth Increment

Descale		: is a function of Remainder

>> x		: is right bit-shift by x

N			: Transform size

# Proposed Approach

Figure 1 shows a block diagram representation of the current inverse quantizer and inverse transform with the proposed modification labeled in red. The 16-bit clipping between the two 1-D inverse transforms guarantees the desired dynamic range.

Figure 1: Block diagram representation of inverse quantizer and inverse transform with proposed 16-bit clipping (in red).

# Possibility of intermediates greater than 16-bits.

Without the 16-bit clipping it is possible that a set of levels can result in the 16-bit dynamic range being violated. For example, if N=8, QP=12 and all level values are:

819819819...819819819819...819819819819...819...............819819819...819

Then the input coefficients to the first inverse transform is:

327603276032760...32760327603276032760...32760327603276032760...32760...............327603276032760...32760

However, the output of the first inverse transform takes on values:

122594−3301625850−947014077−179289583839122594−3301625850−947014077−179289583839122594−3301625850−947014077−179289583839122594−3301625850−947014077−179289583839122594−3301625850−947014077−179289583839122594−3301625850−947014077−179289583839122594−3301625850−947014077−179289583839122594−3301625850−947014077−179289583839

In this specific example, the first two columns contain data which violates the 16-bit dynamic range.

A detailed analysis of the intermediate dynamic range resulting from the JCTVC-E243[9] design was performed using the methods of JCTVC-E333[10].  From this analysis we have the following plot that indicates the dynamic range of intermediates at a given quantization error magnitude for each transform block size.

For no quantization i.e. zero bits, the intermediate dynamic range is only 15-bits regardless of block size.  Once a small amount of quantization is introduced, an additional bit is needed requiring 16-bits for the intermediates.  In the extreme case where the coefficient data can be any 16-bit value, 16-bit quantization error, the necessary intermediate dynamic range varies based on block size but exceeds 16-bits for all block sizes.

We have generated bitstreams whose coefficient data requires only 16-bits of precision but whose intermediates exceed 16-bits as potentially indicated by the plot.  The current software represents intermediate values as a short data type.  A complier will apply a cast from long to short data types which modifies values exceeding 16-bits of precision.  Different architectures and compilers may handle this overflow differently resulting different reconstructed residual values.

# Results

We implemented the proposed transform dynamic range restriction algorithm in HM 3.0 [3] / HM 3.1 [4]. We then coded all sequences using the configuration files described in JCTVC-C700 [6] and JCTVC-C710 [7]. The run times of the software varies considerably as the simulations were run on a shared cluster resource. Consequently, the encoding and decoding times are omitted in the summary tables below. Full results are available in the attached XLS sheets. The rate-distortion summary is as follows:

## Common test conditions

### QP: 22, 27, 32, 37

|  | All intra HE |  |  | All intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | Random access HE |  |  | Random access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | Low delay (B) HE |  |  | Low delay (B) LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |

## CE10 test conditions

### Low QP: 1, 5, 9, 13

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |

### High QP: 36, 42, 47, 51

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |

Here, we observe that the impact on BD bitrate [8] is 0.0% for all test conditions. We assert that the proposed approach enables design of systems which require a lower memory bus width and size.

# Conclusion

The method proposed in JCTVC-D071/JCTVC-E411 to guarantee 16-bit dynamic range was modified and evaluated for the new common test conditions and configurations proposed in CE10 performance evaluation. The memory bus width and size requirement of the inverse transform is reduced at no cost in coding efficiency.

# Patent rights declaration(s)

SHARP may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] Kiran Misra, Louis Kerofsky, Andrew Segall, “On transform dynamic range,” JCTVC-D071/ m18821, 4th Meeting: Daegu, KR, 20-28 January, 2011.

[2] Kiran Misra, Louis Kerofsky, Andrew Segall, “Enforcing the 16-bit inverse transform dynamic range,” JCTVC-E411/ m19949, 5th Meeting: Geneva, CH, 16-23 March, 2011.

[3] High efficiency test model (HM) version 3.0 URL: https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-3.0/

[4] High efficiency test model (HM) version 3.1 URL: https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-3.1/

[5] A. Fuldseth, G. Bjøntegaard (Cisco), M. Sadafale, M. Budagavi (TI),	"Transform design for HEVC with 16 bit intermediate data representation," JCTVC-E24/m19767, 5th Meeting: Geneva, CH, 16-23 March, 2011.

[6] Frank Bossen, "Common test conditions and software reference configurations," JCTVC-D700/m20235, 5th Meeting: Geneva, CH, 16-23 March, 2011.

[7] P. Topiwala, M. Budagavi, A. Fuldseth, R. Joshi, I.-K. Kim, "Description of Core Experiment 10 (CE10): Core Transform Design," JCTVC-E710/m20234, 5th Meeting: Geneva, CH, 16-23 March, 2011.

[8] Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

[9] A. Fuldseth, Gisle Bjøntegaard, M. Sadafale, M. Budagavi, "Transform design for HEVC with 16 bit intermediate data representation," JCTVC-E243/m19767, 5th Meeting: Geneva, 16-23 March, 2011.

[10] Louis Kerofsky, Kiran Misra, Andrew Segall, "Transform Dynamic Range Analysis," JCTVC-E333/m19861, 5th Meeting: Geneva, 16-23 March, 2011.