| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F596r2 |
| --- | --- |

| Title: | The effect of LCU size on coding efficiency in the context of MTU size matching |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Michael Horowitz Shilin Xu 1450-1055 W. Hastings Street Vancouver, BC V6E 2E9 Canada Eun S. Ryu Yan Ye 9710 Scranton Rd, Suite 250 San Diego, CA 92121 USA | Tel: Email: Tel: Email: | +1 778.331.3432 x323 michael@ebriskvideo.com +1 858.210.4803 yan.ye@interdigital.com |
| Source: | eBrisk Video and InterDigital Communications |  |  |

_____________________________

# Abstract

This contribution contains information showing the effect of LCU size on coding efficiency in the context of encoding with a maximum transmission unit (MTU) size constraint. Results are presented comparing a simulation configured to use 1500-byte slices and a separate simulation using 1500-byte slices and tiles each against an HM 3.0 anchor that was configured to use one slice per picture. Separate simulations were run for each of three LCU sizes: 16x16, 32x32, and 64x64. Results show that the coding efficiency loss of 1500-byte per slice relative to the single slice anchor grows with decreasing LCU size. In addition, simulations featuring 1500-byte slices and tiles show higher coding efficiency in nearly all cases than those where only 1500-byte slices were used. It is also noted that gains using tiles and 1500-byte slices relative to using 1500-byte slices only increase with decreasing LCU size.

# Introduction

When transporting a coded bitstream on a packet switched network (e.g., an IP network), packets are subject to a maximum transmission unit (MTU) size constraint, the size (in bytes) of the largest protocol data unit that the network can handle. To minimize overhead due to packet headers and ensure a high degree of error resilience, it can be advantageous to match the size of coded slices to the MTU size and transport one slice per packet. Applications required to partition a picture into multiple slices to match a network MTU size pay a price in terms of reduced coding efficiency compared with those applications that code one slice per picture. This contribution describes an experiment and presents results in which the coding efficiency when imposing a 1500-byte per slice constraint (i.e., the MTU size of an Ethernet v2 network) without, and separately, with tiles [1] are compared against an HM 3.0 version 823 anchor [2] configured to use one slice per picture for each of three LCU sizes: 16x16, 32x32, and 64x64. As is discussed in greater detail later, the experiment results show that the coding efficiency loss of 1500-byte per slice relative to the single slice anchor grows with decreasing LCU size. In addition, the use of Tiles and slices nearly always results in improved coding efficiency relative to using slices only. Further, coding gains realized when using tiles and slices relative to using slices only increase with decreasing LCU size.

Section 2 of the contribution contains a description of the experiments. A summary of results from the experiments is presented in Section 3. Sections 4 and 5 contain the conclusion and references respectively.

# Description of the experiments

This section contains a description of experiments designed to investigate the effect of LCU size on coding efficiency in the context of encoding with an MTU size constraint. The HM 3.0 software version 823 using the common conditions [3] using one slice per picture served as the experiments’ anchor. For simulations using LCU sizes 16x16 and 32x32, MTK_SAO was set equal to 0 due to feature compatibility issues. For LCU size 64x64, HM 3.0 was used without any modifications. Two test cases were considered. For the first test case, the encoder was configured to use 1500-byte slices. For the second case, modified encoder software was configured to use 1500-byte slices and tiles. The modified encoder software, made available to JCT-VC community in April, 2011, added tiles functionality on top of HM 3.0 version 823 [4]. In this test case, the column widths of the tiles were determined, based on a computation of the average number of LCU’s per 1500-byte slice with the goal of producing slices with a square-ish shape. It is expected that a square slice shape will yield higher coding efficiency compared with a very long and narrow slice that would likely result from the use of slices without tiles. The column widths for each sequence class/QP combination, for each simulation type (e.g., intra only), is shown in Table 1. For a given sequence class/QP combination, the same column width was used for all LCU sizes unless otherwise noted. Simulations for the anchor and the two test cases (i.e., slices and tiles and slices only) were run three times, one time for each of the three LCU sizes.

|  | Intra HE | Intra LoCo | Random Access HE | Random Access LD | Low Delay HE | Low Delay LoCo |
| --- | --- | --- | --- | --- | --- | --- |
| Class A |  |  |  |  |  |  |
| QP 22 | 128/128/2560 | 128/128/2560 | 320 | 256 | --- | --- |
| QP 27 | 128/128/2560 | 128/128/2560 | 448 | 384 | --- | --- |
| QP 32 | 256 | 256 | 640 | 512 | --- | --- |
| QP 37 | 256 | 256 | 640 | 640 | --- | --- |
| Class B |  |  |  |  |  |  |
| QP 22 | 112/128/1920 | 112/128/1920 | 384 | 384 | 256 | 384 |
| QP 27 | 320 | 320 | 384 | 384 | 512 | 512 |
| QP 32 | 384 | 384 | 512 | 512 | 640 | 640 |
| QP 37 | 512 | 512 | 640 | 640 | 960 | 960 |
| Class C |  |  |  |  |  |  |
| QP 22 | 112/96/832 | 112/96/832 | 256 | 192 | 192 | 192 |
| QP 27 | 144/128/832 | 144/128/832 | 448 | 256 | 256 | 256 |
| QP 32 | 192 | 192 | 448 | 448 | 448 | 448 |
| QP 37 | 256 | 256 | 832 | 832 | 832 | 832 |
| Class D |  |  |  |  |  |  |
| QP 22 | 80/96/416 | 64/96/416 | 256 | 256 | 256 | 256 |
| QP 27 | 112/96/416 | 112/96/416 | 416 | 416 | 256 | 256 |
| QP 32 | 128 | 128 | 416 | 416 | 416 | 416 |
| QP 37 | 256 | 256 | 416 | 416 | 416 | 416 |
| Class E |  |  |  |  |  |  |
| QP 22 | 256 | 256 | --- | --- | 448 | 448 |
| QP 27 | 320 | 320 | --- | --- | 1280 | 1280 |
| QP 32 | 320 | 320 | --- | --- | 1280 | 1280 |
| QP 37 | 448 | 448 | --- | --- | 1280 | 1280 |

Table 1. Column widths in units of sample number for each sequence class/QP combination.

# Results

This section contains a summary of results from experiments the comparing coding efficiency of pictures partitioned into 1500 byte slices against anchors in which one slice per picture was used. Three separate cases were considered (using three separate anchors): one in which the LCU size was set to 16x16, another with LCU size 32x32 and one for LCU size 64x64. In each case, results are presented comparing the anchor to a 1500-byte per slice encoding and to a 1500-byte per slice and tiles encoding each using the same LCU size as the anchor. Detailed results are presented in the accompanying spreadsheets. The simulations were run on separate machines so the encode and decode times are not comparable.

The results show that the coding efficiency loss of a 1500-byte per slice encoding relative to the single slice anchor grows with decreasing LCU size. This observation should not be confused with the fact that the coding efficiency of the single slice anchors increases with increasing LCU size (which is also true). We believe the coding efficiency loss increase with decreasing LCU size results from the fact that smaller LCU sizes create longer and narrower slices. Narrower slices have more slice boundary for a given unit of slice area. More slice boundaries result, in general, in more broken dependencies and therefore lower coding efficiency.

Also of note, the simulations using slices and tiles showed higher coding efficiency in nearly all cases compared to those where only slices were used. In many cases, the efficiency of tiles and slices was significantly higher. In an observation related to above discussion, we note that the gains using tiles and slices vs. slices only increase with decreasing LCU size.

|  | Intra HE |  | Intra LoCo |  |
| --- | --- | --- | --- | --- |
|  | Y BD-rate |  | Y BD-rate |  |
|  | Slices | Slices+Tiles | Slices | Slices+Tiles |
| Class A | 4.6 | 4.5 | 4.0 | 3.9 |
| Class B | 5.4 | 4.6 | 5.1 | 4.4 |
| Class C | 5.3 | 4.7 | 5.5 | 5.0 |
| Class D | 3.5 | 3.3 | 4.0 | 3.8 |
| Class E | 7.7 | 5.2 | 7.4 | 4.7 |
| All | 5.2 | 4.5 | 5.1 | 4.3 |
| Enc Time[%] | --- | --- | --- | --- |
| Dec Time[%] | --- | --- | --- | --- |
|  | Random access HE |  | Random access LoCo |  |
|  | Y BD-rate |  | Y BD-rate |  |
|  | Slices | Slices+Tiles | Slices | Slices+Tiles |
| Class A | 5.7 | 5.5 | 4.1 | 3.8 |
| Class B | 4.6 | 4.0 | 4.3 | 3.6 |
| Class C | 3.9 | 3.6 | 3.9 | 3.6 |
| Class D | 2.4 | 2.4 | 2.5 | 2.5 |
| Class E |  |  |  |  |
| All | 4.2 | 3.9 | 3.8 | 3.4 |
| Enc Time[%] | --- | --- | --- | --- |
| Dec Time[%] | --- | --- | --- | --- |

|  | Low delay HE |  | Low delay LoCo |  |
| --- | --- | --- | --- | --- |
|  | Y BD-rate |  | Y BD-rate |  |
|  | Slices | Slices+Tiles | Slices | Slices+Tiles |
| Class A |  |  |  |  |
| Class B | 3.4 | 3.0 | 3.0 | 2.4 |
| Class C | 2.6 | 2.4 | 2.5 | 2.2 |
| Class D | 1.6 | 1.6 | 1.7 | 1.6 |
| Class E | 1.6 | 1.6 | 1.3 | 1.3 |
| All | 2.4 | 2.2 | 2.2 | 2.0 |
| Enc Time[%] | --- | --- | --- | --- |
| Dec Time[%] | --- | --- | --- | --- |

Table 2. Results comparing a 1500-byte per slice encoding against a one slice per picture anchor for a 64x64 LCU size.

|  | Intra HE |  | Intra LoCo |  |
| --- | --- | --- | --- | --- |
|  | Y BD-rate |  | Y BD-rate |  |
|  | Slices | Slices+Tiles | Slices | Slices+Tiles |
| Class A | 5.8 | 4.5 | 5.1 | 3.9 |
| Class B | 7.2 | 4.4 | 7.1 | 4.2 |
| Class C | 7.0 | 4.2 | 7.3 | 4.4 |
| Class D | 4.5 | 3.3 | 4.9 | 3.6 |
| Class E | 9.5 | 4.8 | 9.6 | 4.4 |
| All | 6.7 | 4.2 | 6.7 | 4.1 |
| Enc Time[%] | --- | --- | --- | --- |
| Dec Time[%] | --- | --- | --- | --- |
|  | Random access HE |  | Random access LoCo |  |
|  | Y BD-rate |  | Y BD-rate |  |
|  | Slices | Slices+Tiles | Slices | Slices+Tiles |
| Class A | 6.5 | 5.4 | 5.7 | 3.9 |
| Class B | 5.8 | 3.8 | 5.9 | 3.6 |
| Class C | 4.7 | 3.8 | 4.8 | 3.7 |
| Class D | 2.8 | 2.8 | 2.8 | 2.8 |
| Class E |  |  |  |  |
| All | 5.0 | 3.9 | 4.9 | 3.5 |
| Enc Time[%] | --- | --- | --- | --- |
| Dec Time[%] | --- | --- | --- | --- |
|  | Low delay HE |  | Low delay LoCo |  |
|  | Y BD-rate |  | Y BD-rate |  |
|  | Slices | Slices+Tiles | Slices | Slices+Tiles |
| Class A |  |  |  |  |
| Class B | 3.9 | 3.0 | 3.8 | 2.5 |
| Class C | 2.7 | 2.3 | 2.9 | 2.2 |
| Class D | 1.8 | 1.9 | 1.7 | 1.6 |
| Class E | 1.5 | 1.5 | 1.3 | 1.2 |
| All | 2.6 | 2.2 | 2.6 | 2.0 |
| Enc Time[%] | --- | --- | --- | --- |
| Dec Time[%] | --- | --- | --- | --- |

Table 3. Results comparing a 1500-byte per slice encoding against a one slice per picture anchor for a 32x32 LCU size.

|  | Intra HE |  | Intra LoCo |  |
| --- | --- | --- | --- | --- |
|  | Y BD-rate |  | Y BD-rate |  |
|  | Slices | Slices+Tiles | Slices | Slices+Tiles |
| Class A | 8.4 | 3.9 | 8.2 | 3.7 |
| Class B | 9.7 | 3.9 | 10.2 | 4.1 |
| Class C | 9.0 | 3.9 | 9.7 | 4.2 |
| Class D | 5.1 | 3.1 | 5.7 | 3.5 |
| Class E | 10.2 | 4.4 | 11.0 | 4.3 |
| All | 8.5 | 3.8 | 8.9 | 4.0 |
| Enc Time[%] | --- | --- | --- | --- |
| Dec Time[%] | --- | --- | --- | --- |
|  | Random access HE |  | Random access LoCo |  |
|  | Y BD-rate |  | Y BD-rate |  |
|  | Slices | Slices+Tiles | Slices | Slices+Tiles |
| Class A | 7.8 | 4.8 | 8.3 | 4.1 |
| Class B | 7.5 | 3.8 | 8.2 | 3.6 |
| Class C | 5.6 | 4.0 | 5.9 | 3.8 |
| Class D | 3.0 | 2.9 | 3.0 | 2.9 |
| Class E |  |  |  |  |
| All | 6.1 | 3.9 | 6.5 | 3.7 |
| Enc Time[%] | --- | --- | --- | --- |
| Dec Time[%] | --- | --- | --- | --- |
|  | Low delay HE |  | Low delay LoCo |  |
|  | Y BD-rate |  | Y BD-rate |  |
|  | Slices | Slices+Tiles | Slices | Slices+Tiles |
| Class A |  |  |  |  |
| Class B | 4.5 | 2.9 | 5.0 | 2.9 |
| Class C | 3.1 | 2.2 | 3.4 | 2.3 |
| Class D | 1.8 | 1.8 | 1.8 | 1.7 |
| Class E | 1.7 | 1.7 | 2.6 | 2.5 |
| All | 2.9 | 2.2 | 3.3 | 2.3 |
| Enc Time[%] | --- | --- | --- | --- |
| Dec Time[%] | --- | --- | --- | --- |

Table 4. Results comparing a 1500-byte per slice encoding against a one slice per picture anchor for a 16x16 LCU size.

# Conclusion

This contribution describes experiments designed to investigate the relative coding efficiency of a single slice anchor compared with an encoding in which the picture is partitioned into 1500-byte slices. Separate experiments were conducted for each of three LCU sizes: 16x16, 32x32, and 64x64. The results show that the coding efficiency loss of 1500-byte per slice encodings relative to the single slice anchor grows with decreasing LCU size. A result that is notably different from the well-known fact that coding efficiency of the single slice anchors increases with increasing LCU size (which is also true). We believe the coding efficiency loss increase with decreasing LCU size results from the fact that smaller LCU sizes create longer, narrower slices. It was also shown that simulations using 1500-byte slices and tiles had higher coding efficiency in nearly all cases to those where only 1500-byte slices were used. Finally, it was shown that coding gains using tiles and 1500-byte slices relative to using 1500-byte slices only increased with decreasing LCU size.

# References

[1] A. Fuldseth, M. Horowitz, S. Xu, A. Segall, and M. Zhou, “Tiles,” JCTVC-F335, Torino, IT, July 2011.

[2] JCT-VC, “Test Model HM 3.0”, http://hevc.kw.bbc.co.uk/svn/jctvc-a124/branches/HM-3.0-dev/

[3] F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-E700, Geneva, March 2011.

[4] “Tiles software based on HM 3.0”, https://s3.amazonaws.com/ebrisk_ftp/tiles+related+files/Jul+2011+meeting+tiles+software/released+version/HM3.0-Tiles_v0.zip

# Patent rights declaration(s)

eBrisk Video and InterDigital Communications may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Entries having the form a/b/c show column widths for cases a, b, and c, corresponding to16x16, 32x32, and 64x64 LCU sizes respectively. Entries having a single number used one column width for all three LCU sizes.