| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F598 |
| --- | --- |

| Title: | Adaptive significance map coding for large transform |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yinji Piao Junghye Min Jianle Chen | Tel: Email: | +82-31-2798832 yj1003.piao@samsung.com jh643@samsung.com jianle.chen@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes an adaptive context type selection method for significance map coding. The proposed method selects the best context model type according to coefficient distributions. The selected context model type is signaled to decoder for each transform unit when the coefficient distribution satisfied a given condition. The method provides 0.3% gain overall and 0.6% for class A in HE, All Intra configuration.

# Introduction

For large transforms, the significance maps have various distribution patterns as can be seen in Figure 1.  In HM3.0, the context model for significance map coding is selected based on the sum of the significance in the predefined neighborhood [1][2]. This approach provides highly adaptive feature for context selection which is effective for large transforms with many variations in coefficient distributions.

But even in large transforms, coefficient distributions are still depending on the coefficient positions in the transform units. The second image in Figure 1 shows typical coefficient distributions with more dense probability in low frequency area.

For most of cases, the context selection scheme in current HM does not consider coefficient’s position which is also very important factor for separation of the different coefficient distributions. Therefore, exploiting both neighborhood dependency and coefficient positions can be a natural way to improve the coding efficiency for coefficient coding. The proposed method selects and sends the best context model type to decoder which is not serious burden for large transforms.

Figure 1. Significance map of large transforms

Figure 2. Two context types for significance flag

# Adaptive context selection for significance map coding

Two types of context (Figure 2) are used for significance map coding of large transforms in this proposal. One is current neighborhood-based context, and the other is position-based context which is used in 4x4 and 8x8 transform. Since the statistics of each unit is different at 16x16 and 32x32 transform cases, 16 additional position-based contexts are added both to 16x16 and 32x32. Considering that residuals from chroma component are rarely distributed in high frequency positions, the position-based context is only applied to luma component.

The context type to be used should be indicated to the decoder. In proposal method, the alternate context is only considered when last non-zero coefficient locates outside the predefined region as marked in Figure 3 to reduce unnecessary signaling for context type. This is possible because last coefficient position is parsed before significance map in current HM3.0. The threshold here is set to Threshold=TUsize>>1.

Figure 3. Region for context type signaling

Table I. Number of context for significance map coding

# Test results

Adaptive context selection was implemented on top of HM3.0 s/w only for I_SLICE and tested under common test conditions [3]. The SIMPLE_CONTEXT_SIG macro was turned off in the proposed method since the context sharing in 16x16 and 32x32 is not efficient when location-based context was additionally included. So the number of contexts used for significance map coding is increased by 32 comparing to previous HM2.0 (Table I).

As shown in Table II, adaptive context type selection provides 0.3% bit savings on average and 0.6% gain on class A where the maximum gain 1.1% is from Nebuta sequence.

Table II. Test results for adaptive context selection in HM3.0.

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.6 | -0.5 | -0.5 | -0.2 | -0.4 | -0.8 |  |  |  |
| Class B | -0.3 | -0.2 | -0.2 | -0.1 | -0.1 | 0.0 | 0.0 | 0.5 | 0.0 |
| Class C | -0.1 | -0.2 | -0.2 | 0.0 | 0.0 | -0.2 | 0.1 | 0.1 | 0.1 |
| Class D | -0.1 | -0.2 | -0.2 | 0.0 | -0.1 | -0.3 | 0.1 | 0.6 | 1.2 |
| Class E | -0.1 | -0.1 | 0.0 |  |  |  | 0.1 | 1.8 | 0.4 |
| Overall | -0.3 | -0.2 | -0.2 | -0.1 | -0.1 | -0.3 | 0.1 | 0.7 | 0.4 |
| Enc Time[%] | 106% |  |  | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  | 100% |  |  |

# Conclusions

An enhanced context type selection based significance map coding method is proposed. Our result shows that current significance map coding method is not optimal even though it is an efficient tool for coefficient coding of large transforms. The increased number of contexts may bring more complexity in hardware aspect, but combining position information with current neighborhood dependent context has shown potential to further improve significance map coding. We recommend our proposal to be a candidate as a part of the next HM or CE for further investigation.

# References

- “Description of video coding technology proposal by Fraunhofer HHI”, JCTVC-A116, 1st JCT-VC Meeting, Dresden, DE, April 2010.

- “Parallel processing friendly simplified context selection of significance map”, JCTVC-D260, 4th JCT-VC Meeting, daegu, January 2011.

- F. Bossen Common HM test conditions and software reference configurations JCTVC-E700, 5th. JCTVC, Meeting, Geneva, Mar. 2011.

# Patent rights declaration(s)

Samsung Electronics, Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).