| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F599 |
| --- | --- |

| Title: | Chroma interpolation filters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Koohyar Minoo Jian Lou Ajay Luthra 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-2521 kminoo@motorola.com +1-858-404-3427 lou@motorola.com +1-858-404-3470 aluthra@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

This document proposes a set of interpolation filters for sub-pxiel motion compensation of Chroma samples. The choice of interpolation filters are selected to match the proposed Luma sub-pixel interpolation filters in JCTVC-F574.

# Introduction

In JCTVC-F574 [1], Motorola Mobility has proposed a set of sub-pixel interpolation filters which improves the average coding efficiency amongst the 6 relevant anchor conditions. The design criteria for those Luma filters require a re-design of corresponding Chroma filters. This document introduces two sets of Chroma filters which match the proposed Luma interpolation filters in section 2.2 and 2.3 of JCTVC-F574.

Fig. 1 illustrates the positions of the resulting sub-pixel positions between full-pel pixels along a pixel line within an image, where L1, L0, R0, and R1 are full-pel pixels, H is a half-pel pixel between L0 and R0, FL0, FL1, and FL2 are fractional-pel pixels between L0 and H, and FR0, FR1, and FR2 are fractional-pel pixels between H and R0.

Fig. 1. Fractional-Pel Positions

In the current HEVC version, the Chroma interpolation operations are defined as

FL0 = (-3*L1 + 60*L0 + 8*R0 - 1*R1 + 32) >> 6;

FL1 = (-4*L1 + 54*L0 + 16*R0 - 2*R1 + 32) >> 6;

FL2 = (-5*L1 + 46*L0 + 27*R0 - 4*R1 + 32) >> 6;

H     = (-4*L1 + 36*L0 + 36*R0 - 4*R1 + 32) >> 6;

FR2 = (-4*L1 + 27*L0 + 46*R0 - 5*R1 + 32) >> 6;

FR1 = (-2*L1 + 16*L0 + 54*R0 - 4*R1 + 32) >> 6;

FR0 = (-1*L1 + 8*L0 + 60*R0 - 3*R1 + 32) >> 6;

# Proposed sub-pel interpolation filters

## 4-tap Interpolation filters, set 1:

A 4-tap FIR filter with 6-bit coefficients is proposed to match the proposed Luma filters in section 2.2 of JCTVC-F574.

FL0 = (-2*L1 + 61*L0 + 6*R0 - 1*R1 + 32) >> 6;

FL1 = (-4*L1 + 54*L0 + 16*R0 - 2*R1 + 32) >> 6;

FL2 = (-4*L1 + 42*L0 + 30*R0 - 4*R1 + 32) >> 6;

H     = (-4*L1 + 36*L0 + 36*R0 - 4*R1 + 32) >> 6;

FR2 = (-4*L1 + 30*L0 + 42*R0 - 4*R1 + 32) >> 6;

FR1 = (-2*L1 + 16*L0 + 54*R0 - 4*R1 + 32) >> 6;

FR0 = (-1*L1 + 6*L0 + 61*R0 - 2*R1 + 32) >> 6;

## 4-tap Interpolation filters, set 2:

A 4-tap FIR filter with 6-bit coefficients is proposed to match the proposed Luma filters in section 2.3 of JCTVC-F574.

FL0 = (-2*L1 + 61*L0 + 6*R0 - 1*R1 + 32) >> 6;

FL1 = (-4*L1 + 54*L0 + 16*R0 - 2*R1 + 32) >> 6;

FL2 = (-5*L1 + 44*L0 + 29*R0 - 4*R1 + 32) >> 6;

H     = (-4*L1 + 36*L0 + 36*R0 - 4*R1 + 32) >> 6;

FR2 = (-4*L1 + 29*L0 + 44*R0 - 5*R1 + 32) >> 6;

FR1 = (-2*L1 + 16*L0 + 54*R0 - 4*R1 + 32) >> 6;

FR0 = (-1*L1 + 6*L0 + 61*R0 - 2*R1 + 32) >> 6;

# Simulation environment

## Software

The software used to run the simulations is HM3.0 [2]. The binaries were compiled using Microsoft Visual Studio 2008 64bit. The coding conditions used for the simulations are specified in [3]. YUV file output during the decoding was disabled when collecting the decoding time statistics.

## Hardware

Two sets of PCs with identical settings were used for simulations, one set with Microsoft Windows 7 64bits, Intel Xeon 3.0GHz with 8G memory and one set with Microsoft Windows 7 64bits, Intel Xeon 3.2GHz with 12G memory. The anchor results from the PCs with the same settings are used for comparison. Therefore, both the encoding time and decoding time comparisons are reliable.

# Simulation Results

The encoder reconstructed YUV files match the decoder reconstructed YUV files. The experimental results of the rate-distortion and speed comparison with HM3.0 anchor are summarized in Table 1 and 2. The experimental results of the rate-distortion and speed comparison with F574 using the same Luma interpolation filters are summarized in Table 3 and 4 (The experiments for Set 1 Chroma and case 2.2 of JCTVC-F574 were not conducted on the same set of PCs, so the speed comparisons are not provided).

Table 1 Rate-distortion and speed performance for Set 1 Chroma and case 2.2 of JCTVC-F574 interpolation filters comparing with HM3.0 anchor

|  |  |  | Random access |  |  |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate |  | U BD-rate | V BD-rate |  | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | -0.1 |  | 0.1 | 0.1 |  | -0.3 |  | 0.8 | 0.9 |  |
| Class B | -0.2 |  | 0.1 | 0.2 |  | -0.3 |  | 0.3 | 0.2 |  |
| Class C | -0.1 |  | 0.3 | 0.2 |  | 0.3 |  | 0.4 | 0.3 |  |
| Class D | -0.2 |  | 0.5 | 0.5 |  | 0.8 |  | 0.8 | 1.0 |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| All | -0.2 |  | 0.2 | 0.2 |  | 0.1 |  | 0.6 | 0.6 |  |
| Enc Time[%] | 97% |  |  |  |  | 96% |  |  |  |  |
| Dec Time[%] | 98% |  |  |  |  | 97% |  |  |  |  |
|  | Low delay |  |  |  |  | Low delay LC |  |  |  |  |
|  | Y BD-rate |  | U BD-rate | V BD-rate |  | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.3 |  | 0.3 | 0.4 |  | -0.7 |  | 0.3 | 0.4 |  |
| Class C | -0.1 |  | 0.2 | 0.3 |  | 0.3 |  | 0.7 | 0.5 |  |
| Class D | 0.2 |  | 0.5 | 0.8 |  | 0.8 |  | 1.5 | 1.7 |  |
| Class E | -0.5 |  | -0.6 | -1.7 |  | -1.7 |  | -1.2 | -2.1 |  |
| All | -0.2 |  | 0.2 | 0.1 |  | -0.3 |  | 0.4 | 0.3 |  |
| Enc Time[%] | 98% |  |  |  |  | 97% |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  | 98% |  |  |  |  |
|  | Low delay (P) |  |  |  |  | Low delay LC (P) |  |  |  |  |
|  | Y BD-rate |  | U BD-rate | V BD-rate |  | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.8 |  | -0.4 | -0.6 |  | -1.5 |  | -0.1 | 0.0 |  |
| Class C | -1.0 |  | -0.4 | -0.4 |  | -0.8 |  | 0.2 | 0.2 |  |
| Class D | -0.8 |  | -0.1 | -0.8 |  | -0.6 |  | 1.2 | 0.9 |  |
| Class E | -1.4 |  | -0.4 | -1.4 |  | -3.3 |  | -1.3 | -1.7 |  |
| All | -0.9 |  | -0.3 | -0.8 |  | -1.5 |  | 0.1 | 0.0 |  |
| Enc Time[%] | 98% |  |  |  |  | 97% |  |  |  |  |
| Dec Time[%] | 98% |  |  |  |  | 97% |  |  |  |  |

Table 2 Rate-distortion and speed performance for Set 2 Chroma and case 2.3 of JCTVC-F574 interpolation filters comparing with HM3.0 anchor

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.3 | -0.1 | 0.2 | 0.3 |
| Class B | -0.1 | 0.0 | 0.1 | -0.3 | 0.1 | 0.0 |
| Class C | -0.2 | 0.0 | 0.0 | -0.4 | 0.1 | -0.1 |
| Class D | -0.2 | -0.1 | -0.1 | -0.5 | 0.0 | 0.2 |
| Class E |  |  |  |  |  |  |
| All | -0.1 | 0.0 | 0.1 | -0.3 | 0.1 | 0.1 |
| Enc Time[%] | 97% |  |  | 97% |  |  |
| Dec Time[%] | 101% |  |  | 98% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.2 | 0.5 | -0.2 | 0.3 | 0.3 |
| Class C | -0.1 | 0.1 | 0.0 | -0.6 | 0.6 | 0.6 |
| Class D | -0.4 | -0.4 | -0.7 | -0.7 | 1.0 | 1.1 |
| Class E | 0.4 | 0.0 | -0.5 | 0.4 | 0.0 | -1.2 |
| All | 0.0 | 0.0 | -0.1 | -0.3 | 0.5 | 0.3 |
| Enc Time[%] | 97% |  |  | 97% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | Low delay (P) |  |  | Low delay LC (P) |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.5 | -0.2 | -0.2 | -0.1 | 0.4 | 0.3 |
| Class C | -1.2 | -0.5 | -0.7 | -1.0 | 0.5 | 0.5 |
| Class D | -1.6 | -1.1 | -1.1 | -1.5 | 1.0 | 0.4 |
| Class E | -0.7 | 0.0 | -0.4 | 1.0 | 0.1 | 0.2 |
| All | -1.0 | -0.5 | -0.6 | -0.5 | 0.5 | 0.4 |
| Enc Time[%] | 97% |  |  | 98% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |

Table 3 Rate-distortion and speed performance for Set 1 Chroma and case 2.2 of JCTVC-F574 interpolation filters comparing with F599 using the same Luma interpolation filters

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | 0.0 | 0.1 | 0.5 | 0.4 |
| Class B | 0.0 | 0.0 | 0.1 | 0.0 | 0.3 | 0.2 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 |
| Class D | 0.0 | 0.1 | 0.2 | 0.0 | 0.3 | 0.5 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.1 | 0.0 | 0.3 | 0.3 |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | -0.2 | 0.1 | 0.0 | 0.1 | 0.5 |
| Class C | -0.1 | -1.0 | -1.1 | 0.0 | -0.5 | -0.6 |
| Class D | -0.1 | -2.6 | -2.6 | 0.0 | 0.2 | -0.5 |
| Class E | 0.0 | -0.6 | -1.6 | -0.1 | 0.5 | -1.3 |
| All | -0.1 | -1.1 | -1.2 | 0.0 | 0.1 | -0.3 |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay (P) |  |  | Low delay LC (P) |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | -0.5 | -0.3 | 0.0 | 0.1 | 0.3 |
| Class C | 0.0 | -1.1 | -1.3 | -0.1 | -0.7 | -0.6 |
| Class D | -0.1 | -2.4 | -3.1 | 0.0 | -0.2 | -0.5 |
| Class E | 0.0 | 0.2 | -0.6 | 0.0 | 1.5 | 0.2 |
| All | 0.0 | -1.0 | -1.3 | 0.0 | 0.1 | -0.1 |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

Table 4 Rate-distortion and speed performance for Set 2 Chroma and case 2.3 of JCTVC-F574 interpolation filters comparing with F599 using the same Luma interpolation filters

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.1 | 0.0 | 0.0 | -0.1 | -0.3 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | -0.2 | 0.0 | 0.0 | -0.2 | -0.4 |
| Class D | 0.0 | -0.2 | -0.1 | 0.0 | -0.1 | 0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | -0.1 | 0.0 | 0.0 | -0.1 | -0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | -0.6 | -0.3 | 0.0 | 0.1 | 0.5 |
| Class C | -0.1 | -1.2 | -1.3 | -0.1 | -0.8 | -0.7 |
| Class D | -0.1 | -3.3 | -4.0 | -0.1 | -0.6 | -0.5 |
| Class E | 0.0 | -0.2 | -1.5 | 0.2 | 0.9 | -0.7 |
| All | -0.1 | -1.4 | -1.7 | 0.0 | -0.2 | -0.3 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay (P) |  |  | Low delay LC (P) |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | -0.5 | -0.3 | 0.0 | 0.3 | 0.4 |
| Class C | -0.1 | -1.2 | -1.4 | -0.1 | -0.6 | -0.8 |
| Class D | -0.1 | -3.0 | -3.5 | -0.1 | -0.3 | -1.4 |
| Class E | -0.1 | -0.3 | -0.3 | -0.1 | 1.9 | 1.0 |
| All | -0.1 | -1.3 | -1.3 | -0.1 | 0.2 | -0.2 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# References

[1] J.Lou, K.Minoo, D.Baylon, K.Panusopone, L.Wang, A.Luthra “CE3: Fixed interpolation filter tests by Motorola Mobility,” JCT-VC document, JCTVC-F574, July 2011

[2] https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-3.0

[3] JCT-VC, “Common test conditions and software reference configurations”, JCTVC-E700, March 2011

Motorola Mobility Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).