| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-F600_r1 WG11 Number: m21034 |
| --- | --- |

| Title: | CE7: Cross check of JCTVC-F229, Performance analysis of adaptive DCT/DST selection |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Robert Cohen Anthony Vetro Huifang Sun Mitsubishi Electric Research Laboratories 201 Broadway Cambridge, MA 02139 U.S.A | Tel: Email: | +1-617-621-7503 cohen@merl.com avetro@merl.com hsun@merl.com |
| Source: | Mitsubishi Electric Research Laboratories |  |  |

_____________________________

# Abstract

This document reports the outcome of Mitsubishi Electric Research Laboratory’s (MERL) cross-verification of experiments 3.1 and 3.4 from NHK’s JCTVC-F229 contribution, “Performance analysis of adaptive DCT/DST selection.” This activity is part of Core Experiment 7 on additional transforms. A brief analysis of the software implementation is followed by a summary of experimental results. The simulation results performed for the cross-verification match those provided by NHK, and examination of the software confirms that it is implementing the proposed adaptive transform selection techniques.

# Introduction

Core Experiment 7 (CE7) [1] was created to evaluate the performance of several alternative transforms when added to HM3.0 [2], which is the software platform currently being used for evaluating the HEVC Test Model (HM). One of these proposals is NHK’s “Performance analysis of adaptive DCT/DST selection,” JCTVC-D229 [3]. The proposed methods are similar to those used in their contribution JCTVC-E107 [4] on adaptive DCT/DST selection, from the previous JCT-VC meeting. We provide an analysis of their implementation in the next section, followed by simulation results and conclusions.

# Analysis of implementation

We were provided with a modified version of the HM 3.0 source code. This software implements an adaptive DCT/DST decision for each luma transform operation. In the encoder, for both Intra and Inter slices, each TU is transformed using a 2-D DCT and a 2-D DST. The transform that yields the lowest rate-distortion cost is signaled prior to signaling the associated transform coefficients. The DST used in this implementation is the Type-II DST (DST-II), which is similar to a DCT with a sign change to alternate inputs to each butterfly operation. For the horizontal portion of the transform, butterfly operations that are common to the DCT and DST-II are shared. This combined transform thus has one set of inputs and two sets of outputs, and it is less complex than performing a horizontal DCT and DST separately.

The main difference between this software and the earlier software from JCTVC-E107 is that the new software contains a fast mode decision FAST_ATS_MODE_DECISION, based on the absolute sum of coefficients for the DCT vs. DST. The transform corresponding to the smaller sum is used as the current transform. If the fast mode is not enabled for a given transform size, then both the DCT and DST are checked in a rate-distortion optimization loop.

There are also several experiments and experimental flags associated with the software. The following flags were set, as specified by NHK:

For experiment 3.1, which applies the proposed transforms to Intra and Inter TUs from 8x8 up to 32x32, using R-D optimization, the settings are:

#define ADAPTIVE_DCT_DST                1

#define ENABLE32x32DST_INTER            1

#define ENABLE32x32DST_INTRA            1

#define ENABLE16x16DST_INTER            1

#define ENABLE16x16DST_INTRA            1

#define ENABLE8x8DST_INTER              1

#define ENABLE8x8DST_INTRA              1

#define ENABLE4x4DST_INTER              0

#define ENABLE4x4DST_INTRA              0

#define FAST_ATS_MODE_DECISION          1

#define FAST_MD_32x32INTER              0

#define FAST_MD_32x32INTRA              0

#define FAST_MD_16x16INTER              0

#define FAST_MD_16x16INTRA              0

#define FAST_MD_8x8INTER                0

#define FAST_MD_8x8INTRA                0

#define FAST_MD_4x4INTER                0

#define FAST_MD_4x4INTRA                0

For experiment 3.4, which applies the proposed transforms only to Inter TUs with size 16x16 or 32x32, using R-D optimization, the settings are:

#define ADAPTIVE_DCT_DST                1

#define INTRA_DST_OFF                   1

#define NO_TTF                          0

#define ENABLE32x32DST_INTER            1

#define ENABLE32x32DST_INTRA            0

#define ENABLE16x16DST_INTER            1

#define ENABLE16x16DST_INTRA            0

#define ENABLE8x8DST_INTER              0

#define ENABLE8x8DST_INTRA              0

#define ENABLE4x4DST_INTER              0

#define ENABLE4x4DST_INTRA              0

#define FAST_ATS_MODE_DECISION          1

#define FAST_MD_32x32INTER              0

#define FAST_MD_32x32INTRA              0

#define FAST_MD_16x16INTER              0

#define FAST_MD_16x16INTRA              0

#define FAST_MD_8x8INTER                0

#define FAST_MD_8x8INTRA                0

#define FAST_MD_4x4INTER                0

#define FAST_MD_4x4INTRA                0

The software for experiment 3.4 was slightly different from that used for experiment 3.1, in that the software for 3.4 included the flag “INTRA_DST_OFF”, which prevents the proposed method from operating on any Intra-coded TU. In both experiments, the fast decision method is not enabled.

As in their earlier contribution, the DCT and DST-II are tested for luma components of both Inter and Intra CUs. Chroma blocks always use the DCT. For the horizontal part of the transform, the single combined DCT/DST function generates two sets of horizontal transform coefficients. For the vertical part of the transform, in a rate-distortion optimization loop, the DCT from HM is applied vertically to generate 2-D DCT transform coefficients. When the DST is tested in the R-D loop, a subset of the simultaneously-computed horizontal DST coefficients, as pictured in JCTVC-F229, are changed in sign. The resulting coefficients are then passed to a vertical transform using the HM DCT. Transform sizes of 4x4, 8x8, 16x16, and 32x32 are implemented in this way. The flags are entropy-coded prior to coding the TU coefficients.

In the decoder, the transform type flags are parsed, and then the corresponding inverse 2-D DCT from HM, or the inverse 2-D DST-II, is performed.

# Cross-verification of experimental results

We compiled and ran the HM 3.0 source code and the JCTVC-F229 modified source code on a 64-bit Linux cluster that uses Intel Xeon 2.5GHz processors. Detailed results from the simulations are included in the accompanying spreadsheet. The BD-rate (and PSNR) results obtained from the cross-verification match those provided to us by NHK. Because the proposed tool also operates on inter-predicted slices, low-delay configurations were used as well. The few minor differences in the tables are because one spreadsheet has data entered with three digits after the decimal point, and the other spreadsheet uses four. The encode/decode time differences are within acceptable margins.

## CE7 Tool 5, Experiment 3.1

The proposed transforms are applied to Intra and Inter TUs from 8x8 up to 32x32, using R-D optimization. Cross-check results are summarized inTable 1.

Table 1. Summary of MERL’s cross-verification results for CE7 Tool 5, Experiment 3.1

|  | All intra HE |  |  | All intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.4 | 0.0 | -0.1 | 0.2 | -0.1 | -0.1 |
| Class B | -0.4 | -0.1 | 0.0 | 0.4 | 0.2 | 0.2 |
| Class C | -0.3 | 0.0 | 0.0 | 0.2 | 0.2 | 0.1 |
| Class D | -0.2 | 0.0 | 0.1 | 0.1 | 0.1 | 0.1 |
| Class E | -0.1 | -0.2 | -0.1 | 1.3 | 0.1 | 0.3 |
| Overall | -0.3 | -0.1 | -0.1 | 0.4 | 0.1 | 0.1 |
| Enc Time[%] | 119% |  |  | 135% |  |  |
| Dec Time[%] | 100% |  |  | 103% |  |  |
|  | Random access HE |  |  | Random access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.6 | 0.5 | -0.1 | 0.3 | 0.4 | 0.4 |
| Class B | -0.6 | 0.4 | 0.5 | 0.0 | 0.9 | 0.9 |
| Class C | -0.3 | 0.3 | 0.2 | 0.1 | 0.7 | 0.8 |
| Class D | -0.4 | 0.2 | 0.3 | -0.1 | 0.5 | 0.7 |
| Class E |  |  |  |  |  |  |
| Overall | -0.5 | 0.4 | 0.3 | 0.1 | 0.6 | 0.7 |
| Enc Time[%] | 113% |  |  | 111% |  |  |
| Dec Time[%] | 100% |  |  | 97% |  |  |
|  | Low delay (B) HE |  |  | Low delay (B) LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.0 | 0.8 | 0.5 | -0.5 | 0.7 | 0.7 |
| Class C | -0.6 | 0.4 | 0.4 | -0.2 | 0.9 | 1.0 |
| Class D | -0.7 | 0.7 | 1.2 | -0.4 | 0.9 | 1.2 |
| Class E | -0.2 | 1.7 | 2.4 | 0.6 | 2.5 | 1.6 |
| Overall | -0.7 | 0.8 | 1.0 | -0.2 | 1.1 | 1.1 |
| Enc Time[%] | 109% |  |  | 109% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Low delay (P) HE |  |  | Low delay (P) LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.9 | 0.8 | 0.6 | -0.4 | 1.0 | 1.4 |
| Class C | -0.6 | 0.5 | 0.4 | -0.2 | 0.9 | 1.0 |
| Class D | -0.7 | 1.0 | 1.0 | -0.4 | 0.8 | 0.4 |
| Class E | -0.2 | 2.1 | 1.9 | 0.5 | 1.9 | 1.7 |
| Overall | -0.6 | 1.0 | 0.9 | -0.2 | 1.1 | 1.1 |
| Enc Time[%] | 117% |  |  | 119% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |

  - CE7 Tool 5, Experiment 3.4

The proposed transforms are applied only to Inter-predicted TUs of size 16x16 or 32x32, using R-D optimization. Cross-check results are summarized in Table 2.

Table 2. Summary of MERL’s cross-verification results for CE7 Tool 5, Experiment 3.4

|  | Random access HE |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.4 | -0.2 | 0.0 | -0.2 | 0.2 | 0.4 |
| Class B | -0.3 | 0.1 | 0.1 | -0.2 | 0.4 | 0.4 |
| Class C | -0.1 | 0.1 | 0.0 | 0.0 | 0.3 | 0.3 |
| Class D | -0.1 | 0.0 | 0.0 | 0.0 | 0.1 | 0.2 |
| Class E |  |  |  |  |  |  |
| Overall | -0.2 | 0.0 | 0.0 | -0.1 | 0.2 | 0.3 |
| Enc Time[%] | 105% |  |  | 104% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay (B) HE |  |  | Low delay (B) LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.7 | 0.5 | 0.5 | -0.5 | 0.5 | 0.4 |
| Class C | -0.3 | -0.1 | 0.2 | -0.2 | 0.7 | 0.4 |
| Class D | -0.3 | 0.2 | 0.6 | -0.2 | 0.5 | 0.5 |
| Class E | -0.2 | 0.8 | 0.4 | 0.0 | 1.1 | 0.7 |
| Overall | -0.4 | 0.4 | 0.4 | -0.2 | 0.6 | 0.5 |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay (P) HE |  |  | Low delay (P) LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.6 | 0.3 | 0.0 | -0.4 | 0.8 | 0.7 |
| Class C | -0.3 | 0.2 | 0.2 | -0.2 | 0.4 | 0.5 |
| Class D | -0.2 | 0.1 | 0.0 | -0.1 | 0.5 | 0.6 |
| Class E | -0.3 | 0.7 | 0.2 | 0.1 | 1.3 | 1.0 |
| Overall | -0.3 | 0.3 | 0.1 | -0.2 | 0.7 | 0.7 |
| Enc Time[%] | 107% |  |  | 109% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

# Summary and Conclusions

The cross-verifications performed for this contribution match and confirm the information reported by NHK. The bit-rate and PSNR values were identical except for minor rounding differences, and runtimes were close enough to be considered a match.

## Experiment 3.1

The compression efficiency improvements for Luma were about 0.1% to 1.0%. In a few cases there were losses of 0.1% to 1.3%. Generally, the high efficiency conditions yielded more BD-Rate improvement than the low complexity configurations. For the Intra low complexity experiment, there was an average loss of 0.4%. Chroma BD-Rate metrics can be ignored since the proposed methods make no changes to chroma components.

For encoding times, the proposed tool had encode-time ratios of approximately 119-135% for the Intra configurations. The random access and low delay configurations had encode time ratios of about 109-113%. The main reason for the complexity increase was the rate-distortion decision process being performed for the two transforms. When compared to encode time ratios for the earlier JCTVC-E107 proposal (which were compared to HM 2.0), these new ratios are about 20% better than before for all-Intra configurations, and a 2% to 15% percent better for the other configurations.

The decoding times for the proposed tool were within a few percent of those from HM 3.0. Because the inverse DST-II can be implemented using alternate sign changes followed by the HM inverse DCT, the small complexity-related difference between the proposed decoder and HM 3.0 are these extra sign flips.

## Experiment 3.2

Average improvements in compression efficiency ranged from 0.1% to 0.3%. The proposed tool had encoding time ratios ranging from 103% to 109%. Decoding times were within a few percent of those from HM 3.0. Since this experiment used a subset of the configuration from Experiment 3.1, please see above for additional details.

# References

- R. Cohen, C. Yeo, R. Joshi, F. Fernandes, “Description of Core Experiment 7: Additional Transforms”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E707, Geneva, CH, March 2011.

- HM 3.0 software, available at https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware

- A. Ichigaya, Y. Sugito, and S. Sakaida, “CE7.5: Performance analysis of adaptive DCT/DST selection”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F229, Geneva, CH, March 2011.

- A. Ichigaya, Y. Sugito, and S. Sakaida, “CE7.6: Simplified adaptive transform selection”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E107, Geneva, CH, March 2011.

- F. Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E700, Geneva, CH, March 2011.