| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F601 |
| --- | --- |

| Title: | Joint Sub-pixel Interpolation for bi-predicted Motion Compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Koohyar Minoo Jian Lou 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-2521 kminoo@motorola.com +1-858-404-3427 lou@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

This document proposes an interpolation scheme for temporal prediction of PUs(prediction units), when bi-prediction is used. In such cases the optimal filter per reference list would depend on the joint sub-pixel offsets. For example if four sub-pixel offsets per reference block/list can be indicated for prediction of each PU, then a set of up to 16 pairs of filters can be designed to optimally conduct a bi-prediction motion compensation for that PU. This scheme in intended to be used to increase the coding efficiency of video compression schemes, or alternatively to reduce the complexity of motion-compensation, in terms of memory access bandwidth and/or number of operations performed for sub-pixel interpolation filtering.

# Introduction

The current HEVC test model like many of its predecessors such as AVC utilizes a specific interpolation filter for motion compensation for each reference block solely based on the coded information for the choice of sub-pixel offset for that reference block. For example if the choice of sub-pixel motion compensation indicates a half-pixel shift for List 0, then the same half-pixel interpolation filter would be applied to the reference block in List 0 regardless of the sub-pixel position choice for List 1.

It seems beneficial to use the joint sub-pixel positions from each reference block, to design and perform phase-delay cancellation in two reference blocks and achieve better prediction for the current PU.

.

Fig. 1 illustrates the positions of the resulting half-pel and quarter-pel pixels between full-pel pixels along a pixel line within an image, where L3, L2, L1, L0, R0, R1, R2, and R3 are full-pel pixels, H is a half-pel pixel between L0 and R0, FL is a sub-pel pixel between L0 and H, and FR is a sub-pel pixel between H and R0.

Fig. 1 Sub-Pel Positions

# Proposed sub-pel interpolation filters

## To achieve phase cancellation

For bi-prediction of a PU, let’s assume two reference blocks from List 0 and List 1 are available. Also assume each reference block indicates a sub-pixel phase shift of α1 and α2 for List 0 and List 1 prediction, respectively.  Furthermore a simple average would be conducted to generate the reference signal for prediction of current PU. The choice of filters for preprocessing of reference block before averaging takes place are noted by H1(ω) and H2(ω).

If we represent the signal in the current PU with S(ω), and the two signals in the two reference blocks by P1(ω) and P2(ω), we can make the following assumption:

P1(ω)=S(ω)⋅eia1ω

P2(ω)=S(ω)⋅eia2ω

The predicted signal would be

P(ω)=12S(ω)⋅eia1ω⋅H1(ω)+S(ω)⋅eia2ωH2(ω)

For ideal prediction of S(ω), one needs to design H1(ω) and H2(ω) such that:

H1(ω)⋅eia1ω+H2(ω)⋅eia2ω=2

The above formula shows that optimal H1(ω) or H2(ω), in general, depend on both α1 and α2. So the optimal interpolation filters for reference “i” in the general case that “n” references are available would have the form of Hi(a1,⋅⋅,an,ω)   i∈1,⋅⋅,n

Note that the same principle applies if any number of prediction blocks with variety of super-imposition (such as weighted averaging, etc.) would be used for derivation of final predicted signal.

## To reduce memory access and number of operations

In bi-prediction of a PU, as the two reference blocks should be, relatively, follow the corresponding pixels in that PU (perhaps with the exception of a sub-pixel shift) a complexity aware implementation of sub-pixel motion compensation filters can be designed by utilizing asymmetric filters for each predicted block, where right/top pixels are considered by filter coefficients from the filter applied to one of the reference blocks (e.g. from List 0) and the opposite (left/down) pixels are considered by the second filter applied to the other reference blocks (e.g. from List 1).

Example below shows an instance of jointly designed filter sets with complexity of 6 taps per prediction. Note that sample delays are indicated in front of each filter.

For List 0

h1(0, 0)  = {0,       0,      64,       0,       0,       0}  : delay 3

h1(0, 1)  = {-1,       4,       0,      62,       0,       1}  : delay 4

h1(0, 2)  = {4,      -2,      64,      -1,       2,      -4} : delay 3

h1(0, 3)  = {0,      62,       0,       4,      -1,       0} : delay 2

h1(1, 0)  = {-9,      61,      15,      -7,       3,      -1} : delay 2

h1(1, 1)  = {-7,      59,      15,      -6,       6,      -2} : delay 2

h1(1, 2)  = {-2,       9,      -8,      59,      14,      -5}  : delay 4

h1(1, 3)  = {-1,       6,     -11,      60,      14,      -5} : delay 4

h1(2, 0)  = {1,     -11,      40,      41,     -13,       7} : delay 3

h1(2, 1)  = {-12,      39,      41,     -13,       8,      -2}  : delay 2

h1(2, 2)  = {-4,      11,     -12,      40,      39,     -11} : delay 4

h1(2, 3)  = {-2,       8,     -13,      41,      39,     -12}  : delay 4

h1(3, 0)  = {-1,       3,      -6,      15,      61,      -9}  : delay 4

h1(3, 1)  = {-5,      15,      60,     -10,       6,      -1}  : delay 2

h1(3, 2)  = {-5,      14,      59,      -8,       9,      -2} : delay 2

h1(3, 3)  =   {-6,      14,      59,      -8,       6,      -2} : delay 2

For List 1

h2(0, 0)  = {0,       0,      64,       0,       0,       0}  : delay 3

h2(0, 1)  = {-9,      61,      15,      -7,       3,      -1} : delay 2

h2(0, 2)  = {1,     -11,      40,      41,     -13,       7} : delay 3

h2(0, 3)  = {-1,       3,      -6,      15,      61,      -9} : delay 4

h2(1, 0)  = {-1,       4,       0,      62,       0,       1} : delay 4

h2(1, 1)  = {-2,       6,      -8,      59,      14,      -6} : delay 4

h2(1, 2)  = {-12,      39,      41,     -13,       8,      -2} : delay 2

h2(1, 3)  = {-5,      15,      60,     -10,       6,      -1} : delay 2

h2(2, 0)  = {4,      -2,      64,      -1,       2,      -4} : delay 3

h2(2, 1)  = {-2,       9,      -8,      59,      14,      -5} : delay 4

h2(2, 2)  = {-11,      40,      40,     -11,      10,      -3} : delay 2

h2(2, 3)  = {-5,      14,      59,      -8,       9,      -2} : delay 2

h2(3, 0)  = {0,      62,       0,       4,      -1,       0}  : delay 2

h2(3, 1)  = {-1,       6,     -11,      60,      14,      -5} : delay 4

h2(3, 2)  = {-2,       8,     -13,      41,      39,     -12} : delay 4

h2(3, 3)  = {-2,       6,      -6,      15,      59,      -7}  : delay 4

Example below shows an instance of jointly designed filter sets with complexity of 4 taps per prediction. Note that sample delays are indicated in front of each filter.

For List 0

h1(0, 0)  = {  0,      64,       0,       0}  : delay 2

h1(0, 1)  = { -1,       3,      -9,      65}  : delay 4

h1(0, 2)  = { 6,      -5,      68,      -8} : delay 3

h1(0, 3)  = { 65,      -9,       3,      -1} : delay 1

h1(1, 0)  = { 60,      13,      -5,       2} : delay 1

h1(1, 1)  = { -3,       7,     -17,      61} : delay 4

h1(1, 2)  = { 4,     -19,      63,       9}  : delay 3

h1(1, 3)  = { 1,     -10,      61,      14} : delay 3

h1(2, 0)  = { -11,      41,      39,      -2} : delay 2

h1(2, 1)  = { 38,      43,     -14,       4}  : delay 1

h1(2, 2)  = { 7,     -22,      40,      39} : delay 3

h1(2, 3)  = { 4,     -14,      43,      35}  : delay 3

h1(3, 0)  = { 2,      -5,      13,      60}  : delay 3

h1(3, 1)  = { 12,      60,     -12,       2}  : delay 1

h1(3, 2)  = { 9,      65,     -17,       3} : delay 1

h1(3, 3)  =   { 61,     -17,       7,      -3} : delay 0

For List 1

h2(0, 0)  = { 0,      64,       0,       0}  : delay 2

h2(0, 1)  = {60,      13,      -5,       2 } : delay 1

h2(0, 2)  = {-11,      41,      39,      -2 } : delay 2

h2(0, 3)  = {2,      -5,      13,      60 } : delay 3

h2(1, 0)  = {-1,       3,      -9,      65 } : delay 4

h2(1, 1)  = {59,      27,     -10,       4 } : delay 1

h2(1, 2)  = {38,      43,     -14,       4 } : delay 1

h2(1, 3)  = {12,      60,     -12,       2 } : delay 1

h2(2, 0)  = {6,      -5,      68,      -8 } : delay 3

h2(2, 1)  = {4,     -19,      63,       9 } : delay 3

h2(2, 2)  = {39,      40,     -22,       7 } : delay 1

h2(2, 3)  = {9,      65,     -17,       3 } : delay 1

h2(3, 0)  = {65,      -9,       3,      -1 }  : delay 1

h2(3, 1)  = {1,     -10,      61,      14 } : delay 3

h2(3, 2)  = {4,     -14,      43,      35 } : delay 3

h2(3, 3)  = {4,     -10,      27,      59 }  : delay 3

Example below shows an instance of jointly designed filter sets with complexity of 5 taps per prediction. Note that sample delays are indicated in front of each filter.

For List 0

h1(0, 0)  = {0,       0,      64,       0,       0}  : delay 3

h1(0, 1)  = { 58,      -1,      -5,       3,      -1}  : delay 1

h1(0, 2)  = { 1,      -2,      -8,      65,       0} : delay 4

h1(0, 3)  = { 1,      -3,       0,      67,      -1} : delay 4

h1(1, 0)  = { -1,       3,      -8,      65,      15} : delay 4

h1(1, 1)  = { -1,       5,     -15,      59,      14} : delay 4

h1(1, 2)  = { 0,       2,     -15,      61,      15}  : delay 4

h1(1, 3)  = { 0,       1,     -10,      60,      11} : delay 4

h1(2, 0)  = { -1,       4,     -11,      40,      38} : delay 4

h1(2, 1)  = { 37,      41,     -17,       5,      -1}  : delay 1

h1(2, 2)  = { -3,      10,     -24,      42,      38} : delay 4

h1(2, 3)  = { -1,       5,     -17,      40,      40}  : delay 4

h1(3, 0)  = { 11,      58,      -7,       3,      -1}  : delay 1

h1(3, 1)  = { 0,      12,      63,     -10,       1}  : delay 2

h1(3, 2)  = { 0,      16,      59,     -16,       2} : delay 2

h1(3, 3)  =   { -2,       6,     -12,      15,      59} : delay 4

For List 1

h2(0, 0)  = {0,       0,      64,       0,       0}  : delay 3

h2(0, 1)  = { -1,       3,      -8,      65,      15} : delay 4

h2(0, 2)  = { 40,      40,     -11,       4,      -1} : delay 1

h2(0, 3)  = { 11,      58,      -7,       3,      -1} : delay 1

h2(1, 0)  = { 58,      -1,      -5,       3,      -1} : delay 1

h2(1, 1)  = { 59,      15,     -12,       6,      -2} : delay 1

h2(1, 2)  = { 37,      41,     -17,       5,      -1} : delay 1

h2(1, 3)  = { 12,      63,     -10,       1,       0} : delay 1

h2(2, 0)  = { 66,      -6,      -4,       3,      -1} : delay 1

h2(2, 1)  = { 2,     -15,      61,      15,       0} : delay 3

h2(2, 2)  = { 40,      40,     -21,       8,      -2} : delay 1

h2(2, 3)  = { 16,      59,     -16,       2,       0} : delay 1

h2(3, 0)  = { 1,      -3,       0,      67,      -1}  : delay 4

h2(3, 1)  = { 0,       1,     -10,      60,      11} : delay 4

h2(3, 2)  = { -1,       5,     -17,      40,      40} : delay 4

h2(3, 3)  = { 14,      59,     -15,       6,      -2}  : delay 1

# Simulation environment

## Software

The software used to run the simulations is HM3.0 [1]. The binaries were compiled using Microsoft Visual Studio 2008 64bit. The coding conditions used for the simulations are specified in [2] with the exception of the number of frames. YUV file output during the decoding was disabled when collecting the decoding time statistics.

## Hardware

Two sets of PCs with identical settings were used for simulations, one set with Microsoft Windows 7 64bits, Intel Xeon 3.0GHz with 8G memory and one set with Microsoft Windows 7 64bits, Intel Xeon 3.2GHz with 12G memory. The anchor results from the PCs with the same settings are used for comparison. Therefore, both the encoding time and decoding time comparisons are reliable.

# Simulation Results

The encoder reconstructed YUV files match the decoder reconstructed YUV files. The experimental results of the rate-distortion and speed comparison are summarized in Table 1 to Table 3 for the 6taps, 4taps and 5taps joint sub-pixel bi-prediction, only applied to Luma.

Results in table 1 and table 2 are obtained by 6taps and 4taps joint bi-prediction sub-pixel filters and the interpolation filters for ME and Uni-prediction are the ones described in JCTVC-F599 for 8H+8Q.

Results in table 3 are obtained by 5taps joint bi-prediction sub-pixel filters and the interpolation filters for ME and Uni-prediction are the ones described in JCTVC-F574 for 8H+8Q.

While memory access bandwidth and operation savings have been achieved for 6tap bi-prediction filters, there is no coding efficiency loss observed compared to HM3.0

Also for 4tap joint bi-prediction sub-pixel filters with much significant savings in memory access and operation complexity, one would only observe 1.2% drop in coding efficiency.

For comparison of an intermediate complexity the results for 5tap joint bi-prediction sub-pixel filters are give in Table 3. It should be noted that without joint sub-pixel phase delay cancellation designing filters with odd number of taps is not recommended for half pixel position in any of the two lists.

The total drop in coding efficiency in Table 3 is about, 0.5%.

Note that in our design the 5tap and 6tap filters could benefit from a more relaxed condition of the coefficient offset, but due to existing implementation in HM3.0 they stayed within the support area of a symmetric 8tap filter.

Table 1 Rate-distortion and speed performance comparison of 6tap Bi 8tap Uni interpolation for Luma

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | -0.2 | 0.2 | 0.3 | 0.3 | 0.4 |
| Class B | 0.2 | 0.1 | 0.3 | 0.7 | 0.4 | 0.3 |
| Class C | -0.3 | 0.1 | 0.0 | -0.2 | 0.1 | 0.0 |
| Class D | -0.8 | -0.3 | -0.3 | -0.7 | -0.2 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | -0.1 | 0.0 | 0.0 | 0.1 | 0.1 | 0.2 |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 115% |  |  | 120% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.1 | 0.4 | 0.6 | 0.6 | 0.3 | 0.3 |
| Class C | -0.3 | -0.1 | 0.0 | -0.3 | 0.6 | 0.4 |
| Class D | -0.7 | -0.5 | -0.6 | -0.4 | 0.6 | 0.6 |
| Class E | 0.6 | 0.4 | -0.5 | 0.7 | -0.1 | -0.7 |
| All | -0.1 | 0.0 | -0.1 | 0.1 | 0.4 | 0.2 |
| Enc Time[%] | 99% |  |  | 101% |  |  |
| Dec Time[%] | 119% |  |  | 126% |  |  |

Table 2 Rate-distortion and speed performance comparison of 4tap Bi 8tap Uni interpolation for Luma

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 1.1 | 0.8 | 1.2 | 1.7 | 1.1 | 1.4 |
| Class B | 1.6 | 1.1 | 1.2 | 2.3 | 1.4 | 1.4 |
| Class C | 1.1 | 1.2 | 1.2 | 1.5 | 1.1 | 1.1 |
| Class D | 1.5 | 1.4 | 1.3 | 2.4 | 1.5 | 1.6 |
| Class E |  |  |  |  |  |  |
| All | 1.4 | 1.2 | 1.2 | 2.0 | 1.3 | 1.4 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 118% |  |  | 118% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.8 | 1.2 | 1.1 | 0.9 | 1.2 | 1.0 |
| Class C | 0.6 | 1.0 | 0.9 | 0.4 | 1.3 | 1.2 |
| Class D | 0.7 | 0.8 | 1.5 | 1.0 | 1.9 | 2.3 |
| Class E | 1.3 | 0.8 | 0.2 | 0.9 | 1.0 | 0.7 |
| All | 0.8 | 1.0 | 1.0 | 0.8 | 1.4 | 1.3 |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 114% |  |  | 123% |  |  |

Table 3 Rate-distortion and speed performance comparison of 5tap Bi and 8tap Uni prediction for Luma interpolation (none-matching Chroma)

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.6 | 0.6 | 0.7 | 0.4 | 0.9 | 1.0 |
| Class B | 0.8 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 |
| Class C | 0.7 | 0.9 | 0.9 | 0.7 | 0.9 | 0.9 |
| Class D | 0.8 | 1.1 | 1.1 | 1.1 | 1.2 | 1.1 |
| Class E |  |  |  |  |  |  |
| All | 0.7 | 0.8 | 0.9 | 0.7 | 0.9 | 0.9 |
| Enc Time[%] | 111% |  |  | 114% |  |  |
| Dec Time[%] | 120% |  |  | 124% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.2 | 1.2 | 1.1 | -0.2 | 0.8 | 0.4 |
| Class C | 0.3 | 1.8 | 2.1 | 0.0 | 1.9 | 1.6 |
| Class D | 0.3 | 4.0 | 4.7 | 0.5 | 2.1 | 2.9 |
| Class E | 0.6 | 0.6 | 0.9 | -0.2 | -0.6 | -0.4 |
| All | 0.3 | 1.9 | 2.2 | 0.0 | 1.1 | 1.2 |
| Enc Time[%] | 114% |  |  | 118% |  |  |
| Dec Time[%] | 122% |  |  | 132% |  |  |

# References

[1] https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-3.0

[2] JCT-VC, “Common test conditions and software reference configurations”, JCTVC-E700, March 2011

Motorola Mobility Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).