| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F603 |
| --- | --- |

| Title: | Cross-check of Sony’s proposed JCTVC-F091 on Unifying binarizations of Intra modes in HE and LC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | +1-858-845-17952000 wchien@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution provides a summary of the cross-checks performed by Qualcomm on Sony’s proposal JCTVC-F091on Unifying binarizations of Intra modes in HE and LC. The cross-check rate-distortion performance results match those provided by Sony.

# Introduction

In JCTVC-F091[1], a unifiying binarizations are proposed for CABAC and VLC. Golomb codes were proposed to serve this purpose. 	Three modifications are proposed in [1].

- The Golomb codes with 15, 16, 32, and 33 codewords are proposed to code the syntax element rem_intra_pred_mode.

- The codeword adaptation for the syntax element rem_intra_pred_mode are introduced in CABAC. The adaptation is as the same as it in VLC.

- The derivation of the IntraPredMode is modified to the following procedure

- for (cIdx = 0; cIdx < NumMPMCand; cIdx++)

if (IntraPredModeTableE[rem_intra_pred_mode]>=IntraPredModeTableE[candModeList[cIdx]])

rem_intra_pred_mode ++

- IntraPredMode[ xB ][ yB ] = IntraPredModeTableD[rem_intra_pred_mode]

where IntraPredModeTableE and IntraPredModeTableD are the codeword adaptation mapping tables for encoding and decoding, respectively.

# Simulation Results

The test conditions [2] are followed. The implementation is on HM 3.0. Tests were conducted on Intra configurations for both high efficiency and low-complexity settings. Table 1 summarizes the BD-Rate [3] results.

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.36 | -0.04 | 0.03 | -0.12 | -0.07 | -0.04 |
| Class B | -0.34 | -0.06 | -0.01 | -0.13 | -0.10 | -0.08 |
| Class C | -0.28 | -0.07 | -0.06 | -0.16 | -0.09 | -0.12 |
| Class D | -0.16 | -0.11 | -0.11 | -0.06 | -0.01 | -0.02 |
| Class E | -0.22 | 0.10 | 0.04 | -0.03 | 0.04 | 0.05 |
| All | -0.28 | -0.05 | -0.02 | -0.10 | -0.05 | -0.05 |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

The results confirm what was reported in JCTVC-F091.

# References

[1] Ehsan Maani, Ali Tabatabai, “Unifying binarizations of Intra modes in HE and LC”, JCTVC-F091, Torino, Italy, July 2011.

[2] Frank Bossen, “Common test conditions and software reference configurations,” JCTVC-E700, Geneva, Switzerland, Jan 2011.

[3] Gisle Bjontegaard, “Calculation of Average PSNR Differences between RD curves,” ITU-T SC16/Q6, VCEG-M33, Austin, USA, Apr 2001.