| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F604 |
| --- | --- |

| Title: | Detection of a CDR for random access |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Youngo Park Kwang Pyo Choi Chanyul Kim | Email: youngo.park@samsung.com kp5.choi@samsung.com dionism@samsung.com |  |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

- Abstract

The clean decoding refresh (CDR) pictures as a new type of random access points were introduced to HEVC and the discussion related to CDR was proposed in JCTVC-E400. In this proposal, the specific way to find a brother picture mentioned in JCTVC-E400 is proposed.

Introduction

As discussed in JCTVC-E400, to make the decoder more effective, it is required that the detection of a CDR is used for random access or normal play. If decoder can detect the status of decoding, i.e, random access or normal play, the potentially unnecessary pictures could be skipped in decoding process and the DPB could be refreshed like as IDR picture. So in the previous proposal, they proposed the way to detect a brother picture where the frame_num or POC of a brother picture is signaled in CDR. As the frame_num is wrap arounded and POC also could be refreshed when it meets IDR picture, the way to use frame_num or POC for detection of a brother picture is not sufficient. In this proposal, another way to detect the CDR placed in the middle of random access or normal play is proposed.

Algorithm Details

As the POC could be refreshed when it faces the IDR picture in the bitstream and the frame_num could be wrap arounded, the unique way to know the current position is required to detect brother picture.

The new syntax, namely pic_discontinuity_cnt is introduced in slice header and the value increases as the following cases:

- POC is reset

- POC LSB is wrap arounded

- the current picture is a CDR

The example is shown in figure 1.

Figure 1. The example of random access detection

The POC information of brother pictures is also included in CDR slice header.

Syntax and Semantics

| slice_header( ) { | Descriptor |
| --- | --- |
| <snip> |  |
| if(nal_unit_type == 4) { |  |
| pic_discontinuity_cnt | u(v) |
| num_brother_frames | ue(v) |
| for(i=0; i<num_brother_frames; i++) |  |
| brother_frame_poc[i] | ue(v) |
| } |  |
| } |  |

Conclusion

In this proposal, another way to detect brother pictures is proposed in order to know the decoding process is in random access or normal play status regardless of POC refreshing or frame_num wrap around.

Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).