| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F605 WG11 Number: m21039 |
| --- | --- |

| Title: | CE6.d: Parallel Intra Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Zhao and Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: Email: | Tel: +1-360-817-8487 asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

This contribution reports results for the parallel intra prediction concept that was proposed in JCTVC-E315. The parallel prediction unit (PPU) defines a group of pixels that are intra-coded in a parallel fashion. Parallelization is achieved by partitioning the intra-coded blocks into two sets.  Blocks in the first set are predicted in parallel using available pixels outside the PPU; blocks in the second set are also predicted in parallel using available pixels outside the PPU as well as pixels from the first set of blocks.

Here, the performance of the parallel intra prediction is reported in the context of HM3.0.  Three configurations are considered.  First, a “checker board” partitioning is considered.  Second, a “stripe” configuration is reported.  Finally, for comparison, the impact of disabling 4x4 prediction completely (but allowing 4x4 transforms) is reported.  For the case of parallel intra prediction with stripe partition, an average coding efficiency impact of 0.2%, 0.5% and 1.3% for, respectively, low delay, random access, and all intra common test conditions is reported. For the case of checker board partition, there is a coding efficiency impact of 1.5% and 1.8% for all intra and intra loco coding. In comparison, disabling 4x4 prediction (8x8 prediction with 4x4 residuals) results in 3.9% and 5.3% rate increase for intra and intra loco configurations.

# Introduction

Intra prediction is currently achieved by partitioning a largest coding unit (LCU) into one or more blocks through a recursive splitting process. Each block is predicted spatially and subsequently refined, and the prediction is performed sequentially using neighboring reconstructed blocks. The prediction process requires that the causal neighbors of the current block must be completely reconstructed before processing the current block.  This results in a set of serial dependencies, and these dependencies impact the complexity for both the encoder and decoder processes.

# Parallel Intra Prediction

## Parallel Prediction Unit

In our previous contribution (JCTVC- E315), we introduced the concept of parallel intra-prediction unit (PPU).  The PPU defines the size of a block that can be coded using parallelization.  The motivation for the PPU is straightforward – the serial bottleneck in intra-prediction is not determined by simply the number of blocks within an LCU, but also by the size of the blocks that are being intra-coded.  For blocks larger than a defined PPU size, normal sequential prediction is performed. For blocks that fall within a PPU, if they are further partitioned, parallel intra prediction may be employed. Here, we define PPU size to be 8. For an 8x8 block that are partitioned to four 4x4 blocks either by PU split or TU split, parallel intra prediction is employed.

## Parallel Intra Prediction

Parallel intra prediction consists of three steps.

- Partitioning the PPU into two sets

- Predicting the 1st set blocks (shaded blocks) in parallel using the PPU block neighbors

- Predicting the 2nd set blocks (white blocks) in parallel using PPU block neighbors and also the neighbors from the 1st set blocks.

We tested the proposed parallel intra prediction with three types of partitions.

(1) Checker board partition

| As shown in Fig. 1, four 4x4 blocks are grouped into two sets using checker board pattern. Block 1 and 2 are the 1st set and are predicted in parallel using 8x8 PPU neighbors. Then block 0 and 3 are predicted using its 4x4 block neighbors. With this partition, it can achieve 2X Parallelism. In prior meetings, we proposed to use bi-prediction from bottom and right neighbors to improve the coding efficiency of the 2nd set blocks. Due to complexity concern, we removed the bi-directional prediction in this contribution. | Fig. 1 - Parallel Prediction with checker board partition for PPU of 8x8 |
| --- | --- |

(2) Stripe partition

| In the configuration, an 8x8 PPU is partitioned into two 8x4 stripes. Firstly, block 0 and 1 are predicted in parallel using 8x8 PPU neighbors. Then block 2 and 3 are predicted in parallel using their 8x4 block neighbors including those reconstructed pixels from block 0 and 1. With this partition, it can achieve 2X parallelism. | Fig. 2 - Parallel Prediction with stripe partition for PPU of 8x8 |
| --- | --- |

(3) 8x8 prediction with 4x4 residuals

| In the configuration, all 4 blocks are treated as first set blocks. They are restricted to have the same prediction mode, and are simply predicted using their 8x8 block neighbors. But each 4x4 block can have its own residual and transform. In this way, 4x parallelism can be achieved. However, as will be seen in the results later, disabling 4x4 prediction will result in large bit rate increase. | Fig. 3 - Parallel Prediction with stripe partition for PPU of 8x8 |
| --- | --- |

## Results

As specified in CE6, we integrated the parallel intra-prediction technology into the HM3.0 software and tested with the recommended configuration and a PPU of 8x8 on all sequences.

Table 1 below shows the summary results of parallel intra with stripe vs. HM3.0 anchor. Table 2 shows summary results of parallel intra with checker board. Table 3 shows summary results of parallel intra by 8x8 prediction with 4x4 residuals.

(1) Parallel Intra with Stripe Partition

Table 1. Summary RD results of Parallel Intra Prediction with Stripe vs. HM3.0 Anchor

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.9 | 0.2 | 0.3 | 0.9 | 0.6 | 0.7 |
| Class B | 1.0 | 0.6 | 0.5 | 1.3 | 0.9 | 0.8 |
| Class C | 1.5 | 1.1 | 1.1 | 1.8 | 1.3 | 1.1 |
| Class D | 1.8 | 1.5 | 1.4 | 2.0 | 1.5 | 1.5 |
| Class E | 1.5 | 0.6 | 0.8 | 1.6 | 0.5 | 0.7 |
| All | 1.3 | 0.8 | 0.8 | 1.5 | 1.0 | 1.0 |
| Enc Time[%] | 103% |  |  | 106% |  |  |
| Dec Time[%] | 102% |  |  | 104% |  |  |

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | -0.7 | -0.5 | 0.4 | -0.2 | 0.2 |
| Class B | 0.5 | -0.2 | 0.0 | 0.6 | 0.2 | 0.2 |
| Class C | 0.7 | 0.5 | 0.2 | 0.8 | 0.3 | 0.3 |
| Class D | 0.7 | 0.3 | 0.4 | 0.8 | 0.4 | 0.6 |
| Class E |  |  |  |  |  |  |
| All | 0.5 | 0.0 | 0.0 | 0.6 | 0.2 | 0.3 |
| Enc Time[%] | 103% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.1 | 0.3 | -0.2 | 0.2 | 0.0 | -0.3 |
| Class C | 0.2 | 0.1 | 0.3 | 0.3 | 0.4 | 0.4 |
| Class D | 0.2 | 0.4 | 0.4 | 0.2 | 0.4 | 0.2 |
| Class E | 0.2 | -0.5 | -0.8 | 0.2 | 0.8 | 0.6 |
| All | 0.2 | 0.1 | 0.0 | 0.2 | 0.3 | 0.2 |
| Enc Time[%] | 101% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |

(2) Checker board

Table 3. Summary RD results of Parallel Intra Prediction with Checker Board vs. HM3.0 Anchor

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.9 | 0.3 | 0.3 | 0.9 | 0.5 | 0.6 |
| Class B | 1.0 | 0.5 | 0.5 | 1.2 | 0.8 | 0.8 |
| Class C | 1.9 | 1.4 | 1.4 | 2.2 | 1.5 | 1.4 |
| Class D | 2.2 | 1.8 | 1.9 | 2.6 | 2.0 | 1.9 |
| Class E | 1.7 | 0.7 | 1.0 | 2.1 | 0.7 | 0.9 |
| All | 1.5 | 0.9 | 1.0 | 1.8 | 1.1 | 1.1 |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |

(3) 8x8 prediction with 4x4 residuals

Table 4. Summary RD results of Parallel Intra Prediction by 8x8 prediction with 4x4 residuals

vs. HM3.0 Anchor

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 1.8 | 0.4 | 0.5 | 2.2 | 1.2 | 1.4 |
| Class B | 2.4 | 0.9 | 0.7 | 3.4 | 1.8 | 1.6 |
| Class C | 5.5 | 2.8 | 2.8 | 7.5 | 4.2 | 3.9 |
| Class D | 6.6 | 3.7 | 3.8 | 9.0 | 4.8 | 4.8 |
| Class E | 3.7 | 1.4 | 1.7 | 4.8 | 1.5 | 2.0 |
| All | 3.9 | 1.8 | 1.9 | 5.3 | 2.7 | 2.7 |
| Enc Time[%] | 76% |  |  | 67% |  |  |
| Dec Time[%] | 98% |  |  | 93% |  |  |

From above results, it can be seen that parallel intra prediction with stripe partition gives the best performance. It has bit rate increase of 0.2%, 0.5% and 1.3% for, respectively, low delay, random access, and all intra common test conditions. In comparison, disabling intra 4x4 prediction, i.e. using 8x8 prediction with 4x4 residuals results in large bit rate increase -- an average 4% ~ 5% BD rate increase, and for class D, the rate increase for Intra LoCo is as high as 9%. Therefore to achieve parallel intra prediction, simply disabling 4x4 prediction while allowing 8x8 prediction with 4x4 residuals is not a good option. Parallel intra prediction with stripe and checker board has less impact on the coding performance, and therefore are more preferable.

# Conclusion

We propose a parallel prediction unit, or PPU, for the HEVC HM design.  The parallel prediction unit supports the parallelization of intra-coded blocks within the PPU in a two step, parallel process.  Results show that the increased parallelization comes with a little loss in coding efficiency.  Specifically, we observe an average loss of 0.2%-0.7% for inter coding scenarios.  We assert that this loss is negligible and well justified by the parallelization capability.

Given this level of performance and the significance of parallelization for future standards, we propose the JCT-VC adopt parallel intra prediction method into the HM.

# Patent rights declaration(s)

SHARP Labs of America may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- JCTVC-D074, “Parallel Prediction Unit for Parallel Intra Coding,” 4th JCT-VC Meeting, Daegu, Jan. 2011.

- JCTVC-E315, “Parallel Prediction Unit for Parallel Intra Coding,” 5th JCT-VC Meeting, Geneva, Mar. 2011.

- JCTVC-E706, “Description of Core Experiment 6: Intra Prediction Improvement”, 5th JCT-VC Meeting, Geneva, Mar. 2011.

- JCTVC-E700, “Common test conditions and software reference configurations,” 5th JCT-VC meeting, Geneva, Mar. 2011.