| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F606 |
| --- | --- |

| Title: | Memory and Parsing Friendly CABAC Context |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien Marta Karczewicz Xianglin Wang 5775 Morehouse Dr. San Diego, CA 92121, USA | Tel: Email: | 1-858-845-1795 wchien@qualcomm.com martak@qualcomm.com xianglin@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes some changes to CABAC context used in coding of various syntax elements. The changes aim at reducing the memory requirements in implementation, as well as removal of parsing related complications associated with the current context for these syntax elements. Simulation results show that the proposed context modifications do not incur coding performance loss.

# Introduction

In CABAC coding, information from neighboring blocks are often used in designing the context for certain syntax elements. The following Fig. 1 shows one example that is most often used where information from the current block’s immediate left block (A) and upper block (B) are used.  This requires the availability of related information from both the left block and the upper block.

Fig. 1 Neighboring left block and upper block

To store such neighboring block information, some additional memory is needed. As shown in Fig. 2, for coding a current LCU, blocks located in the red and green areas need to have their related information stored. The green area covers left blocks and the red area covers upper blocks. Depending on the specific syntax element, such information may be stored at different block levels. For most syntax elements, it needs to be stored at 4x4 block level.

LCU

Fig. 2 Blocks with information needed for context modeling

Memory needed for such purpose may not be significant for each syntax element. However, when there are many syntax elements with similar context models, memory usage becomes considerable. Taking a 1920x1080 video frame as an example and assuming such information from a block only takes 1-bit, corresponding memory needed would be (1920+64)/4 - 1 = 495 bits.

In addition to memory usage, in certain cases the neighboring block dependent context may also cause problem on parsing robustness. One such example is that when merge mode is used for a block with temporal motion vector selected, if there is a packet loss for a reference frame, the inter prediction direction for that merge block may not be correctly derived. As a result, context dependent on the neighboring block may also be wrong.

# Proposed Method

In this proposal, syntaxes with neighboring block dependent context or parsing include the following:

- skip flag

- split_coding_unit_flag

- cbp of luma block

- bin0 of chroma intra mode

- no_residual_data_flag

- merge_flag

- bin0 of ref_idx

- bin0 of mvd

- inter_pred_flag

- cbp of intra chroma block

Proposed changes are listed as follows.

- Reduce to just one context for no_residual_data_flag, merge_flag, bin0 of ref_idx and bin0 of mvd respectively;

- For inter_pred_flag, the context is changed to be only CU depth dependent;

- For cbp of intra chroma block, the context is changed to be the same as that currently used for inter chroma block, i.e. transform depth dependent only.

Memory requirement can be dramatically reduced with the proposed changes. The following Table 1 shows a calculation of memory usage based on a 1920x1080 video frame (assuming a SCU of 8x8). With the proposed context modifications, memory requirement is about 1/4 of that originally needed.

Table 1. Context related memory usage comparison

| Syntax | Original memory needed (bit) | Proposed (bit) |
| --- | --- | --- |
| Skip flag | (1920+64)/4 - 1 = 495 | 495 |
| split_coding_unit_flag (2-bit for CU depth) | ((1920+64)/16-1)*2 = 246 | 246 |
| cbp of luma block | (1920+64)/4 - 1 = 495 | 495 |
| bin0 of chroma intra mode | (1920+64)/4 - 1 = 495 | 495 |
| no_residual_data_flag (1-bit at 8x8 level) | (1920+64)/8 – 1 = 247 | 0 |
| merge_flag | (1920+64)/4 - 1 = 495 | 0 |
| bin0 of ref_idx (1-bit for each direction) | ((1920+64)/4–1)*2 = 990 | 0 |
| bin0 of mvd(1-bit each for x, y, and for each direction) | ((1920+64)/4 – 1)*4 = 1980 | 0 |
| inter_pred_flag | (1920+64)/4 - 1 = 495 | 0 |
| cbp of intra chroma block(1-bit each for U and V) | ((1920+64)/4–1)*2 = 990 | 0 |
| Total | 6928 | 1731 |

Additionally, it should be noted that with the proposed changes the parsing robustness issue described above is also solved.

# Simulation Results

The proposed method is implemented on HM3.0 and simulated under the common test conditions [1]. Results in Table 2 show that there is virtually no change in coding performance.

Table 2. Simulation results on proposed context modifications

|  | HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| AI | 0.07 | -0.95 | -0.91 |
| RA | -0.02 | -0.66 | -0.77 |
| LB | 0.00 | -0.19 | -0.42 |

# Conclusions

With the proposed context modifications, memory requirement associated with context can be significantly reduced. Such context modifications also solve the issue with parsing robustness. Based on simulations, the proposed context modifications do not incur coding performance loss. Therefore we recommend such context modifications to be adopted into HM.

# Reference

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, JCT-VC 5th Meeting: Geneva, March, 2011.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Proposed text change

    - Transform tree semantics

cbf_cb[ x0 ][ y0 ][ trafoDepth ] equal to 1 specifies that the Cb transform block contains one or more transform coefficient levels not equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding units.

When cbf_cb[ x0 ][ y0 ][ trafoDepth ] is not present, the value of cbf_cb[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 0.

cbf_cr[ x0 ][ y0 ][ trafoDepth ] equal to 1 specifies that the Cr transform block contains one or more transform coefficient levels not equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding units.

When cbf_cr[ x0 ][ y0 ][ trafoDepth ] is not present, the value of cbf_cr[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 0.

## Derivation process for ctxIdx

Inputs to this process are binIdx, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset.

Output of this process is ctxIdx.

Table 9-7 shows the assignment of ctxIdx increments (ctxIdxInc) to binIdx for all ctxIdxTable and ctxIdxOffset.

…

Table 9-1 – Association of ctxIdx and syntax elements for each slice type in the initialisation process

|  | Syntax element | ctxIdxTable | Slice Type |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | I | P | B |
| slice_header() | alf_cu_flag | Error: Reference source not found | 0 2 | 3..5 | 6..8 |
| coding_tree() | split_coding_unit_flag | Error: Reference source not found | 0...2 | 3..5 | 6..8 |
| coding_unit() | skip_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | cu_qp_delta | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | pred_type | Error: Reference source not found | 0 | 1..4 | 5..9 |
| prediction_unit() | prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | rem_intra_luma_pred_mode | Error: Reference source not found | 0 | 1 | 2 |
|  | intra_chroma_pred_mode | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | merge_flag | Table 9-2 |  | 0..2 0 | 3..5 1 |
|  | merge_idx | Error: Reference source not found |  | 0..3 | 4..7 |
|  | inter_pred_flag | Table 9-3 |  |  | 0..2 0..3 |
|  | ref_idx_lc, ref_idx_l0, ref_idx_l1 | Table 9-4 |  | 0..5 0..1 | 6..11 2...3 |
|  | mvd_l0[ ][ ][ 0 ] | Table 9-5 |  | 0…4 | 10…14 |
|  | mvd_lc[ ][ ][ 0 ], mvd_l1[ ][ ][ 0 ] | Table 9-5 |  |  | 10…14 |
|  | mvd_l0[ ][ ][ 1 ] | Table 9-5 |  | 5…9 | 15…19 |
|  | mvd_lc[ ][ ][ 1 ], mvd_l1[ ][ ][ 1 ] | Table 9-5 |  |  | 15…19 |
|  | mvp_idx_lc, mvp_idx_l0, mvp_idx_l1 | Error: Reference source not found |  | 0..1 | 2..3 |
| transform_tree() | no_residual_data_flag | Table 9-6 |  | 0..3 0 | 4..7 1 |
|  | split_transform_flag | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | cbf_luma | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | cbf_cb | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | cbf_cr | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
| residual_coding() | last_significant_coeff_x | Error: Reference source not found | 0..40 | 41..81 | 82..122 |
|  | last_significant_coeff_y | Error: Reference source not found | 0..40 | 41..81 | 82..122 |
|  | significant_coeff_flag | needs update | Error: Reference source not found | Error: Reference source not found | Error: Reference source not found |
|  | coeff_abs_level_greater1_flag | Error: Reference source not found | 0..79 | 80..159 | 160..239 |
|  | coeff_abs_level_greater2_flag | Error: Reference source not found | 0..79 | 80..159 | 160..239 |

Table 9-2 – Values of variable m and n for merge_flag ctxIdx

| Initialisation variables | merge_flag ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 |  |  |  |
| m | 0 | 0 | 0 |  |  |  |
| n | 64 | 64 | 64 |  |  |  |

Table 9-3 – Values of variable m and n for inter_pred_flag ctxIdx

| Initialisation variables | inter_pred_flag ctxIdx |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| m | -2 | -5 | -9 | 1 |
| n | 58 | 70 | 85 | 61 |

Table 9-4 – Values of variable m and n for ref_idx_lc, ref_idx_l0, ref_idx_l1 ctxIdx

| Initialisation variables | ref_idx_lc, ref_idx_l0, ref_idx_l1 ctxIdx |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 |  |  |  | 1 | 2 | 3 |  |  | 4 | 5 | 6 |
| m | -6 |  |  |  | 1 | 0 | -9 |  |  |  | -18 | 0 |
| n | 59 |  |  |  | 59 | 64 | 55 |  |  |  | 55 | 64 |

Table 9-5 – Values of variable m and n for mvd_lc, mvd_l0, mvd_l1 ctxIdx

| Initialisation variables | mvd_lc, mvd_l0, mvd_l1 ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 |  |  | 1 | 2 | 3 | 4 | 5 |  |  | 6 | 7 | 8 | 9 |
| m | 5 |  |  | 14 | 16 | 6 | 9 | 2 |  |  | 14 | 17 | 6 | 8 |
| n | 61 |  |  | 39 | 49 | 60 | 71 | 63 |  |  | 37 | 46 | 60 | 73 |
|  | 10 |  |  | 11 | 12 | 13 | 14 | 15 |  |  | 16 | 17 | 18 | 19 |
| m | 1 |  |  | 11 | 14 | 4 | 7 | 1 |  |  | 11 | 15 | 5 | 8 |
| n | 67 |  |  | 49 | 56 | 69 | 77 | 65 |  |  | 48 | 52 | 67 | 76 |

Table 9-6 – Values of variable m and n for no_residual_data_flag ctxIdx

| Initialisation variables | no_residual_data_flag ctxIdx |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 |  |  |  | 2 |  |  |  |
| m | -14 |  |  |  | -26 |  |  |  |
| n | 72 |  |  |  | 88 |  |  |  |

| Table 9-7 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| alf_cu_flag | Error: Reference source not found | 0 | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na |
|  |  | 3 | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na |
|  |  | 6 | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na |
| split_coding_unit_flag | Error: Reference source not found | 0 | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na |
|  |  | 3 | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na |
|  |  | 6 | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na |
| skip_flag | Error: Reference source not found | 0 | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na |
|  |  | 3 | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na |
| cu_qp_delta | Error: Reference source not found | 0 | 0 | 2 | 3 | 3 | 3 |
|  |  | 4 | 0 | 2 | 3 | 3 | 3 |
|  |  | 8 | 0 | 2 | 3 | 3 | 3 |
| pred_type | Error: Reference source not found | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | 1 | 2 | 3 | na |
|  |  | 5 | 0 | 1 | 2 | 3 | 4 |
| prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
|  |  | 2 | 0 | na | na | na | na |
| rem_intra_luma_pred_mode | Error: Reference source not found | 0 | 0 | 0 | 0 | 0 | 0 |
|  |  | 1 | 0 | 0 | 0 | 0 | 0 |
|  |  | 2 | 0 | 0 | 0 | 0 | 0 |
| intra_chroma_pred_mode | Error: Reference source not found | 0 | 0,1,2 (subclause Error: Reference source not found) | 3 | 3 | 3 | na |
|  |  | 4 | 0,1,2 (subclause Error: Reference source not found) | 3 | 3 | 3 | na |
|  |  | 8 | 0,1,2 (subclause Error: Reference source not found) | 3 | 3 | 3 | na |
| merge_flag | Table 9-2 | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
| merge_idx | Error: Reference source not found | 0 | 0,1,2,3 (subclause Error: Reference source not found) | 0,1,2,3 (subclause Error: Reference source not found) | 0,2,3 (subclause Error: Reference source not found) | 3 | na |
|  |  | 4 | 0,1,2,3 (subclause Error: Reference source not found) | 0,1,2,3 (subclause Error: Reference source not found) | 0,2,3 (subclause Error: Reference source not found) | 3 | na |
| inter_pred_flag | Table 9-3 | 0 | cuDepth | na | na | na | na |
| ref_idx_l0 | Table 9-4 | 0 | 0 | 1 | 1 | 1 | 1 |
| ref_idx_l0, ref_idx_l1, ref_idx_lc | Table 9-4 | 2 | 0 | 1 | 1 | 1 | 1 |
| mvd_l0[ ][ ][ 0 ] | Table 9-5 | 0 | 0 | 1 | 2 | 3 | 4 |
| mvd_l0[ ][ ][ 1 ] | Table 9-5 | 5 | 0 | 1 | 2 | 3 | 4 |
| mvd_l0[ ][ ][ 0 ], mvd_lc[ ][ ][ 0 ], mvd_l1[ ][ ][ 0 ] | Table 9-5 | 10 | 0 | 1 | 2 | 3 | 4 |
| mvd_l0[ ][ ][ 1 ], mvd_lc[ ][ ][ 1 ], mvd_l1[ ][ ][ 1 ] | Table 9-5 | 15 | 0 | 1 | 2 | 3 | 4 |
| mvp_idx_l0 | Error: Reference source not found | 0 | 0 | 1 | 1 | 1 | 1 |
| mvp_idx_l0, mvp_idx_l1, mvp_idx_lc | Error: Reference source not found | 2 | 0 | 1 | 1 | 1 | 1 |
| no_residual_data_flag | Table 9-6 | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
| split_transform_flag | Error: Reference source not found | 0 | cuDepth + trafoDepth | na | na | na | na |
|  |  | 4 | cuDepth + trafoDepth | na | na | na | na |
|  |  | 8 | cuDepth + trafoDepth | na | na | na | na |
| cbf_luma | Error: Reference source not found | 0 | 0,1,2,3 (subclause Error: Reference source not found) | na | na | na | na |
|  |  | 4 | 0,1,2,3 (subclause Error: Reference source not found) | na | na | na | na |
|  |  | 8 | 0,1,2,3 (subclause Error: Reference source not found) | na | na | na | na |
|  | Error: Reference source not found |  |  |  |  |  |  |
| cbf_cb | Error: Reference source not found | 0 | trafoDepth | na | na | na | na |
|  |  | 4 | trafoDepth | na | na | na | na |
|  |  | 8 | trafoDepth | na | na | na | na |
|  | Error: Reference source not found |  |  |  |  |  |  |
| cbf_cr | Error: Reference source not found | 0 | trafoDepth | na | na | na | na |
|  |  | 4 | trafoDepth | na | na | na | na |
|  |  | 8 | trafoDepth | na | na | na | na |

Table 9-8 – Specifcation of ctxIdxInc using left and above syntax elements

| Syntax element | condL | condA | ctxIdxInc |
| --- | --- | --- | --- |
| alf_cu_flag | alf_cu_flag[ xL ][ yL ] | alf_cu_flag[ xA ][ yA ] | ( condL && availableL ) + ( condA && availableA ) |
| split_coding_unit_flag | cuDepth[ xL ][ yL ] > cuDepth[ xP ][ yP ] | cuDepth[ xA ][ yA ] > cuDepth[ xP ][ yP ] | ( condL && availableL ) + ( condA && availableA ) |
| skip_flag | skip_flag[ xL ][ yL ] | skip_flag[ xA ][ yA ] | ( condL && availableL ) + ( condA && availableA ) |
| intra_chroma_pred_mode | IntraPredMode[ xL ][ yL ] = = 4 | IntraPredMode [ xA ][ yA ] = = 4 | ( condL && availableL ) + ( condA && availableA ) |
| cbf_luma | cbf_luma[ xL ][ yL ] | cbf_luma[ xA ][ yA ] | ( condL && availableL ) + ( condA && availableA ) << 1 |