| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F610 |
| --- | --- |

| Title: | Fine granularity QP change at slice level |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xianglin Wang, Rajan Joshi, Geert Van Der Auwera, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: +1-858-651-5135 Email: xianglin@qualcomm.com rajanj@qualcomm.com geertv@qualcomm.com martak@qualcomm.com |  |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

A method for sending offset values to modify the quantizer step size at the slice level is proposed. The step-size change is signaled as a percentage of the original quantizer step-size, resulting in much higher granularity compared to what could be achieved through a QP change. The BD-rate for RA-HE and LB-HE configurations is -0.5℅ and 0.9℅, respectively. The BD-rate for RA-LC and LB-LC configurations is -1.1℅ and 1.8℅, respectively. When 'Nebuta' and 'BQTerrace' are excluded from the BD-rate calculations, the BD-rate for RA-HE, LB-HE, RA-LC and LB-LC configurations is -0.9℅, 1.0℅, -1.1℅, and 1.9℅, respectively.

- Introduction

In CE4 Subtest 3 [1], Research in Motion Ltd. (RIM) proposed sending offsets for quantizer reconstruction levels at the slice level. While analyzing their proposal [3], we noticed that the offsets were generally always positive and increased with the level magnitude. We investigated whether it was necessary to send offsets for multiple levels. This proposal describes the outcome of this analysis and proposes an alternative method of achieving slightly improved performance. In RIM's proposal, for each non-intra slice, a number of reconstruction level offsets are signaled to the decoder. On the encoder side, the previous slice with the same QP value is used to gather level offset statistics. For each non-intra slice, up to 4 different sets of offsets may be signaled for intra/inter and luma/chroma. A threshold is applied to the histogram values to signal whether an offset is included in the slice header and this information is signaled to the decoder.

# Proposal

Instead of signaling offsets for different quantization levels, in this proposal, an offset is signaled in the slice header to change the quantizer step size. For each non-intra slice, up to four offsets (intra/inter and luma/chroma) may be signaled to the decoder. The modified quantizer step-size is represented as the original step-size plus the signaled offset. The offset is represented as a percentage of the quantizer step-size, as in RIM's proposal, resulting in much finer granularity than QP change. On the encoder side, the statistics are collected in the same way and offset values are calculated based on the statistics. Then, based on the offsets calculated, a modified quantizer step-size is derived. The modified quantizer step-size is used for deriving the reconstruction levels on the encoder and decoder side.

# Results

The proposed method is implemented in HM3.0. Simulation results are presented for the common test conditions. Table 1 shows the results. The BD-rate for RA-HE and LB-HE configurations is -0.5℅ and 0.9℅, respectively. The BD-rate for RA-LC and LB-LC configurations is -1.1℅ and 1.8℅, respectively. Detailed results are provided in the accompanying excel file. Rate variation measure (RVM) [2] numbers are also reported for LB configurations. It can be seen that there is a small increase in the RVM. During CE4 discussions, there was a discussion of potential problems with BD-rate calculations for 'Nebuta' and 'BQTerrace'. Table 2 reports the BD-rate results and RVM numbers without considering those two sequences.

In Table 3, the overall BD-rate results for this proposal are compared with those reported by RIM [3] and MediaTek [4] for. CE4 Subtest 3. For MediaTek, we have used their best results, which include change in lambda value. In Table 4, the same results are reported without 'Nebuta' and 'BQTerrace'.

Tables 5 and 6 report the average rate differences (in kbps) between the proposed method and HM30 anchors for all sequences and without 'Nebuta' and 'BQTerrace', respectively.

It can be seen from the results that this proposal achieves good performance with minimal signaling and little increase in complexity.

Table 1: BD-rate comparison for all sequence against HM30 anchor.

Table 2: BD-rate results without 'Nebuta' and 'BQTerrace' against HM30 anchor

| configuration | proposal | RIM | MediaTek |
| --- | --- | --- | --- |
| RA-HE | -0.5 | -0.3 | -1.3 |
| RA-LC | -1.1 | -1.0 | -1.6 |
| LB-HE | -0.9 | -0.7 | -0.7 |
| LB-LC | -1.8 | -1.3 | -0.9 |

Table 3: BD-rate comparison of the proposed technique with RIM and MediaTek results in CE4 subtest 3 (all sequences).

| configuration | proposal | RIM | MediaTek |
| --- | --- | --- | --- |
| RA-HE | -0.9 | -0.6 | -0.9 |
| RA-LC | -1.1 | -0.9 | -1.1 |
| LB-HE | -1.0 | -0.8 | -0.6 |
| LB-LC | -1.9 | -1.4 | -0.9 |

Table 4: BD-rate comparison of the proposed technique with RIM and MediaTek results in CE4 subtest 3 (without Nebuta and BQTerrace sequences).

| configuration | ∆-rate-I | ∆-rate-B | ∆-rate (all frames) |
| --- | --- | --- | --- |
| RA-HE | -1.56 | -251.95 | -247.42 |
| RA-LC | -25.91 | -602.06 | -591.78 |
| LB-HE | 12.16 | 22.14 | 22.14 |
| LB-LC | -22.32 | -103.72 | -103.53 |

Table 5: Average bit-rate differences between the proposed method and HM30 (all sequences)

| configuration | ∆-rate-I | ∆-rate-B | ∆-rate (all frames) |
| --- | --- | --- | --- |
| RA-HE | -3.29 | -36.68 | 36.38 |
| RA-LC | -28.74 | -74.63 | -73.14 |
| LB-HE | -8.10 | 10.29 | -8.05 |
| LB-LC | -23.57 | -97.75 | -97.57 |

Table 6: Average bit-rate differences between the proposed method and HM30 (excluding Nebuta and BQTerrace)

- Conclusions

A method for sending offset values to modify the quantizer step size at the slice level is proposed. The step-size change is signaled as a percentage of the original quantizer step-size, resulting in much higher granularity compared to what could be achieved through a QP change. The BD-rate for RA-HE and LB-HE configurations is -0.5℅ and 0.9℅, respectively. The BD-rate for RA-LC and LB-LC configurations is -1.1℅ and 1.8℅, respectively. When 'Nebuta' and 'BQTerrace' are excluded from the BD-rate calculations, the BD-rate for RA-HE, LB-HE, RA-LC and LB-LC configurations is -0.9℅, 1.0℅, -1.1℅, and 1.9℅, respectively. A small increase in RVM measure is observed.

- References

[1]. K. Sato et al, "Description of Core Experiment 4: Quantization", JCTVC-E704, Geneva, CH, March 2011.

[2] M. Horowitz, G. Bjontegaard, T.K. Tan, “Rate Variation Measure”, VCEG-AM10, Kyoto, Japan, Jan. 2010.

[3] X. Yu, D. Wang, and D. He, "CE4-subtest3.3.2: Quantization with Adaptive Reconstruction Levels", JCTVC-F276, Turin, Italy, July 2011.

[4] X. Li, X. Guo, and S. Lei, " CE4 Subtest3: Adaptive De-Quantization Offset", JCTVC-F119, Turin, Italy, July 2011.

# Syntax for Working Draft

### Clause 7.3.3 Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| …… |  |
| slice_qp_delta | se(v) |
| qp_offset_param() |  |
| if( adaptive_loop_filter_enabled_flag ) |  |
| …… |  |
| } |  |

The following table specifies the qp_offset parameters syntax.

| qp_offset_param() { | Descriptor |
| --- | --- |
| qp_offset_slice_flag | u(1) |
| if(qp_offset_slice_flag) |  |
| { |  |
| qp_offset_interY_flag | u(1) |
| if (qp_offset_interY_flag) |  |
| { |  |
| qp_offset_delta_idx[slice_qp][QP_OFFSET_ INTERY] | vlc(v) |
| qp_offset_interUV_flag | u(1) |
| if (qp_offset_interUV_flag) |  |
| { |  |
| qp_offset_delta_idx[slice_qp][QP_OFFSET_INTERUV] | vlc(v) |
| } | u(1) |
| } |  |
| qp_offset_intraY_flag | u(1) |
| if(qp_offset_intraY_flag) |  |
| { |  |
| qp_offset_delta_idx[slice_qp][QP_OFFSET_TYPE_INTRAY] | vlc(v) |
| qp_offset_intraUV_flag | u(1) |
| if(qp_offset_intraUV_flag) |  |
| { |  |
| qp_offset_delta_idx[slice_qp][QP_OFFSET_TYPE_INTRAUV] | vlc(v) |
| } |  |
| } |  |
| } |  |
| } |  |

Name association to qp_offset_data_type_id

| qp_offset_data_type_id | Data type |
| --- | --- |
| 0 | QP_OFFSET_TYPE_INTRAY |
| 1 | QP_OFFSET_TYPE_INTRAUV |
| 2 | QP_OFFSET_TYPE_INTERY |
| 3 | QP_OFFSET_TYPE_INTERUV |

The following semantics is to be added to the Clause 7.4.3 Slice header semantics.

Clause 7.4.3 Slice header semantics

qp_offset_slice_flag specifies whether there are any qp offsets transmitted for this slice.

qp_offset_interUV_flag specifies whether there is any qp offset for InterUV data.

qp_offset_intraY_flag specifies whether there is any qp offset for IntraY data.

qp_offset_intraUV_flag specifies whether there is any qp offset for IntraUV data.

qp_offset_delta_idx[qP][DataType] specifies the qp offset for a given type of Data with quantization parameter qP. Four data types are defined for transmitting qp offsets, i.e., InterY, InterUV, IntraY, and IntraUV. The received index qp_offset_delta_idx[qP][DataType] is firstly converted to a delta value within [-15, 15] by the following procedure:

QPoffsetd[qP][DataType] = Idx2Delta [ qp_offset_delta_idx[qP][DataType]] ;

where Idx2Delta is defined as

int Idx2delta[16] = {-15, -13, -11, -9, -7, -5, -3, 0, 1, 3, 5, 7, 9, 11, 13, 15};
Second, the delta value is used to update the corresponding reconstruction level by

QPoffset[qP][DataType] += QPoffsetd[qP][DataType];

ReconLevel[qP][DataType][i] = (i*QPoffset[qP][DataType]) + (i<<7), i > 0 ;

The following semantics change is to be added to Clause 8.5.3 Scaling process for transform coefficients. Specifically, equation (8-158) is changed to be as follows:

dij = sgn (cij) * ( ReconLevel[qP][DataType][ abs(cij )] * LevelScale(nS)x(nS)[ qP%6 ][ i ][ j ] ) << ( qP/6 + trafoPrecisionExt - 7), with i, j = 0..nS-1	(8-1)

where abs(cij) is the index of the quantized transform coefficient cij and ReconLevel[qP][DataType][ abs(cij )] determines the reconstruction level corresponds to this index.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).