| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F617_r1 |
| --- | --- |

| Title: | Intra prediction based on repetitive pixel replenishment |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seiji Mochizuki Ryoji Hashimoto Kenichi Iwata 5-20-1 Josuihoncho, Kodaira, Tokyo 187-8588, Japan | Tel: Email: | +81-42-312-9038 seiji.mochizuki.zc@renesas.com ryoji.hashimoto.te@renesas.com kenichi.iwata.yb@renesas.com |
| Source: | Renesas Electronics Corporation |  |  |

_____________________________

# Abstract

This contribution presents evaluation results of intra prediction by template matching based on repetitive pixel replenishment (Intra-RPR). The simulation results reportedly show that this proposed technique has an average of -0.3 % (Y), 0.1 % (U), and 0.1 % (V) BD-rate improvement on Intra high efficiency condition and an average of -0.5 % (Y), 0.1 % (U), and 0.0 % (V) BD-rate improvement on Intra low complexity condition against HM3.0. It is also reported that the encoding time increases at 118 % and 133 %, respectively on Intra high efficiency and Intra low complexity, and there is no significant increase as to the decoding time.

# Introduction

Number of bits derived from intra picture average 26 % in alpha anchor of CfP [1] and 53% at most. Thus, focus was on a much reduction of intra-bit and improvement quality of intra picture. It is also good effect on the following picture.

Whereas intra-frame prediction in AVC and current HEVC draft [2] uses only neighboring pixel, more pixels are used to create predicted image in the proposal. Key idea is based on that two dimensional picture has own correlation. Similar texture often exists in a frame. Thus this overlapping redundancy can be reduced by using template matching based on intra vector prediction. This scheme can use more pixels to generate predicted images than the intra prediction of AVC and current HEVC draft.

Based on the above concept, the intra prediction using repetitive pixel replenishment (Intra-RPR) had been proposed at the JCT-VC meetings and it has been reported that this technique has significant improvement of coding efficiency [3][4][5][6]. In this contribution, Intra-RPR was added to HM3.0 with inhibiting increase of computational complexity and evaluated on the coding efficiency and the encoding/decoding time.

# Algorithm

The concept of Intra-RPR is described in Figure 1. The Intra-RPR is to use a displacement vector to select a previously-decoded area within the same picture to use to form a prediction block.

Intra prediction block is created by motion searches around current block. If reference block includes a not-coded area, intra vector (Vx, Vy) is multiplied as (2Vx, 2Vy), and adaptively padding such region by using new reference pixel as shown in Figure 1. A method was described for filling in areas of the picture that have not yet been coded in repeatedly.

This scheme is especially effective to predict the periodical textures. Intra-RPR greatly improves the predicted images compared to the one generated by the anchor of CfP [1] in the contribution of JCTVC-B033 [4]. Predicted image of the proposal reproduce texture of original image faithfully and it worked well in diagonal lines particularly.

Figure 1 Intra prediction based on RPR

The prediction by Intra vector is conducted at each prediction unit (PU) for both luminance and chrominance. But, Intra vector mode is not adapted for PU over 32x32 by the view of trade-off between coding efficiency and computational complexity, because this scheme is more effective for the complicated texture.

Intra vector is searched only in restricted reference area specified reference range (RR) as shown in Figure 1. This reference range is fixed at 16 of integer pixel accuracy for PU of more than 8x8 size and at 12 of integer pixel accuracy for PU of 4x4 size. When the region pointed by the intra vector near reference area boundary includes pixels outside of reference area, those pixels shall be copied from boundary pixels. This restriction reduces computational complexity, line memory size to hold pixels in reference area and the bit cost for vector representation. The accuracy of Intra vector is half pixel for luminance and Intra vector for chrominance is derived with Intra vector for luminance. Because this scheme is effective for the complicated texture, in the case that DC, Horizontal and Vertical are selected as intra prediction mode for current PU, Intra vector search can be skipped to reduce computational complexity.

The Intra vectors are signaled by calculating PMV to neighboring PUs. The method of calculating PMV is the following.

- When left PU is Intra vector mode, PMV shall be equal to Intra vector of the left PU.

- When above PU is Intra vector mode, PMV shall be equal to Intra vector of the above PU.

- Otherwise, PMV shall be inferred to be equal to (-2, -2) of half pixel accuracy.

When current PU is coded by this scheme, DCT and zig-zag scan for the PU shall be selected as transform type and scanning order.

This scheme can be added to other intra prediction mode of current HEVC draft. When the entropy coding mode is CABAC, 1 bit flag intra_vector_flag is added after intra part mode. If current PU is coded by Intra vector mode, the flag shall be set to 1. When the entropy coding mode is CAVLC, intra prediction mode 15 and 27, for 4x4 and otherwise respectively, is replaced with Intra vector mode. If current PU is coded by Intra vector mode, intra prediction mode shall be set to 15 for 4x4 and 27 for others. In this case, replaced intra prediction mode is not used. The detailed bitstream syntax and decoding process of Intra-RPR is described in Annex of this document.

- Experimental results

Intra-RPR integrated to HM3.0 was evaluated under the common test configuration, I slice only coding structure. BD-rate (ΔBitrate [%]) was measured by Excel sheet provided on the JCT-VC reflector.

Table 1 shows the experimental results of Intra-RPR compared with HM3.0 anchor. Intra-RPR has an average of -0.3 % (Y), -0.1 % (U), and -0.2 % (V) BD-rate improvement on Intra high efficiency condition and an average of -0.3 % (Y), -0.1 % (U), and -0.2 % (V) BD-rate improvement on Intra low complexity condition against HM3.0. It is also reported that the encoding time increases at 129 % and 126 %, respectively on Intra high efficiency and Intra low complexity, and there is no significant increase as to the decoding time.

Table 1 BD-rate and encoding/decoding time by Intra-RPR against HM3.0

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.02 | -0.01 | -0.02 | -0.11 | -0.03 | -0.06 |
| Class B | -0.33 | -0.32 | -0.35 | -0.48 | -0.45 | -0.45 |
| Class C | -0.62 | 0.56 | 0.53 | -0.97 | 0.41 | 0.36 |
| Class D | -0.18 | 0.58 | 0.56 | -0.32 | 0.64 | 0.54 |
| Class E | -0.26 | -0.20 | -0.17 | -0.49 | -0.10 | -0.25 |
| Overall | -0.29 | 0.12 | 0.10 | -0.47 | 0.08 | 0.02 |
| Enc Time[%] | 118% |  |  | 133% |  |  |
| Dec Time[%] | 98% |  |  | 96% |  |  |

- Conclusion

The intra prediction based on repetitive pixel replenishment (Intra-RPR) is proposed. This proposed method improves the quality of predicted image greatly. The average BD-rate reduction is -0.3 % (Y), 0.1 % (U), and 0.1 % (V) BD-rate improvement on Intra high efficiency condition and -0.5 % (Y), 0.1 % (U), and 0.0 % (V) on Intra low complexity, with increasing the encoding time at less than 135 %. There is no significant increase as to the decoding time.

# Patent rights declaration(s)

Renesas Electronics Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/16, “Joint Call for Proposals on Video Compression Technology,” WG11 document N11113 and Q6/16 document VCEG-AM91, Kyoto, Jan. 2010.

[2] Thomas Wiegand, Woo-Jin Han, Benjamin Bross, Jens-Rainer Ohm, Gary J. Sullivan, “WD3: Working Draft 3 of High-Efficiency Video Coding,” document JCTVC-E603, Geneva, Mar. 2011.

[3] Seiji Mochizuki, Kenichi Iwata, “Response to Joint Call for Proposals on Video Compression Technology: Renesas,” document JCTVC-A126, Dresden, Apr. 2010.

[4] Kenichi Iwata, Seiji Mochizuki, Ryoji Hashimoto, “Intra prediction based on repetitive pixel replenishment,” document JCTVC-B033, Geneva, Jul. 2010.

[5] Seiji Mochizuki, Kenichi Iwata, Ryoji Hashimoto, “Evaluation of intra prediction based on repetitive pixel replenishment,” document JCTVC-C189, Guangzhou, Oct. 2010.

[6] Kenichi Iwata, Seiji Mochizuki, Ryoji Hashimoto, “Intra prediction based on repetitive pixel replenishment with adaptive block size,” document JCTVC-D251, Daegu, Jan. 2011.

# Annex

### Syntax

| prediction_unit( x0, y0, log2PUWidth, log2PUHeight, PartIdx , InferredMergeFlag ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( NumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode == PART_2Nx2N && log2PUWidth >= Log2IPCMCUSize ) |  |
| pcm_flag | u(1) ¦ ae(v) |
| if( pcm_flag ) { |  |
| while ( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| for( i = 0; i < 1 << ( log2CUSize << 1 ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( 1 << ( log2CUSize << 1 ) ) >> 1; i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| } else { |  |
| if ( entropy_coding_mode_flag && PartIdx == 0 ) |  |
| intra_vector_flag[ x0 ][ y0 ] | ae(v) |
| if ( !intra_vector_flag[ x0 ][ y0 ] ¦¦ !entropy_coding_mode_flag )) { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| if( NumMPMCand > 1 ) |  |
| mpm_idx[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| Else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ce(v) ¦ ae(v) |
| if( IntraPredMode[ x0 ][ y0 ] == 2 ) |  |
| planar_flag_luma[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| else if(( log2PUWidth == 2 && IntraPredMode[ x0 ][ y0 ] == 15 && !entropy_coding_mode_flag ) ¦¦ ( log2PUWidth > 2 && IntraPredMode[ x0 ][ y0 ] == 27 && !entropy_coding_mode_flag )) |  |
| Intra_vector_flag[ x0 ][ y0 ] = 1 |  |
| } |  |
| if ( intra_vector_flag[ x0 ][ y0] ) { |  |
| Intra_vector_difference[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| Intra_vector_difference[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| } else { |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| SignaledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] == 3 : intra_chroma_pred_mode[ x0 ][ y0 ] == 2 ) |  |
| if( IntraPredMode[ x0 ][ y0 ] != 2 && IntraPredMode[ x0 ][ y0 ]!=34 && SignaledAsChromaDC ) |  |
| planar_flag_chroma[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| } |  |
| } |  |
| } else { /* MODE_INTER */ |  |
| : |  |

### Semantics

intra_vector_flag specifies the type of prediction unit. If intra_vector_flag is equal to 1, the prediction unit is coded as intra vector mode, otherwise the prediction unit is coded as intra prediction mode.

When intra_vector_flag is not present, it is inferred to be equal to 0.

intra_vector_difference[x0][y0][compIdx] specifies the difference between a intra vector component to be used and its prediction. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. The horizontal motion vector component difference is assigned compIdx = 0 and the vertical motion vector component is assigned compIdx = 1.

When any of the two components is not present, the inferred value is 0.

### Decoding process

##### Derivation Process for Intra Vector Difference

Input of this process is bitstream

Output of this process is Intra Vector Difference

Each component of Intra Vector Difference is decoded from bitstream.

If entropy_coding_flag == 0(CAVLC)

Intra Vector Difference is decoded by se(v)

Otherwise (entropy_coding_flag is equal to 1(CABAC))

First, intra_vector_differnece_zero_flag is decoded by binary decision with Context index IntraVectorZero

If intra_vector_differnece_zero_flag is equal to 0

Intra Vector Difference[x/y] is equal to 0

Otherwise, intra_vector_big4_flag is decoded by binary decision decoding with Context Index IntraVectorOne

If intra_vector_big4_flag is equal to 0, additional 3 bins are decoded with bypass decoding.

Absolute value of Intra Vector Difference[x/y]) = bin0 * 2 + bin1 + 1

If last bin is equal to 0, Intra Vector Difference is positive value.

Otherwise (last bin is equal to 1), Intra Vector Difference is negative value.

Otherwise, additional 6 bins are decoded with bypass mode.

Absolute value of Intra Vector Difference[x/y]) =

bin0 * 32 + bin1 * 16 + bin2 * 8 + bin3 * 4+ bin4 * 2 + bin5 + 5

If last bin is equal to 0, Intra Vector Difference is positive value.

Otherwise (last bin is equal to 1), Intra Vector Difference is negative value.

##### Derivation Process for Intra Vector Predictor

Inputs of this process are neighboring blocks of current block

Output of this process is Intra Vector Predictor

If left block is available and prediction mode of left block is intra vector,

Intra Vector Predictor of current block is equal to the intra vector of left block.

Otherwise (left block is unavailable or prediction mode of left block is not intra vector),

if up block is available and prediction mode of up block is intra vector,

Intra Vector Predictor of current block is equal to the intra vector of up block.

Otherwise,

Intra Vector Predictor of current block is set to default value (-1, -1)

##### Specification of Intra_Vector prediction mode

Inputs to this process are:

–	neighbouring samples p[ x, y ], with x, y = -1..2*nS-1,

–	a variable nS specifying the prediction size,The

–	a luma Intra vector iv given in half-luma-sample units,

–	a chroma Intra vector ivC given in quarter-chroma-sample units,

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS-1.

This intra prediction mode is invoked when intra_vector_flag is equal to 1.

Intra vector iv are derived by

iv[ 0 ] = ivp[ 0 ] + intra_vector_difference[ x0 ][ y0 ][ 0 ]

iv[ 1 ] = ivp[ 1 ] + intra_vector_difference[ x0 ][ y0 ]	[ 1 ]

Intra vector predictor ivp are derived by

- when left PU is coded by intra vector mode, ivp[ 0 ] = ivA[ 0 ], ivp[ 1 ] = ivA[ 1 ]

- when above PU is coded by intra vector mode, ivp[ 0 ] = ivB[ 0 ], ivp[ 1 ] = ivB[ 1 ]

- otherwise, ivp[ 0 ] = -2, ivp[ 1 ] = -2

where ivA is Intra vector of left PU and ivB is Intra vector of above PU.

The position of the reference samples ( refPosXa[ x, y ], refPosYa[ x, y ] ), ( refPosXb[ x, y ], refPosYb[ x, y ] ), ( refPosXc[ x, y ], refPosYc[ x, y ] ), ( refPosXd[ x, y ], refPosYd[ x, y ] ), with x, y = 0..nS-1, are derived by

refPosXa[ x, y ] = refPosXc[ x, y ] = x + ( iv[ 0 ] >> 1 )
refPosXb[ x, y ] = refPosXd[ x, y ] = x + ( iv[ 0 ] >> 1 ) + 1
refPosYa[ x, y ] = refPosYb[ x, y ] = y + ( iv[ 1 ] >> 1 )
refPosYc[ x, y ] = refPosYd[ x, y ] = y + ( iv[ 1 ] >> 1 ) + 1
with x, y = 0..nS-1

When the position of the reference samples is not coded yet, they are derived by

refPosXa[ x, y ] = refPosXc[ x, y ] = x + (( iv[ 0 ] << 1 ) >> 1 )
refPosXb[ x, y ] = refPosXd[ x, y ] = x + (( iv[ 0 ] << 1 ) >> 1 ) + 1
refPosYa[ x, y ] = refPosYb[ x, y ] = y + (( iv[ 1 ] << 1 ) >> 1 )
refPosYc[ x, y ] = refPosYd[ x, y ] = y + (( iv[ 1 ] << 1 ) >> 1 ) + 1
with x, y = 0..nS-1

This derivation process for the position of the reference samples are repeated until the position is available.

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1, are derived by

predSamples[ x, y ] = 
		p[ refPosXa[ x, y ], refPosYa[ x, y ] ]
with x, y = 0..nS-1 where iv[ 0 ] % 2 = 0 and iv[ 1 ] % 2 = 0

predSamples[ x, y ] = 
		( p[ refPosXa[ x, y ], refPosYa[ x, y ] ] +
		 p[ refPosXb[ x, y ], refPosYb[ x, y ] ] + 1 ) >> 1
with x, y = 0..nS-1 where iv[ 0 ] % 2 = 1 and iv[ 1 ] % 2 = 0

predSamples[ x, y ] = 
		( p[refPosXa[ x, y ], refPosYa[ x, y ] ] +
		 p[ refPosXc[ x, y ], refPosYc[ x, y ] ] + 1 ) >> 1
with x, y = 0..nS-1 where iv[ 0 ] % 2 = 0 and iv[ 1 ] % 2 = 1

predSamples[ x, y ] = 
		( p[refPosXa[ x, y ], refPosYa[ x, y ] ] +
		 p[ refPosXb[ x, y ], refPosYb[ x, y ] ] +
		 p[ refPosXc[ x, y ], refPosYc[ x, y ] ] +
		 p[ refPosXd[ x, y ], refPosYd[ x, y ] ] + 2 ) >> 2
with x, y = 0..nS-1 where iv[ 0 ] % 2 = 1 and iv[ 1 ] % 2 = 1