| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F626 |
| --- | --- |

| Title: | Cross-verification result of JCTVC-F427 proposed by Sony |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kimihiko Kazui 4-1-1 Kamikodanaka, Nakahara-ku 211-8588 Kawasaki JAPAN | Tel: Email: | +81-44-754-2639 kazui.kimihiko@jp.fujitsu.com |
| Source: | FUJITSU LABORATORIES LTD. |  |  |

_____________________________

# Abstract

This document reports the cross-verification result of JCTVC-F427 proposed by Sony [1]. JCTVC-F427 proposes to modify the derivation process of luma motion vector candidates.

The result is identical to the result reported from the proponent.

# Verified scheme

This cross-verification tests item (b) in JCTVC-F427.

Item (b) proposes to modify the derivation process of luma spatial motion vector candidates. Specifically, the following procedure is added to the process.

- If all of the spatial predictors are not available, the spatial motion vector predictor is set ( 0, 0 ) regardless the availability of the temporal predictor.

# Simulation result

Table 1 summarizes the coding gain in BD-Bitrate of the scheme proposed in JCTVC-F427 item (b) over HM3.0 with the fix of bug #146. The test condition accords with the condition specified in JCTVC-E700 [2].

Table 1: Summary results of BD-Rate comparing Proposal (item (b)) to HM3.0

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 |
| Class B | -0.1 | -0.1 | 0.0 | 0.0 | -0.1 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 |
| Class D | -0.1 | 0.1 | 0.0 | -0.1 | -0.1 | 0.0 |
| Class E |  |  |  |  |  |  |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.2 | -0.5 | -0.1 | -0.1 | 0.0 |
| Class C | 0.0 | 0.0 | -0.1 | -0.1 | 0.0 | -0.1 |
| Class D | -0.1 | -0.1 | 0.1 | 0.0 | 0.2 | -0.1 |
| Class E | -0.1 | 0.3 | 0.0 | -0.1 | 0.0 | 0.7 |
| Overall | 0.0 | 0.1 | -0.1 | -0.1 | 0.0 | 0.1 |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

The result is identical to the result reported from the proponent (Sony).

References

[1] K.Sato, "Consideration of Temporal Predictor", JCTVC-F427, July 2011, Torino IT

[2] "Common test conditions and software reference configurations", JCTVC-E700, March 2011, Geneva CH