| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F627 WG11 Number: m21065 |
| --- | --- |

| Title: | Cross-check of Motorola Mobility’s proposal on combined reference list construction (JCTVC-F573) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chuohao Yeo 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408-2000 chyeo@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

This contribution provides a summary of the cross-checks performed by the Institute for Infocomm Research on a Motorola Mobility’s proposal (JCTVC-F573) that changes the default combined reference list construction in HM3. The cross-check rate-distortion performance results match those provided by Motorola Mobility. The BD-Rate figures for modified Random Access HE and LC are -0.1% and 0.0% respectively, in which the configuration is modified to have 3 non-reference B frames inserted between GPB frames.

# Introduction

In HM3, when uni-directional inter prediction is performed, the reference frame is chosen from a combined reference list construction from reference lists 0 and 1. The combined list construction simply adds reference indices from lists 0 and 1 alternately while avoiding repeats.

In JCTVC-F573, Motorola Mobility proposed an alternative for the construction of the combined list [1]. In the proposal, reference frames from lists 0 and 1 are added to the combined list in ascending order of their temporal distance from the source frame, while ties are broken by first choosing the reference frame in the temporal past (list 0) of the source frame.

We have studied the code provided by the proponents, and it implements the proposed combined list construction.

# Simulation Results

The implementation is on HM 3.0. Tests were conducted on a modified random access configurations on the specified QPs (22, 27, 32, 37) [2]. The modification to the random access configuration was to disable hierarchical-B (HierarchicalCoding = 0), change the GOP size to 4 (GOPSize = 4, RateGOPSize = 4). Furthermore, the code was modified to enable non-reference B frames to be placed between GPB frames. Therefore, in the tested configuration, 3 non-reference B frames are inserted between GPB frames. The anchor uses this modified random access configuration, while the proposed scheme also uses the same modified random access configuration together with the proposed combined reference list construction.

## RD results

Table 1 summarizes the BD-Rate results for the proposed scheme. The results match what was provided by the proponents.

Table 1. BD-Rate results for F573

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | -0.1 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class C | -0.1 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class D | -0.1 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| Overall | -0.1 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

More detailed coding results can be found in the Excel attachment.

# Conclusion

We have cross-checked Motorola Mobility’s proposal [1], studied the algorithm and verified their coding simulation results.

# References

[1] S. Fang, Y. Yu and L. Wang, “The Construction of Combined List for HEVC,” in JCTVC-F573, Torino, Italy, July. 2011.

[2] Frank Bossen, “Common test conditions and software reference configurations,” JCTVC-E700, Geneva, Switzerland, Mar 2011.