| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F628 |
| --- | --- |

| Title: | CE6.d: Cross Check Result for Sharp’s Parallel Intra Coding (JCTVC-F605) by ETRI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Seunghyun Cho Sukho Lee Nakwoong Eum 138 Gajeongno, Yuseong-gu, Daejeon, 305-700, KOREA | Tel: Email: | +82-42-860-6274 shcho@etri.re.kr shlee99@etri.re.kr nweum@etri.re.kr |
| Source: | Electronics and Telecommunications Research Institute (ETRI) |  |  |

_____________________________

# Abstract

This contribution contains cross-checking results of the software containing the tool of JCTVC-F605 proposed by SHARP Corporation on Core Experiment 6 subset d of intra prediction improvement. The software contains three parallel configurations for an 8x8 block that is divided to four 4x4 blocks:

- Checker board partition

- Stripe partition

- 8x8 prediction with 4x4 residuals

This document provides experimental results of “8x8 prediction with 4x4 residuals”.

The verification task has been done successfully and the results perfectly match those proposed in JCTVC-F605.

# Cross-check Results

The software under test was based on HM version 3.0 and the experimental results of the proponent were provided on July 2, 2011 by SHARP. The software was complied fine and the cross-checking was performed on 64bit environments. The simulation has been conducted under CE common test conditions [3], [4] for both high efficiency and low complexity conditions in the case of I-slice only.

## Configurations

The following specifies configuration of the software that is cross checked:

- 8x8 prediction with 4x4 residuals:

All 4x4 blocks have the same mode and they are predicted using 8x8 block neighbors. But each 4x4 is 4x4 transformed.

## Environments

Table 1 shows the computational environments used for cross-checking. Three machines which have exactly same environments are used for the experiments for cross-checking. Class A sequences are tested on machine 1, class B and E sequences are tested on machine 2, and class C and D sequences are tested on machine 3 for both anchor and proposal.

Table 1: Computational environments for cross-checking

| CPU type | Intel Core I7-980X (12M Cache, 3.33GHz) |
| --- | --- |
| Main memory | 12GB DDR3 |
| Storage | 1TB HDD connected with SATA2 |
| Operating System | CentOS (Linux ver. 2.6.18-194.3.1.el5) |
| Compiler | GCC 4.1.2.20080704 (Red Hat 4.1.2-48) |

## Experimental results

The encoded images match those of the decoded ones, and the PSNR and bitrate values are identical to the results provided by SHARP. Table 2 shows the cross-checking results of 8x8 prediction with 4x4 residuals. It is noticed that disabling 4x4 prediction i.e., 8x8 prediction with 4x4 residuals results in 9.0% rate increase for class D under intra LC configuration. Overall rate increases are 3.9% and 5.3% for intra HE and intra LC configurations. Therefore we agree with the SHARP’s opinion that simply disabling 4x4 prediction while allowing 8x8 prediction with 4x4 residuals is not a good option due to the substantial increase of bitrate.

We checked the source code and confirmed that the technique integrated in the software matches JCTVC-F605 contributed by SHARP completely.

Table 2: Cross-checking results of Parallel Intra Prediction using 8x8 prediction with 4x4 residuals vs. HM3.0 Anchor

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 1.8 | 0.4 | 0.5 | 2.2 | 1.2 | 1.4 |
| Class B | 2.4 | 0.9 | 0.7 | 3.4 | 1.8 | 1.6 |
| Class C | 5.5 | 2.8 | 2.8 | 7.5 | 4.2 | 3.9 |
| Class D | 6.6 | 3.7 | 3.8 | 9.0 | 4.8 | 4.8 |
| Class E | 3.7 | 1.4 | 1.7 | 4.8 | 1.5 | 2.0 |
| Overall | 3.9 | 1.8 | 1.9 | 5.3 | 2.7 | 2.7 |
| Enc Time[%] | 80% |  |  | 72% |  |  |
| Dec Time[%] | 101% |  |  | 98% |  |  |

The accompanied spreadsheet contains the detailed data of the cross-checking results.

# References

- JCTVC-D074, “Parallel Prediction Unit for Parallel Intra Coding,” 4th JCT-VC Meeting, Daegu, Jan. 2011.

- JCTVC-E315, “Parallel Prediction Unit for Parallel Intra Coding,” 5th JCT-VC Meeting, Geneva, Mar. 2011.

- JCTVC-E706, “Description of Core Experiment 6: Intra Prediction Improvement”, 5th JCT-VC Meeting, Geneva, Mar. 2011.

- JCTVC-E700, “Common test conditions and software reference configurations,” 5th JCT-VC meeting, Geneva, Mar. 2011.