| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, 16-23 March, 2011 | Document: JCTVC-F632 WG11 Number: m21071 |
| --- | --- |

| Title: | Cross-check of Huawei’s proposal (JCTVC-F584): Modification to DC prediction in SDIP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | J. Jung, J. Le Tanou Orange Labs 38-40 rue du Général Leclerc 9279 ISSY LES MOULINEAUX | Tel: Email: | +33 1 45 29 53 66 joelb.jung@orange-ftgroup.com jletanou.ext@orange-ftgroup.com |
| Source: | Orange Labs |  |  |

_____________________________

# Abstract

The document reports cross-check results of Huawei’s proposal JCTVC-F584. A change on DC prediction of SDIP PU is proposed with only shifting to avoid division operation. It has been verified that common conditions are followed; results provided by Huawei for case 1 (Simplified DC prediction in SDIP), case 2 (all harmonization for SDIP as described in JCTVC-F532) and case 3 (all harmonization for SDIP plus the proposed modification) are complete and match with the results reported in this document. The source code has also been checked, the software has been compiled and no issue is reported.

# Introduction

DC prediction in HM3.0 uses two sides of the reference samples when above and left reference samples are available. DC value computation ends up with a division operation by the sum of width and height. For non SDIP PU the sum is a form of power of 2 and shifting could be applied, it is not always possible for SDIP PU.

In JCTVC-F584, a DC prediction simplification for SDIP PU is proposed, which exploits only one side of the reference samples corresponding to the longer side of current SDIP PU in order to replace division operation with shifting. Impact on R-D performance of the simplification is evaluated.

# Results

Three test cases are evaluated from JCTVC-F584:

- Proposed simplification on DC prediction in SDIP (Table 1)

- All harmonization for SDIP as described in JCTVC-F532 (Table 2)

- All harmonization for SDIP plus proposed modification on DC prediction in SDIP (Table 3)

The software has been verified and does not raise any issue.

The cross check of the results have been done for all classes and IHE/ILC configurations sequences. Results obtained by Huawei and Orange Labs are reported in the following tables.

A perfect match is achieved for BD-Rate results. For information, Encoding/Decoding times are provided but these measurements cannot be considered as truly accurate since computed on a PC cluster.

Table 1: Cross-check results of the simplified DC prediction in SDIP

|  | Intra HE Y BD-rate |  | Intra LC Y BD-rate |  |
| --- | --- | --- | --- | --- |
|  | Huawei | Orange Labs | Huawei | Orange Labs |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 |
| AVG | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] Dec Time[%] | 100% 101% | 100% 98% | 100% 101% | 99% 104% |

Table 2: Cross-check results of all harmonization for SDIP as described in JCTVC-F532

|  | Intra HE Y BD-rate |  | Intra LC Y BD-rate |  |
| --- | --- | --- | --- | --- |
|  | Huawei | Orange Labs | Huawei | Orange Labs |
| Class A | -0.04 | -0.04 | -0.41 | -0.41 |
| Class B | -0.19 | -0.19 | -0.65 | -0.65 |
| Class C | -0.03 | -0.03 | -0.35 | -0.35 |
| Class D | -0.04 | -0.04 | -0.29 | -0.29 |
| Class E | -0.47 | -0.47 | -1.20 | -1.20 |
| AVG | -0.14 | -0.14 | -0.55 | -0.55 |
| Enc Time[%] Dec Time[%] | 101% 99% | 101% 99% | 104% 103% | 101% 106% |

Table 3: Cross-check results of all the harmonization plus the proposed modification

|  | Intra HE Y BD-rate |  | Intra LC Y BD-rate |  |
| --- | --- | --- | --- | --- |
|  | Huawei | Orange Labs | Huawei | Orange Labs |
| Class A | -0.04 | -0.04 | -0.41 | -0.41 |
| Class B | -0.20 | -0.20 | -0.98 | -0.98 |
| Class C | -0.03 | -0.03 | -0.37 | -0.37 |
| Class D | -0.06 | -0.06 | -0.30 | -0.30 |
| Class E | -0.47 | -0.47 | -1.21 | -1.21 |
| AVG | -0.15 | -0.15 | -0.56 | -0.56 |
| Enc Time[%] Dec Time[%] | 101% 101% | 101% 98% | 103% 103% | 101% 106% |

# Conclusion

This verification contribution confirms the results obtained by Huawei for the different test cases described in JCTVC-F584. The proposed simplification has no impact on R-D performance of HM3.0-SDIP software. Small BD-rate gain is reported based on HM3.0-SDIP with all harmonization as described in JCT-VC532.