| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F638 |
| --- | --- |

| Title: | CE6.b: Cross-verification of Qualcomm’s JCTVC-F556 section 4 on SDIP Harmonization for HE Residual Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jianle Chen | Email: | v.seregin@samsung.com jianle.chen@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

In this document software of additional harmonization of HE Residual Coding with SDIP, described in section 4 of SDIP Harmonization with Deblocking, MDIS, MDCS, and HE Residual Coding proposal JCTVC-F556, was checked and proponent’s simulation results were verified.

# Introduction

Software based on HM3.0+SDIP was provided by proponent. The following modifications were done related to residual coding for CABAC case:

## Coefficient scanning

- Three scanning patterns are used for 8x2, 2x8, 16x4 and 4x16 rectangular TUs: horizontal, vertical and kind of zig-zag, and one scanning pattern type of zig-zag is applied for 32x8 and 8x32 TUs.

- Scans are chosen according to intra mode direction and mapping table is same as anchor.

- Wavefront type of zig-zag is enabled by default and controlled by QC_WAVEFRONT_SDIP macro

## Last Significant Coefficient Position

Last position coding is same as anchor with necessary adjustments, following anchor mechanism, for rectangular case.

## Significance Map

- Contexts are shared for the rectangular TUs with same size, e.g. 8x2 and 2x8.

- Necessary adjustments, following anchor, were performed for rectangular TUs context modelling: with 64 coefficients or less position dependent context modelling is used and neighbour derived context models is applied for larger TUs.

## Coefficient Level Coding

- Since 4x4 sub-block division is not possible for rectangular shape with size smaller than 4 by one of the sides, JCTVC-E335/ JCTVC-F288 part 1 [2] algorithm is reused for this purpose. Block is divided into sub-sets of 16 consecutive coefficients in the scan order, and the sub-sets are treated similar to anchor’s 4x4 sub-blocks.

- Also, this mechanism is applied for square TUs.

# Test Results

Software was compiled on 64-bit Linux and executed on Linux cluster. The following tables represent test results, more details can be found in accompanying Excel sheets.

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.15 | 0.35 | 0.21 |
| Class B | -0.29 | -0.08 | -0.19 |
| Class C | -0.26 | -0.16 | -0.16 |
| Class D | -0.22 | -0.05 | -0.06 |
| Class E | -0.09 | 0.14 | 0.16 |
| Overall | -0.21 | 0.03 | -0.03 |
| Enc Time[%] | 94% |  |  |
| Dec Time[%] | 97% |  |  |

It is also verified that there is no mismatch between encoder and decoder reconstructed YUV files and coding efficiency results are identical with proponent’s data.

# References

- G. Van der Auwera, J. Sole, Y. Zheng, X. Wang, I. S. Chong, R. Joshi and M. Karczewicz, “CE6.b: Report on SDIP Harmonization with Deblocking, MDIS, MDCS, and HE Residual Coding”, JCTVC-F556, 6th JCT-VC Meeting, Torino, IT, July, 2011.

- J. Sole, R. Joshi and M. Karczewicz, “Parallelization Unified scans for the significance map and coefficient level coding in high coding efficiency,” JCTVC-E335, 5th JCT-VC Meeting, Geneva, Switzerland, Mar. 2011.