| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F645 |
| --- | --- |

| Title: | Cross-check report for coding with a single, unified reference picture list (JCTVC-F549) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Il-Koo Kim | Email: | ilkoo.kim@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

# Introduction

In JCTVC-F549 [1], a single unified reference picture list is proposed. Concept of reference pair is introduced and single codebook is used instead of 3 list (L0, L1 and combined list)-based coding. In addition, this proposal contains several improvements such as ME modification, all reference checking for the collocated MVP and same estimating cost for the second partition in 2NxN and Nx2N.

# Experimental results

The software based on the HM-3.0[2] was provided by BBC and the test was done under the common test condition of HEVC [3]. The software was configured to emulate the equivalent of the HM-3.0 reference pairs. It is reported that average gain is 0.3% with 3% encoding time and 2% decoding time increase. The BD-rates are matched with those of JCTVC-F549.

Table 1 Test results of single unified reference picture list

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.0 | -1.6 | -1.9 | -1.2 | -1.0 | -1.2 |
| Class B | -0.2 | -0.2 | -0.1 | -0.3 | -0.2 | -0.3 |
| Class C | -0.3 | -0.2 | -0.2 | -0.4 | -0.5 | -0.5 |
| Class D | -0.2 | -0.2 | -0.1 | -0.4 | -0.3 | -0.4 |
| Class E |  |  |  |  |  |  |
| Overall | -0.4 | -0.5 | -0.6 | -0.6 | -0.5 | -0.6 |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1 | 0.2 | 0.0 | 0.0 | -0.1 | -0.3 |
| Class C | 0.0 | 0.1 | 0.0 | -0.2 | -0.3 | -0.5 |
| Class D | -0.1 | -0.2 | 0.0 | -0.3 | -0.4 | -0.6 |
| Class E | -0.1 | 0.3 | 0.1 | -0.5 | -0.7 | -0.8 |
| Overall | 0.0 | 0.1 | 0.0 | -0.2 | -0.4 | -0.5 |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  |

# References

- David Flynn, Nikola Sprljan and Marta Mrak, “Coding with a single, unified reference picture list”, JCTVC-F549, 6th Meeting: Torino, IT, 14-22 July, 2011

- https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-3.0/

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, 5th Meeting: Geneva, CH, 16-23 March, 2011