| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F646 |
| --- | --- |

| Title: | Cross-verification of MediaTek’s JCTVC-F124 on Extended Mode-Dependent Coefficient Scanning |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jianle Chen Vadim Seregin | Email: | jianle.chen@samsung.com v.seregin@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This document reports cross-checking results Extended Mode-Dependent Coefficient Scanning in JCTVC-F124. The results confirm the BD-rates and encoding/decoding time from the proponents. The proponents’ code has also been analysed and found to match to the context in the proposal.

# Test Results

In this cross-checking report, Extended Mode-Dependent Coefficient Scanning in JCTVC-F124 is verified. Software was compiled on 64-bit Linux and executed on Linux cluster.

Three tests with all intra configuration were done to verify the performance of 2 method,

- A modified MDCS is applied for larger block sizes including 16×16 and 32×32;

- MDCS is applied for scanning the absolute values in CABAC entropy coding.

The following tables represent test results, more details can be found in the accompanying Excel sheets.

Table.1  Results of MDCS on large block size

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | -0.2 | -0.2 | -0.3 | -0.7 | -0.6 |
| Class B | -0.1 | -0.6 | -0.6 | -0.9 | -1.4 | -1.4 |
| Class C | -0.1 | -0.3 | -0.3 | -0.2 | -0.5 | -0.6 |
| Class D | -0.1 | -0.3 | -0.2 | -0.4 | -0.5 | -0.4 |
| Class E | -0.5 | -1.0 | -1.0 | -1.2 | -1.8 | -1.5 |
| Overall | -0.1 | -0.5 | -0.4 | -0.6 | -1.0 | -0.9 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Table.1  Results of MDCS for 4x4 magnitude coding

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class B | -0.1 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class C | -0.1 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class D | -0.2 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class E | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Overall | -0.1 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table.1  Results of combination of two methods

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1 | -0.3 | -0.2 | -0.3 | -0.7 | -0.6 |
| Class B | -0.2 | -0.8 | -0.7 | -0.9 | -1.4 | -1.4 |
| Class C | -0.2 | -0.4 | -0.4 | -0.2 | -0.5 | -0.6 |
| Class D | -0.2 | -0.5 | -0.4 | -0.4 | -0.5 | -0.4 |
| Class E | -0.6 | -1.1 | -1.1 | -1.2 | -1.8 | -1.5 |
| Overall | -0.3 | -0.6 | -0.5 | -0.6 | -1.0 | -0.9 |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

# Conclusion

In this document, results of Extended Mode-Dependent Coefficient Scanning in JCTVC-F124 were confirmed. The code has also been analysed and found to match to the description in the proposal.

# References

- X. Zhao, X. Guo, M. Guo, S. Lei, S. Ma, W. Gao, “Extended Mode-Dependent Coefficient Scanning”, JCTVC-F124, 6th JCT-VC Meeting, Torino, IT, July, 2011.