| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F647 |
| --- | --- |

| Title: | Cross-verification of Mitsubishi’s JCTVC-F173 on chroma intra prediction from luma improvement |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jianle Chen Vadim Seregin | Email: | jianle.chen@samsung.com v.seregin@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This document reports cross-checking results of chroma intra prediction from luma improvement in JCTVC-F173. The results confirm the BD-rates and encoding/decoding time from the proponents. The proponents’ code has also been analysed and found to match to the context in the proposal.

# Test Results

In this cross-checking report, chroma intra prediction from luma improvement in JCTVC-F173 is verified. Software was compiled on 64-bit Linux and executed on Linux cluster. The following tables represent test results, more details can be found in the accompanying Excel sheets.

Two tests with all intra configuration were verified. Table 1 shows the result with [1, 2, 1; 1, 2, 1] filtering on reference pixels. And Table 2 shows the results with [1, 0, 1; 1, 0, 1] filtering on reference pixels.

Table.1 the result with [1, 2, 1; 1, 2, 1] filtering on luma reference pixels

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | -0.1 | 0.6 | 0.0 | -0.1 | 0.7 |
| Class B | -0.1 | -0.2 | -0.2 | -0.1 | -0.6 | -0.4 |
| Class C | -0.3 | -1.1 | -1.0 | -0.3 | -1.7 | -1.5 |
| Class D | -0.2 | -0.8 | -0.8 | -0.2 | -1.2 | -1.2 |
| Class E | 0.0 | -0.2 | -0.4 | 0.0 | -0.3 | -0.5 |
| Overall | -0.1 | -0.5 | -0.3 | -0.1 | -0.8 | -0.6 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table.2 the results with [1, 0, 1; 1, 0, 1] filtering on luma reference pixels

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | -0.4 | 0.4 | 0.0 | -0.5 | 0.3 |
| Class B | -0.1 | -0.3 | -0.2 | -0.1 | -0.7 | -0.4 |
| Class C | -0.3 | -1.1 | -1.1 | -0.3 | -1.8 | -1.7 |
| Class D | -0.2 | -0.7 | -0.7 | -0.2 | -1.1 | -1.1 |
| Class E | 0.0 | -0.2 | -0.4 | 0.0 | -0.3 | -0.6 |
| Overall | -0.1 | -0.6 | -0.4 | -0.1 | -0.9 | -0.7 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusion

In this document, results of chroma intra prediction from luma improvement in JCTVC-F173 were confirmed. The code has also been analysed and found to match to the context in the proposal.

# References

- A. Minezawa, K. Sugimoto, S. Sekiguchi, “An improvement to chroma intra prediction from luma”, JCTVC-F173, 6th JCT-VC Meeting, Torino, IT, July, 2011.