| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F648 |
| --- | --- |

| Title: | CE4: Result of Combination 2.3.g + 2.3.e |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kazushi Sato W12F 1-11-1 GCO Osaki, Shinagawa, 141-0032 Tokyo, Japan | Tel: Email: | +81 3 5435 3308 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

In CE4 subtest 2 QP prediction method for more efficient delta-QP coding method is investigated.

Temporal QP prediction (2.4.b) provides best result in coding efficiency but implementation cost is concern and further study would be needed. As for spatial prediction 2.3.g (using intra prediction direction for QP prediction) provides best result for intra frames. For inter frames 2.3.f (using intra/inter mode for QP prediction) and 2.3.e (average of “Left” and “Above”) provides similar result and better among others.

Harmonization of proposals should be investigated in this subtest and this document provides result with combination 2.3.g + 2.3.e.

# Introduction

At the 5th JCTVC Mtg Sub-LCU-Level delta-QP coding has been adopted. As of QP prediction simple scheme similar to the one in AVC is adopted but methods for more efficiency QP prediction have been investigated in CE4 subtest2.

As for spatial prediction 2.3.g (using intra prediction direction for QP prediction) provides best result for intra frames. For inter frames 2.3.f (using intra/inter mode for QP prediction) and 2.3.e (average of “Left” and “Above”) provides similar result and better among others.

In this document combination of 2.3.g and 2.3.e has been studied.

# Algorithm Description

The following is the algorithm description of CE4: 2.3.g [5]:

For intra CU, QP of either top or left neighboring CU indicated by direction of intra prediction is employed as predictor.

The following is the algorithm description of CE4: 2.3.e [6]:

if (Left and Above) predQP=(Left + Above) / 2

else if (Left and Aboveleft) predQP=(Left + Aboveleft) / 2

else if (Above and Aboveleft) predQP=(Above + Aboveleft) / 2

else if (Left) predQP=Left

else if (Above) predQP=Above

else if (AboveLeft) predQP=Aboveleft

else predQP=prevQP

“To be tested” as described applies 2.3.g method for intra frames and 2.3.f method for inter frames.

# Simulation

## Simulation Condition

The followings are description of anchor and to be tested:

[Anchor]

HM-3.0-dev-r863-CE4_r2 (Distributed to CE4 participants as the anchor)

[To be tested]

The proposed method is implemented by the proponent of top of the anchor software.

Macro setting in “Typedef.h” is as follows:

#define REFERENCE_QP_TYPE                 241

#define SECONDARY_QP_TYPE                 235

Intra, Intra_LC, Low-delay, Low-delay_LC, Random-access, and Random-access_LC conditions have been tested as described in [1].

The value of MaxCuDepthDQP is set as 3 (8x8).

## Simulation Result

Summary of the result is shown in Table 3.2.1. Complete result is shown in JCTVC-F648.xls.

This result is verified by NEC [4].

Table 3.2.1

|  | Intra |  |  |  | Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.2 | -0.3 | -0.2 | -7.9% | -0.2 | -0.2 | -0.2 | -7.0% |
| Class B | -0.4 | -0.4 | -0.4 | -11.4% | -0.3 | -0.3 | -0.3 | -10.1% |
| Class C | -0.5 | -0.5 | -0.5 | -13.4% | -0.5 | -0.5 | -0.5 | -12.5% |
| Class D | -0.4 | -0.5 | -0.4 | -12.7% | -0.4 | -0.4 | -0.4 | -12.6% |
| Class E | -0.8 | -0.9 | -0.8 | -17.8% | -0.7 | -0.7 | -0.7 | -15.5% |
| All | -0.5 | -0.5 | -0.4 | -12.3% | -0.4 | -0.4 | -0.4 | -11.3% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 102% |  |  |  | 100% |  |  |  |
|  | Random access |  |  |  | Random access LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.2 | 0.0 | -0.3 | -4.3% | -0.1 | 0.0 | 0.0 | -3.0% |
| Class B | -0.2 | -0.1 | -0.2 | -5.9% | -0.2 | -0.2 | -0.2 | -5.0% |
| Class C | -0.3 | -0.3 | -0.3 | -9.4% | -0.4 | -0.4 | -0.3 | -9.7% |
| Class D | -0.2 | -0.1 | -0.2 | -7.8% | -0.3 | -0.5 | -0.3 | -8.5% |
| Class E |  |  |  |  |  |  |  |  |
| All | -0.2 | -0.1 | -0.2 | -6.8% | -0.2 | -0.3 | -0.2 | -6.5% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 98% |  |  |  |
|  | Low delay |  |  |  | Low delay LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.1 | -0.4 | -0.4 | -4.0% | -0.1 | -0.2 | -0.3 | -3.1% |
| Class C | -0.3 | -0.4 | -0.3 | -8.1% | -0.5 | -0.3 | -0.5 | -8.5% |
| Class D | -0.2 | -0.1 | 0.3 | -6.7% | -0.3 | -0.4 | -0.5 | -7.6% |
| Class E | -0.1 | 0.0 | 0.3 | -7.4% | -0.3 | -0.4 | 0.1 | -6.7% |
| All | -0.2 | -0.3 | -0.1 | -6.3% | -0.3 | -0.3 | -0.3 | -6.2% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 102% |  |  |  | 101% |  |  |  |

# Conclusion

In this document combination of 2.3.g &2.3.e as defined in CE4 subtest2 has been studied. Simulation result shows that gain is obtained by -0.5%, -0.4%, -0.2%, -0.2%, -0.2% and -0.3% with IT, IT_Loco, RA, RA_Loco, LD and LD_Loco conditions respectively.

Gain with 2.3.e is a little less than 2.3.f but complexity is lower, as 2.3.f requires intra/inter mode information for QP prediction but 2.3.e does not need to access such information. It is possible that 2.3.g and 2.3.f and 2.3.e be combined for more improvement in coding efficiency [7].

# References

[1] “WD3: Working Draft 3 of High-Efficiency Video Coding”, JCTVC-E603, March 2011

[2] “Common test conditions and software reference configurations”, JCTVC-E700, March 2011

[3] “Description of Core Experiment 4: Quantization”, JCTVC-E704, March 2011

[4] “CE4 Subtest 2: Cross-check report for JCTVC-F648 (test 2.3.g and 2.3.e.r1 combined)”, JCTVC-F644, July 2011

[5] “CE4 Subtest 2: Delta QP prediction results of test 2.2.b and 2.3.f”, JCTVC-F300, Jul 2011

[6] “CE4: Result of 2.3.d and 2.3.e”, JCTVC-F420, July 2011

[7] “CE4 Subtest 2: Spatial QP prediction: combination of test 2.3.g, 2.3.f and 2.3.e”, JCTVC-F661, July 2011