| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F654 |
| --- | --- |

| Title: | Cross verification for Qualcomm's proposal JCTVC-F552 on Parallel Processing of Residual Data in HE |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Hisao Sasai Takahiro Nishi 1006 Kadoma, Kadoma, Osaka, Japan | Tel: Email: | +81 6 6900 9689 sasai.hisao@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution is a report of cross-check result for Qualcomm's proposal in JCTVC-F552. The experimental results perfectly match with the one provided by Qualcomm in the rate-distortion performance. In addition to the cross-verification, additional possibility of parallel processing of residual data is observed. For coefficient levels in I-slice, no context update rather than context update delay shows better performance.

# Experimental Condition

The software to be tested was provided by Qualcomm and its performance relative to HMv3 software was checked under the common test condition described in JCTVC-E700 [1].
The computing platform is Linux 64bits and the executables were compiled on 64-bit Linux with gcc 4.4.3.

# Results

For the data in groups of 16 coefficients, BD BR and execution time is shown in Table 1.

Table 1. BD BR for Qualcomm's proposal (high efficiency)

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.47 | -0.52 | -0.57 | -0.68 | -1.04 | -1.21 |  |  |  |
| Class B | -0.20 | -0.59 | -0.60 | -0.13 | -0.80 | -0.66 | 0.03 | -1.48 | -1.55 |
| Class C | -0.21 | -0.42 | -0.43 | 0.00 | -0.66 | -0.58 | 0.03 | -1.44 | -1.58 |
| Class D | -0.18 | -0.50 | -0.48 | -0.04 | -0.69 | -0.91 | -0.17 | -2.27 | -2.00 |
| Class E | -0.19 | -0.91 | -0.78 |  |  |  | -0.11 | -2.43 | -3.17 |
| Overall | -0.25 | -0.57 | -0.56 | -0.21 | -0.79 | -0.83 | -0.05 | -1.84 | -1.97 |
| Enc Time[%] | 97% |  |  | 98% |  |  | 98% |  |  |
| Dec Time[%] | 97% |  |  | 98% |  |  | 98% |  |  |

For the delayed probability updated with k = 1, BD BR and execution time is shown in Table 2.

Table 2. BD BR for Qualcomm's proposal (high efficiency)

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.32 | -0.34 | -0.39 | -0.60 | -1.19 | -1.41 |  |  |  |
| Class B | -0.09 | -0.49 | -0.45 | -0.05 | -0.72 | -0.49 | 0.08 | -1.24 | -1.37 |
| Class C | -0.13 | -0.34 | -0.34 | 0.09 | -0.60 | -0.62 | 0.08 | -1.46 | -1.32 |
| Class D | -0.11 | -0.43 | -0.37 | 0.01 | -0.76 | -0.81 | -0.05 | -1.75 | -1.54 |
| Class E | -0.09 | -0.80 | -0.64 |  |  |  | -0.17 | -2.14 | -2.31 |
| Overall | -0.15 | -0.46 | -0.43 | -0.13 | -0.81 | -0.81 | 0.00 | -1.59 | -1.58 |
| Enc Time[%] | 98% |  |  | 98% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  | 99% |  |  |

In addition to the cross-verification, additional possibility of parallel processing of residual data is observed. For the delayed probability update with k = ∞ for coefficient level for I-slice and with k = 1 for significance, BD BR and execution time is shown in Table 3.

Table 3. BD BR for Qualcomm's proposal with no updating for coefficient level for I-slice (high efficiency)

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.45 | -0.35 | -0.43 | -0.57 | -0.99 | -1.62 |  |  |  |
| Class B | -0.25 | -0.59 | -0.61 | -0.09 | -0.57 | -0.43 | 0.06 | -1.26 | -1.49 |
| Class C | -0.28 | -0.56 | -0.55 | 0.05 | -0.41 | -0.57 | 0.04 | -1.23 | -1.33 |
| Class D | -0.31 | -0.73 | -0.63 | 0.01 | -0.64 | -1.05 | -0.06 | -2.23 | -1.54 |
| Class E | -0.28 | -0.91 | -0.86 |  |  |  | -0.19 | -2.63 | -1.89 |
| Overall | -0.32 | -0.61 | -0.60 | -0.15 | -0.65 | -0.89 | -0.02 | -1.75 | -1.54 |
| Enc Time[%] | 97% |  |  | 98% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  | 99% |  |  |

# Conclusion

The results provided by the proponent for Qualcomm's proposal on Parallel Processing of Residual Data in HE (JCTVC-F552) [2] were confirmed. In addition to the context update delay, no context updating has better performance in some cases. The possibility of no context updating should be investigated in CE/AHG.

# References

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, Geneva, March, 2011.

- J. Sole et. al., “Parallel processing of residual data in HE”, JCTVC-F552, Torino, July 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).