| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F657 M21101 |
| --- | --- |

| Title: | Cross verification of Samsung's proposal JCTVC-F587 on Reduction of reference picture list checking for temporal motion vector prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA | Tel: Email: | +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

The document reports cross-check results of Samsung’s proposal JCTVC-F587 on Reduction of reference picture list checking for temporal motion vector prediction. It has been verified that the results provided by Samsung in JCTVC-F587 are accurate.

# Test Settings and Conditions

The simulations of this document have used HM3.0 software, the common test conditions and reference configurations specified in [1]. The simulation platform at Texas Instruments is LSF equipped with Intel(R) Xeon(R) CPU X5570 64 bits Linux machines of different frequencies.

# Experimental Results

The proposed simplification disables the HM3.0 “crossing” check and works in a similar fashion as AVC, i.e. the co-located motion vectors of priority direction (i.e. list 0 or list 1) is first checked and if available used. Otherwise, the co-located motion vectors of other direction (i.e. list 1 or list 0) are further checked and used for temporal MVP derivation if available.

The test results are given in Table 1 for Simplified setting 1, Table 2 for Simplified setting 2, and Table 3 for Simplified setting 3.

- Simplified setting 1: the priority direction is defined as the one opposite to the direction specified by ColDir. For example, if the current PU uses co-located reference frames from list 0, then list 1 motion vectors of co-located PU are first checked. As shown in Table 1, there is an average loss of 0.3% in LB-HE and 0.4% loss in LB-LC by this simplification.

- Simplified setting 2: on top of simplified setting 1 an additional check is added. If it is low-delay coding the priority direction is defined as the prediction direction that is same as the current PU. Otherwise, simplified setting 1 applies. As shown Table 2, this additional low-delay coding check has eliminated loss reported for simplified setting 1.

- Simplified setting 3: same as simplified setting 2, only difference is that the low-delay coding flag is detected at sequence level in setting 3 instead of relying on signaling in the configuration file.

The results reported in this document perfectly match the results provided by Samsung in [2]. The anchor used here is HM3.0 with bug fix 146, the run time reported in the document may not be accurate.

|  | Random access |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.3 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 96% |  |  | 95% |  |  |
| Dec Time[%] | 91% |  |  | 89% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.2 | 0.4 | 0.6 | 0.6 | 0.4 | 0.2 |
| Class C | 0.2 | 0.3 | 0.2 | 0.3 | 0.2 | 0.0 |
| Class D | 0.4 | 0.4 | 0.8 | 0.3 | -0.1 | 0.2 |
| Class E | 0.3 | 0.3 | 0.2 | 0.6 | 0.8 | 0.2 |
| All | 0.3 | 0.3 | 0.5 | 0.4 | 0.3 | 0.1 |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |

Table 1: BD-rate comparison relative to HM3.0 anchor for simplified setting 1

|  | Random access |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.3 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 104% |  |  | 97% |  |  |
| Dec Time[%] | 107% |  |  | 95% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 99% |  |  | 104% |  |  |
| Dec Time[%] | 97% |  |  | 108% |  |  |

Table 2: BD-rate comparison relative to HM3.0 anchor for simplified setting 2

|  | Random access |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.3 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 104% |  |  | 97% |  |  |
| Dec Time[%] | 107% |  |  | 95% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 98% |  |  | 104% |  |  |
| Dec Time[%] | 99% |  |  | 108% |  |  |

Table 3: BD-rate comparison relative to HM3.0 anchor for simplified setting 3

# References

[1] F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-E700, Geneva, April 2011.

[2] I-K Kim, W.-J Han, JH Park, “Reduction of reference picture list checking for temporal motion vector prediction” JCTVC-F587, 6th Meeting: Torino, IT, 14-22 July, 2011