| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F663 |
| --- | --- |

| Title: | On cu_qp_delta range constraint |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Keiichi Chono Hirofumi Aoki 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 3757 5439 chono@ct.jp.nec.com hao@bu.jp.nec.com |
| Source: | [Information and Media Processing Labs, NEC Corporation] |  |  |

_____________________________

# Abstract

The current entropy coding method for cu_qp_delta syntax cannot map a signed syntax element value to the code number without redundancy when the absolute value of the minimum negative value differs substantially from that of the maximum positive value. However, new entropy coding methods presented in JCTVC-F174 and JCTVC-F46 enable to map a signed syntax element value to the code number without redundancy even when the absolute value of the minimum negative value differs substantially from that of the maximum positive value. If the new methods are integrated into the HM design, the current cu_qp_delta range constraint in the WD draft text, “–( 26+ QpBdOffsetY / 2 ) to +( 25+ QpBdOffsetY / 2 ), inclusive,” may not be appropriate. A straightforward cu_qp_delta range constraint, “– ( | 0 - QPY,PREV | ) to + (51 - QPY,PREV ), inclusive” would be appropriate. It is recommended that JCTVC experts review this contribution and discuss the appropriate cu_qp_delta range constraint in the HM design.

# Problem statement

The current WD draft text specifies that the decoded value of cu_qp_delta is in the range of –( 26+ QpBdOffsetY / 2 ) to +( 25+ QpBdOffsetY / 2 ), inclusive. It seems that the range constraint comes from the H.264/MPEG-4 AVC standard. Authors believe that one of the reasons why the H.264/MPEG-4 AVC standard has such a constraint rather than straightforward one as 0 ≤ predQP + cu_qp_delta ≤ 51 is that mapping from a signed syntax element value to the code number becomes redundant when the absolute value of the minimum negative value, | minDQP |, differs substantially from that of the maximum positive value, | maxDQO |.

# Solutions

JCTVC-F174 and JCTVC-F46 propose efficient entropy coding methods for cu_qp_delta syntax that can map a signed syntax element value to the code number without redundancy even when | minDQP | and | maxDQO | differ substantially.

JCTVC-F174 proposes signaling maximum and minimum QP values (MaxQP, MinQP) in a slice and using a new indexing method for possible cu_qp_delta values specified by MaxQP, MinQP, and predQP values. Specifically, the new indexing method assigns smaller code number to smaller absolute cu_qp_delta value. Therefore JCTVC-F174 can map a signed syntax element value to the code number without redundancy even when | minDQP | and | maxDQO | differ substantially. Figure1 illustrates the new indexing results when MaxQP=32, MinQP=24 and predQP=30 (minDQP and maxDQP are -6 and 2, respectively.).

Figure 1	Example of JCTVC-F175 cu_delta_qp indexing for (minDQP, maxDQP) = (-6, 2).

JCTVC-F046 proposes a new binary representation of cu_qp_delta syntax for CABAC based on a truncated unary binarization. The key idea is to signal the significance of cu_qp_delta value with the first bin; if the value is significant, then it signals its sign with the second bin, and its absolute value with the remaining bins. Placing the sign in the second bin allows decoders to infer the truncated bin values by the encoder even when| minDQP | and | maxDQP | are different. Therefore JCTVC-F046 can also map a signed syntax element value to the code number without redundancy even when | minDQP | and | maxDQO | differ substantially. Figure2 illustrates the new indexing results when MaxQP=32, MinQP=24 and predQP=30. (Appendix B shows the JCTVC-F046 extension to support any combinations of MinQP, MaxQP, and predQP.)

Figure 2	Example of JCTVC-F046 binary representation for (minDQP, maxDQP) = (-6, 2). Each of gray-colored cells contains sign bit. Yellow-colored cell indicates its bin is truncated.

# Conclusions

New entropy coding methods presented in JCTVC-F174 and JCTVC-F46 enable to map a signed syntax element value to the code number without redundancy even when the absolute value of the minimum negative value differs substantially from that of the maximum positive value. Therefore if the new methods are integrated into the HM design, such a range constraint may not be appropriate. A straightforward cu_qp_delta range constraint, “– ( | 0 - QPY,PREV | ) to + (51 - QPY,PREV ), inclusive” would be appropriate. It is recommended that JCTVC experts review this contribution and discuss the appropriate cu_qp_delta range constraint in the HM design.

# Reference

- K. Chono, H. Aoki, and Y. Senda, "Efficient binary representation of cu_qp_delta syntax for CABAC", JCTVC-F046, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011.

- K. Sugimoto and S. Sekiguchi, " Signaling of Max and Min QP in slice ", JCTVC-F174, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011.

# Patent rights declaration(s)

NEC Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix A: Proposed modification of HEVC Working Draft 3.

Replace the following text

“The decoded value of cu_qp_delta shall be in the range of –( 26+ QpBdOffsetY / 2 ) to +( 25+ QpBdOffsetY / 2 ), inclusive. cu_qp_delta shall be inferred to be equal to 0 when it is not present for any quantization group of coding units.

The value of QPY is derived as

QPY = ( ( ( QPY,PREV + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) - QpBdOffsetY (7-32)

where QPY,PREV is the luma quantization parameter,”

by

“The decoded value of cu_qp_delta shall be in the range of – ( | MinQP - QPY,PREV | ) to + (MaxQP - QPY,PREV ), inclusive. cu_qp_delta shall be inferred to be equal to 0 when it is not present for any quantization group of coding units.

The value of QPY is derived as

QPY = ( QPY,PREV + cu_qp_delta + QpBdOffsetY ) 	(7-32)

where MinQP and MaxQP are the minumum and maximum luma quantization parameters, QPY,PREV is the luma quantization parameter,”

Appendix B: Proposed modification of HEVC Working Draft 3(JCTVC-F046 extension

Replace the following text

“The decoded value of cu_qp_delta shall be in the range of –( 26+ QpBdOffsetY / 2 ) to +( 25+ QpBdOffsetY / 2 ), inclusive. cu_qp_delta shall be inferred to be equal to 0 when it is not present for any quantization group of coding units.

The value of QPY is derived as

QPY = ( ( ( QPY,PREV + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) - QpBdOffsetY (7-32)

where QPY,PREV is the luma quantization parameter,”

by

“The decoded value of cu_qp_delta shall be in the range of – ( | MinQP - QPY,PREV | ) to + (MaxQP - QPY,PREV ), inclusive. cu_qp_delta shall be inferred to be equal to 0 when it is not present for any quantization group of coding units.

The value of QPY is derived as

QPY = ( QPY,PREV + cu_qp_delta + QpBdOffsetY ) 	(7-32)

where MinQP and MaxQP are the minumum and maximum luma quantization parameters, QPY,PREV is the luma quantization parameter,”

Replace the subsection 9.3.2.7 Binarization process for cu_qp_delta by

“9.3.2.7 Binarization process for cu_qp_delta

Input to this process is a request for a binarization for the syntax element cu_qp_delta, a qp prediction value QPY,PREV.

Output of this process is the binarization of the syntax element.

The bin string of this binarization process of a syntax element value synVal is specified by a process equivalent to the following pseudo-code:

absV = Abs( synVal )

if( absV == 0){

put( 0 )

} else {

put( 1 )

signV = (synVal > 0 )? 0 : 1

put (signV )

if(signV){

cMax = QPY,PREV - MinQP - 1

}

else {

cMax = MaxQP - QPY,PREV - 1

}

cNum = absV – 1

if(cMax > 0){

absVGreaterThan1Flag = ( absV == 1 )? 0 : 1

put (absVGreaterThan1Flag )

if( absVGreaterThan1Flag ){

while(--cNum)

{

put( 1 )

}

if(cMax > absV - 1 ) {

put( 0 )

}

}

}

}”

The cu_qp_delta portions of Table 9-49 is modified as follow:

| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| cu_qp_delta | Error: Reference source not found | 0 | 0 | na | 2 | 3 | 3 |
|  |  | 4 | 0 | na | 2 | 3 | 3 |
|  |  | 8 | 0 | na | 2 | 3 | 3 |