| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F665 WG11 Number: m21118 |
| --- | --- |

| Title: | Verification results of MediaTek’s ALF with zero pixel line buffers JCTVC-F054 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage Takashi Watanabe | Email: | tomoo.yamakage@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

Verification results of experiment 2 of JCTVC-F054 (Adaptive Loop Filter with Zero Pixel Line Buffers for LCU-based Decoding) are reported.  Coding efficiency results are exactly matched with the proponent’s results except for low delay P case.  No mismatch between encoder and decoder is observed.  Relative encoding and decoding time are similar to those of proponent’s.  Source code was reviewed and it was confirmed that the code is consistent with the description in JCTVC-F054.

# Introduction

This contribution is a report of verification results of experiment 2 (Sec 3.1 and 3.2) of JCTVC-F054, which removes line buffers for ALF.  4 lines and 2 lines for luma and chroma, respectively, are considered as virtual boundary from the bottom line of an LCU.  When pixels used for filtering cross the virtual boundary, pixels above or below the virtual boundary are replaced with the padding pixels.  In addition, the shapes for luma filtering is 5x5, 7x7 and 9x9 diamond, and the shapes for chroma is 7x7 diamond.

Verification is performed under the High Efficiency test conditions in JCTVC-E700.  Coding efficiency results are exactly matched with the proponent’s results.  Relative encoding and decoding time are similar to those of proponent’s.

# Experimental Results

Detailed results in Excel sheet are attached.  The following table is the summary of experimental results:

| Test case | AI |  |  |  |  | RA |  |  |  |  |  | LDB |  |  |  |  |  |  | LDP |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y |  | U | V |  | Y | U |  | V |  |  | Y |  | U |  | V |  |  | Y |  | U |  | V |  |
| Class A |  | 0.2 | 0.0 |  | 0.0 |  | 0.1 |  | 0.0 |  | -0.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  | 0.1 | 0.0 |  | 0.0 |  | 0.0 |  | -0.3 |  | -0.3 |  |  | -0.4 |  | -0.8 |  | -0.7 |  |  | 0.0 |  | -0.7 |  | -1.2 |
| Class C |  | 0.0 | -0.1 |  | 0.1 |  | 0.0 |  | -0.4 |  | -0.3 |  |  | -0.3 |  | -0.9 |  | -0.7 |  |  | 0.0 |  | -0.7 |  | -0.7 |
| Class D |  | 0.0 | -0.1 |  | -0.2 |  | -0.3 |  | -0.5 |  | -0.7 |  |  | -0.9 |  | -0.9 |  | -0.5 |  |  | -0.3 |  | -0.5 |  | -1.0 |
| Class E |  | 0.2 | -0.3 |  | -0.4 |  |  |  |  |  |  |  |  | 0.3 |  | 0.2 |  | -0.1 |  |  | 0.5 |  | 0.6 |  | -0.3 |
| Average |  | 0.1 | -0.1 |  | -0.1 |  | -0.1 |  | -0.3 |  | -0.4 |  |  | -0.4 |  | -0.7 |  | -0.5 |  |  | 0.0 |  | -0.4 |  | -0.9 |
| Enc time |  | 101% |  |  |  |  | 100% |  |  |  |  |  |  | 100% |  |  |  |  |  |  | 101% |  |  |  |  |
| Dec time |  | 100% |  |  |  |  | 100% |  |  |  |  |  |  | 100% |  |  |  |  |  |  | 101% |  |  |  |  |

Note: The cross-checker did not receive results of low delay P case.

There is no mismatch between encoder and decoder.  Relative encoding and decoding time are similar to those of proponent’s.

# Conclusion

This contribution shows verification results of JCTVC-F054. Coding efficiency results are confirmed except for low delay P case.  No mismatch between encoder and decoder is observed.  Relative encoding and decoding time are also confirmed.  Source code was reviewed and it was confirmed that the code is consistent with the description in JCTVC-F054.

# Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).