| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC- F677 |
| --- | --- |

| Title: | Cross-check for Sony’s Proposal (F287) on Improvements on last nonzero position coding of 4x4 TU in CAVLC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Authors: | Zhi Zhou Shan Liu 2860 Junction Ave. San Jose, CA 95134, USA | Emails: | zhi.zhou@mediatek.com shan.liu@mediatek.com |
| Source: | MediaTek USA |  |  |

_____________________________

# Abstract

This document cross-checked Sony’s proposal JCTVC-F287 “Improvements on last nonzero position coding of 4x4 TU in CAVLC” [1]. The verification was done successfully and the BD-rates match those provided by Sony exactly. The code was studied and found to match the proposal.

# Introduction

Sony provided the HM3.0-based source code of their proposed algorithm. The macros are set as:

#define SONY_TRUNCATE                     0

#define SONY_LP_4x4_YUV                   1

Three LC configurations (AI_LC, RA_LC, and LD_LC) were tested for the provided code.

# Test Results

The BD-rates from our experimental results match those provided by Sony exactly. The encoding and decoding times are within the variability of our machines. The detailed results are listed in JCTVC-F677.xls.

|  | Intra LoCo |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class A | 0.0 | -1.3 |  | -1.2 |  |
| Class B | -0.1 | -1.3 |  | -1.7 |  |
| Class C | -0.2 | -1.1 |  | -1.3 |  |
| Class D | -0.2 | -1.4 |  | -1.2 |  |
| Class E | 0.0 | -3.9 |  | -3.1 |  |
| All | -0.1 | -1.7 |  | -1.6 |  |
| Enc Time[%] | 100% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |
|  |  | Random access LoCo |  |  |  |
|  | Y BD-rate | U BD-rate |  |  | V BD-rate |
| Class A | -0.1 | -1.8 | -1.0 |  |  |
| Class B | 0.0 | -1.3 | -1.6 |  |  |
| Class C | 0.1 | -0.9 | -1.3 |  |  |
| Class D | 0.0 | -1.7 | -1.1 |  |  |
| Class E |  |  |  |  |  |
| All | 0.0 | -1.4 | -1.3 |  |  |
| Enc Time[%] | 101% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |
|  | Low delay LoCo |  |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |  |  |
| Class A |  |  |  |  |  |
| Class B | 0.0 | -0.6 | -0.9 |  |  |
| Class C | 0.0 | -0.4 | -0.9 |  |  |
| Class D | 0.0 | -0.9 | -0.2 |  |  |
| Class E | 0.0 | -1.4 | -2.0 |  |  |
| All | 0.0 | -0.8 | -0.9 |  |  |
| Enc Time[%] | 100% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |

# Conclusion

This report confirms the results provided by Sony in JCTVC-F287.

# References

- Jun Xu, Ali Tabatabai, “Improvements on last nonzero position coding of 4x4 TU in CAVLC”, JCTVC-F287, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Torino, IT, 14-22 July, 2011.

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, March. 2011.

- Gisle Bjontegaard, “Improvements of the BD-PSNR model”, ITU-T Q.6/SG16 VCEG, VCEG-AI11, Berlin, Germany, July 2008.