| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F682 |
| --- | --- |

| Title: | Cross check for Picture Adaptive 1/8-pel Motion Compensation Method (JCTVC-471) by Samsung |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Elena Alshina #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | +82 10 30261305 elena_a.alshina@samsung.com |
| Source: | Samsung Electronics, Ltd |  |  |

_____________________________

# Abstract

Test results reported in Panasonic’s contribution JCTVC-471 are verified Performance results for both variations of algorithm match with data reported by Panasonic. The difference in encoding / decoding time is under noise level. It comes from different platforms used for testing by Panasonic and Samsung.

- Test description

We tested s/w provided by Panasonic with two different settings for proposed adaptive motion compensation method [1]. Test 1 uses (setting B) :

- 1/8_active_flag is set to 1

- num_ref_idx_l0_1/8pel_active is set to 0

- num_ref_idx_l1_1/8pel_active is set to 0.

We evaluate performance and running time for this test in both 6 inter test cases: RA-HE/LC, LD(B)-HE/LC, LD(P)-HE/LC. Results are shown in Table 1.

Table 1. Summary of Test 1 results.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1 | -0.1 | -0.2 | 0.0 | 0.1 | 0.0 |
| Class B | -0.2 | -0.1 | -0.1 | -0.2 | -0.1 | 0.0 |
| Class C | -0.4 | -0.1 | -0.2 | -0.5 | -0.3 | -0.4 |
| Class D | -0.9 | -0.5 | -0.4 | -1.1 | -0.6 | -0.6 |
| Class E |  |  |  |  |  |  |
| Overall | -0.4 | -0.2 | -0.2 | -0.4 | -0.2 | -0.2 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 104% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | 0.1 | -0.1 | -0.1 | -0.1 | -0.3 |
| Class C | -0.2 | -0.1 | -0.3 | -0.3 | 0.0 | -0.3 |
| Class D | -0.1 | 0.3 | 0.4 | -0.4 | -0.3 | -0.2 |
| Class E | 0.0 | 0.5 | -0.2 | 0.0 | -0.1 | -0.2 |
| Overall | -0.1 | 0.2 | 0.0 | -0.2 | -0.1 | -0.2 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.5 | -0.5 | -0.6 | -0.5 | -0.3 | -0.2 |
| Class C | -1.3 | -1.0 | -1.1 | -1.2 | -0.8 | -1.0 |
| Class D | -2.3 | -1.7 | -1.8 | -2.2 | -1.7 | -1.7 |
| Class E | -0.4 | 0.2 | -0.6 | -0.3 | -0.3 | -0.4 |
| Overall | -1.1 | -0.8 | -1.0 | -1.1 | -0.8 | -0.8 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

Test 2 uses (setting A):

- 1/8_active_flag is set to 1

- num_ref_idx_l0_1/8pel_active is set to 4

- num_ref_idx_l1_1/8pel_active is set to 0.

We evaluate performance and running time for this test in 2 test cases (GPB off): LD(P)-HE/LC. Results are shown in Table 2.

Table 2. Summary of Test 1 results.

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.5 | -1.1 | -1.5 | -1.2 | 0.2 | 0.2 |
| Class C | -2.6 | -1.5 | -1.6 | -2.2 | -1.5 | -1.5 |
| Class D | -4.3 | -2.7 | -2.7 | -4.0 | -2.3 | -2.6 |
| Class E | -1.7 | -0.8 | -1.8 | -0.6 | -0.6 | 0.3 |
| Overall | -2.5 | -1.5 | -1.9 | -2.0 | -1.0 | -0.9 |
| Enc Time[%] | 121% |  |  | 130% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |

- Conclusion

- Results reported by Panasonic in JCTVC-F471 are verified. Minor difference in encoding/decoding time is at reasonable range.