| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F684 |
| --- | --- |

| Title: | CE6.b: Additional simulation results of RQT and SDIP harmonization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Keiichi Chono (NEC) Krit Panusopone (Motorola Mobility) | Email: | chono@ct.jp.nec.com krit@motorola.com |
| Source: | Motorola Mobility and NEC |  |  |

_____________________________

# Abstract

This contribution provides additional simulation results of the two RQT and SDIP harmonization schemes tested in CE6.b Test1 [1]: One scheme (which is referred as Case2 in CE6.b Test1) allows the maximum RQT depth value 2 for rectangular PUs (16x4, 4x16, 32x8, and 8x32 PUs); another one (which is referred as Case1 in CE6.b Test1) restricts the maximum RQT depth value to 1 for 32x8 and 8x32 PUs.

Simulation results of Case1 against w/o RQT and SDIP harmonization for RQT depth values 1, 2, and 3 show that:

- For RQT depth=1 case, BD-rate improvements of -0.04% and -0.42% are attained for intra high efficiency and intra low complexity, respectively;

- For RQT depth=2 case, BD-rate improvements of -0.03% and -0.24% are attained for intra high efficiency and intra low complexity, respectively;

- For RQT depth=3 case, BD-rate improvements of -0.03% and -0.24% are attained for intra high efficiency and intra low complexity, respectively.

Simulation results of Case2 against w/o RQT and SDIP harmonization for RQT depth values 1, 2, and 3 are as follows:

- For RQT depth=1 case, BD-rate improvements of -0.04% and -0.42% are attained for intra high efficiency and intra low complexity, respectively;

- For RQT depth=2 case, BD-rate improvements of -0.16% and -0.43% are attained for intra high efficiency and intra low complexity, respectively;

- For RQT depth=3 case, BD-rate improvements of -0.16% and -0.43% are attained for intra high efficiency and intra low complexity, respectively.

It was confirmed that both SDIP and RQT harmonization offer the consistent BD-rate improvements for all RQT depth values.

- Test condition and results

Simulations are conducted based on CE6.b common test conditions. Simulations for different RQT depth values were conducted by Motorola Mobility and NEC by using HM-SDIP software. The results were cross-checked by Hisilicon Technologies.

Tables below present summaries of BD-Rates, and encoder/decoder run-times in comparison with CE6.b Anchors with the same RQT depth value. (Note that the RQT depth=1 results of Case1 and Case2 SDIP are the same, that is, BD-rate values in Table 3 and Table 6 are identical.) Detailed results are found in the attached XLS file.

Table1: Summary results of Case1 RQT depth=3 results (Case1 Harmonization vs. No-Harmonization)

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.02 | 0.11 | 0.21 | -0.17 | -0.66 | -0.50 |
| Class B | -0.02 | 0.20 | 0.23 | -0.22 | -0.55 | -0.54 |
| Class C | -0.03 | 0.22 | 0.24 | -0.26 | -0.49 | -0.47 |
| Class D | -0.03 | 0.23 | 0.18 | -0.23 | -0.36 | -0.33 |
| Class E | -0.03 | 0.25 | 0.25 | -0.31 | -1.09 | -1.10 |
| All | -0.03 | 0.20 | 0.22 | -0.24 | -0.61 | -0.56 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table2: Summary results of Case1 RQT depth=2 results (Case1 Harmonization vs. No-Harmonization)

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.01 | 0.06 | 0.08 | -0.19 | -0.69 | -0.54 |
| Class B | -0.03 | 0.20 | 0.22 | -0.24 | -0.60 | -0.53 |
| Class C | -0.03 | 0.21 | 0.24 | -0.26 | -0.47 | -0.47 |
| Class D | -0.02 | 0.22 | 0.22 | -0.23 | -0.35 | -0.36 |
| Class E | -0.04 | 0.34 | 0.34 | -0.32 | -1.15 | -1.09 |
| All | -0.03 | 0.20 | 0.22 | -0.24 | -0.63 | -0.57 |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table3: Summary results of Case1 RQT depth=1 results (Case1 Harmonization vs. No-Harmonization)

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.02 | 0.07 | 0.29 | -0.29 | -0.80 | -0.76 |
| Class B | -0.03 | 0.16 | 0.22 | -0.46 | -0.83 | -0.91 |
| Class C | -0.05 | 0.22 | 0.24 | -0.44 | -0.72 | -0.74 |
| Class D | -0.05 | 0.18 | 0.20 | -0.35 | -0.51 | -0.49 |
| Class E | -0.04 | 0.29 | 0.27 | -0.59 | -1.75 | -1.58 |
| All | -0.04 | 0.18 | 0.24 | -0.42 | -0.88 | -0.86 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

Table4: Summary results of Case2 RQT depth=3 results (Case1 Harmonization vs. No-Harmonization)

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.03 | -0.08 | -0.04 | -0.16 | -0.62 | -0.46 |
| Class B | -0.21 | -0.22 | -0.22 | -0.57 | -0.66 | -0.63 |
| Class C | -0.06 | 0.12 | 0.12 | -0.32 | -0.51 | -0.50 |
| Class D | -0.09 | 0.14 | 0.12 | -0.31 | -0.37 | -0.34 |
| Class E | -0.47 | -0.46 | -0.42 | -0.86 | -1.33 | -1.14 |
| All | -0.16 | -0.08 | -0.04 | -0.43 | -0.66 | -0.59 |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

Table5: Summary results of Case2 RQT depth=2 results (Case2 Harmonization vs. No-Harmonization)

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.03 | -0.02 | 0.05 | -0.18 | -0.65 | -0.52 |
| Class B | -0.22 | -0.15 | -0.15 | -0.56 | -0.63 | -0.61 |
| Class C | -0.06 | 0.11 | 0.14 | -0.32 | -0.48 | -0.50 |
| Class D | -0.08 | 0.13 | 0.13 | -0.31 | -0.37 | -0.38 |
| Class E | -0.47 | -0.37 | -0.31 | -0.85 | -1.35 | -1.18 |
| All | -0.16 | -0.05 | -0.02 | -0.43 | -0.66 | -0.61 |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

Table6: Summary results of Case2 RQT depth=1 results (Case2 Harmonization vs. No-Harmonization)

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.02 | 0.07 | 0.29 | -0.29 | -0.80 | -0.76 |
| Class B | -0.03 | 0.16 | 0.22 | -0.46 | -0.83 | -0.91 |
| Class C | -0.05 | 0.22 | 0.24 | -0.44 | -0.72 | -0.74 |
| Class D | -0.05 | 0.18 | 0.20 | -0.35 | -0.51 | -0.49 |
| Class E | -0.04 | 0.29 | 0.27 | -0.59 | -1.75 | -1.58 |
| All | -0.04 | 0.18 | 0.24 | -0.42 | -0.88 | -0.86 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

- Reference

[1] A. Tabatabai et al. "Core Experiment 6: Intra Prediction Improvement," JCTVC-E706, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, Switzerland, 16-23 March, 2011.