| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F686 |
| --- | --- |

| Title: | Cross-check report on JCTVC-F479: Chroma DC offset for intra coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Keiichi Chono Hirofumi Aoki 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 3757 5439 chono@ct.jp.nec.com hao@bu.jp.nec.com |
| Source: | [Information and Media Processing Labs, NEC Corporation] |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results on JCTVC-F479: Chroma DC offset for intra coding described in JCTVC-F479. Cross-verification task has been successfully completed, and it is confirmed that for both CU and LCU based Chroma DC offset compensations, the BD-rate results match those provided by proponent.

# Simulation results

Encoding and decoding simulation was run on Windows 7 64-bit computers. Tables below present summary results of BD-Rate and encoder/decoder run-times in comparison with HM-3.0 anchors. Detailed results are found in the attached XLS files.

Table1	Summary results of LCU-based Chroma DC offset compensation

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1 | -2.6 | -4.1 | 0.2 | -1.3 | -2.1 |
| Class B | 0.1 | -1.9 | -3.4 | 0.2 | -1.0 | -1.7 |
| Class C | 0.1 | -1.4 | -1.7 | 0.1 | -0.8 | -1.1 |
| Class D | 0.1 | -0.8 | -1.1 | 0.1 | -0.7 | -1.0 |
| Class E | 0.2 | -6.2 | -4.0 | 0.3 | -3.4 | -2.4 |
| Overall | 0.1 | -2.4 | -2.8 | 0.2 | -1.3 | -1.6 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.2 | -5.3 | -6.7 | 0.3 | -2.8 | -3.4 |
| Class B | 0.2 | -2.2 | -3.1 | 0.2 | -1.2 | -1.7 |
| Class C | 0.2 | -1.0 | -1.6 | 0.2 | -0.9 | -1.1 |
| Class D | 0.2 | -1.4 | -1.7 | 0.2 | -1.2 | -1.2 |
| Class E |  |  |  |  |  |  |
| Overall | 0.2 | -2.4 | -3.3 | 0.2 | -1.5 | -1.8 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2 | -1.5 | -3.2 | 0.2 | -1.3 | -2.3 |
| Class C | 0.2 | -0.8 | -1.2 | 0.2 | -0.6 | -1.0 |
| Class D | 0.1 | -1.2 | -1.2 | 0.1 | -1.0 | -1.0 |
| Class E | 0.2 | -3.3 | -3.3 | 0.1 | -1.5 | -2.0 |
| Overall | 0.2 | -1.6 | -2.2 | 0.2 | -1.1 | -1.6 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |

Table2	Summary results of CU-based Chroma DC offset compensation

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 3.1 | -9.4 | -12.3 | 2.7 | -7.1 | -9.2 |
| Class B | 3.8 | -9.5 | -13.8 | 3.6 | -8.1 | -11.4 |
| Class C | 4.3 | -8.1 | -10.4 | 4.3 | -7.5 | -9.6 |
| Class D | 3.7 | -7.8 | -8.7 | 3.8 | -8.5 | -9.1 |
| Class E | 6.1 | -23.9 | -18.0 | 5.4 | -22.7 | -16.5 |
| Overall | 4.1 | -11.0 | -12.4 | 3.9 | -10.1 | -10.9 |
| Enc Time[%] | 105% |  |  | 104% |  |  |
| Dec Time[%] | 104% |  |  | 104% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 1.4 | -13.8 | -16.9 | 1.4 | -9.4 | -12.2 |
| Class B | 1.8 | -11.3 | -15.9 | 1.8 | -9.4 | -12.8 |
| Class C | 1.9 | -7.6 | -9.9 | 2.0 | -7.1 | -9.0 |
| Class D | 1.5 | -9.1 | -10.3 | 1.6 | -9.2 | -9.8 |
| Class E |  |  |  |  |  |  |
| Overall | 1.7 | -10.5 | -13.4 | 1.7 | -8.8 | -11.1 |
| Enc Time[%] | 104% |  |  | 105% |  |  |
| Dec Time[%] | 104% |  |  | 104% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.8 | -3.2 | -6.1 | 0.9 | -2.1 | -3.3 |
| Class C | 0.9 | -1.6 | -2.3 | 1.1 | -0.9 | -1.6 |
| Class D | 0.6 | -3.6 | -2.8 | 0.8 | -1.4 | -1.0 |
| Class E | 0.6 | -14.1 | -13.2 | 0.6 | -13.0 | -10.8 |
| Overall | 0.7 | -5.0 | -5.7 | 0.9 | -3.7 | -3.7 |
| Enc Time[%] | 105% |  |  | 105% |  |  |
| Dec Time[%] | 104% |  |  | 103% |  |  |