| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F687_r2 |
| --- | --- |

| Title: | CE2: Report on the Combination of OBMC with Motion Merging and Non-Square Quadtree Transform |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |
| Purpose: | Report/Proposal |  |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen1, Yan-Yu Chen, Chung-Lin Lee, Wen-Hsiao Peng2 and Hsueh-Ming Hang 1001 Ta-Hsueh Rd., Hsinchu 30010, Taiwan | Tel: Email: | +886-955-028-902 1cheerchen.cs98g@g2.nctu.edu.tw 2pawn@mail.si2lab.org |  |
| Source: | National Chiao Tung University (NCTU)/ITRI International |  |  |  |

_____________________________

# Abstract

In the context of Core Experiment 2 (JCTVC-E702), this contribution provides test results on the combination of OBMC with Motion Merging (JCTVC-F049) and Non-Square Quadtree Transform (NSQT, JCTVC-F410/JCTVC-F412). When integrated in HM3.0, the combined technique was observed to achieve Y-BD-rate savings of 1.7% for RA-HE, 1.7% for RA-LC, 2.7% for LB-HE, and 3.8% for LB-LC. It performed consistently better in all test cases, with an average of 2.5%, a minimum of 0.9% and a maximum of 7.6%. The highest gain was achieved in Vidyo3, Class E under the LB-LC condition. On average, the encoding time was increased by 30% and the decoding time by 3%.

# Algorithm Description

## OBMC with Motion Merging

In JCTVC-F049 [1], we introduced the technique of OBMC with Motion Merging. Conceptually, it performs OBMC for a 2Nx2N prediction unit (PU) based on two sets of motion parameters, one (Set #1) is derived from motion merging with a reduced candidate set containing only Left, Above and Col merging candidates and the other (Set #2) can be signaled either by motion merging or by explicit coding. Two predictors are created from these motion parameters and combined linearly using an adaptive OBMC window function to form a prediction block. According to the choice of the merging candidate specified in the parameters of Set #1, the encoder can choose between the following window functions (See Figure 1). The details can be found in JCTVC-F049 [1].

## Non-Square Quadtree Transform (NSQT)

The concept of Non-Square Quadtree Transform (NSQT) was introduced in JCTVC-F410 and JCTVC-F412 [2][3] as an alternative to Residual Quadtree Transform (RQT) for symmetric (2NxN/Nx2N) and asymmetric motion partitions (AMP) [4]. With NSQT, the 2Nx2N TU remains at the first level of a residual quadtree, but TUs of size 2Mx0.5M or 0.5Mx2M are used in place of the square transforms at the second and third levels (See Figure 2). In JCTVC-F410 [2], NSQT of type 2Mx0.5M (respectively, 0.5Mx2M) is applied when the PU partition type is 2NxN (respectively, Nx2N), while in JCTVC-F412 [3], it is additionally enabled for asymmetric partitions.

| w1,1 | w1,2 | w1,3 | w1,4 |
| --- | --- | --- | --- |
| w2,1 | w2,2 | w2,3 | w2,4 |
| A | B | Col | A, B are identical |

- Mapping between the selection of window functions, wn,m, and the choice of the merging candidate specified in parameters of Set #1.

| RQT Level 1 | RQT Level 2 | RQT Level 3 |
| --- | --- | --- |

- Examples of non-square TU split at different RQT levels.

## Combining OBMC with Motion Merging and NSQT

This contribution combines OBMC with Motion Merging and NSQT. When a 2Nx2N PU is coded by OBMC with Motion Merging, its transform type is determined as follows:

- NSQT of type 0.5Mx2M is applied if w1,1 or w2,1 is selected.

- NSQT of type 2Mx0.5M is applied if w1,2 or w2,2 is selected.

- RQT is applied if w1,3 or w2,3 is selected.

- RQT is applied if w1,4 or w2,4 is selected.

Additionally, NSQT is enabled for symmetric partitions (2NxN/Nx2N), as in JCTVC-F410 [2].

# Results

The combined technique was implemented in HM3.0 and evaluated in common test conditions [5]. The platform used is summarized below:

OS: 64-bit Microsoft Windows 7 Enterprise + Microsoft Visual Studio 2008 (32-bit Compiler)

Platform: Intel(R) Core(TM) i7-860 (2.80GHz) processor and 16GB RAMs

- BD-rate savings and coding time ratios.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.4 | -2.8 | -3.0 | -1.5 | -1.7 | -1.8 |
| Class B | -1.4 | -3.4 | -3.2 | -1.6 | -2.9 | -2.8 |
| Class C | -2.0 | -3.2 | -3.3 | -1.9 | -2.8 | -2.7 |
| Class D | -2.0 | -2.8 | -2.7 | -1.9 | -2.2 | -2.2 |
| Class E |  |  |  |  |  |  |
| All | -1.7 | -3.1 | -3.0 | -1.7 | -2.4 | -2.4 |
| Enc Time[%] | 128% |  |  | 128% |  |  |
| Dec Time[%] | 102% |  |  | 105% |  |  |
|  | Low Delay B HE |  |  | Low Delay B LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.9 | -5.4 | -5.4 | -3.2 | -5.0 | -5.1 |
| Class C | -2.8 | -4.8 | -4.7 | -3.4 | -4.2 | -4.2 |
| Class D | -3.0 | -4.2 | -4.2 | -3.5 | -3.3 | -3.7 |
| Class E | -3.7 | -4.9 | -5.8 | -5.8 | -5.9 | -6.6 |
| All | -2.7 | -4.9 | -5.0 | -3.8 | -4.6 | -4.8 |
| Enc Time[%] | 132% |  |  | 132% |  |  |
| Dec Time[%] | 102% |  |  | 104% |  |  |

# Conclusion

In view of the results obtained in these experiments, it is recommended to adopt this combined technique into the next release of HM and Working Draft.

# Reference

- C.-C. Chen, Y.-Y. Chen, C.-L. Lee, W.-H. Peng and H.-M. Hang, “CE2: Report of OBMC with Motion Merging,” JCTVC-F049, Torino, July 2011.

- Y. Yuan, X. Zheng, X. Peng, J. Xu, L. Liu, Y. Wang, X. Cao, C. Lai, J. Zheng, Y. He, and H. Yu, “CE2: Non-Square Quadtree Transform for symmetric motion partitions,” JCTVC-F410, Torino, July 2011.

- Y. Yuan, X. Zheng, X. Peng, J. Xu, I-K Kim, L. Liu, Y. Wang, X. Cao, C. Lai, J. Zheng, Y. He, and H. Yu, “CE2: Non-Square Quadtree Transform for symmetric and asymmetric motion partitions,” JCTVC-F412, Torino, July 2011.

- I.-K. Kim, W.-J. Han, J. Park and X. Zheng, “CE2: Test results of asymmetric motion partition (AMP),” JCTVC-F379, Torino, July 2011.

- F. Bossen, “Common test conditions and software reference configurations,” JCTVC-E700, Geneva, Mar. 2011.

# Appendix

## Random Access, High Efficiency

|  | Sequences | BD-rate Y (%) | BD-rate Y (%) | BD-rate Y (%) | EncTime | DecTime (w/ yuv out) | DecTime (w/o yuv out) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class A | Traffic | -1.2 | -2.5 | -2.5 |  |  |  |
|  | PeopleOnStreet | -1.7 | -3.8 | -3.3 |  |  |  |
|  | Nebuta | -1.3 | -1.9 | -3.1 |  |  |  |
|  | SteamLocomotive | -1.3 | -2.9 | -2.9 |  |  |  |
| Class B | Kimono | -0.9 | -1.8 | -1.6 |  |  |  |
|  | ParkScene | -1.2 | -1.6 | -1.7 |  |  |  |
|  | Cactus | -1.6 | -3.0 | -3.0 |  |  |  |
|  | BasketballDrive | -2.0 | -7.1 | -5.8 |  |  |  |
|  | BQTerrace | -1.4 | -3.6 | -3.6 |  |  |  |
| Class C | BasketballDrill | -1.9 | -3.1 | -3.2 |  |  |  |
|  | BQMall | -2.5 | -4.0 | -3.8 |  |  |  |
|  | PartyScene | -1.9 | -3.0 | -2.8 |  |  |  |
|  | RaceHorses | -1.7 | -2.8 | -3.2 |  |  |  |
| Class D | BasketballPass | -1.7 | -3.5 | -3.1 |  |  |  |
|  | BQSquare | -2.2 | -2.2 | -2.1 |  |  |  |
|  | BlowingBubbles | -2.2 | -2.8 | -2.9 |  |  |  |
|  | RaceHorses | -1.7 | -2.8 | -2.9 |  |  |  |
| Average of Class A |  | -1.4 | -2.8 | -3.0 |  |  |  |
| Average of Class B |  | -1.4 | -3.4 | -3.2 |  |  |  |
| Average of Class C |  | -2.0 | -3.2 | -3.3 |  |  |  |
| Average of Class D |  | -2.0 | -2.8 | -2.7 |  |  |  |
| Overall Average |  | -1.7 | -3.1 | -3.0 | 128% | 103% | 102% |

## Low Delay B, High Efficiency

|  | Sequences | BD-rate Y (%) | BD-rate Y (%) | BD-rate Y (%) | EncTime | DecTime (w/ yuv out) | DecTime (w/o yuv out) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class B | Kimono | -1.1 | -1.6 | -2.0 |  |  |  |
|  | ParkScene | -1.7 | -2.4 | -3.2 |  |  |  |
|  | Cactus | -2.3 | -5.3 | -5.7 |  |  |  |
|  | BasketballDrive | -3.1 | -10.3 | -9.0 |  |  |  |
|  | BQTerrace | -1.5 | -7.5 | -6.8 |  |  |  |
| Class C | BasketballDrill | -2.8 | -4.9 | -4.6 |  |  |  |
|  | BQMall | -3.7 | -6.6 | -5.9 |  |  |  |
|  | PartyScene | -2.9 | -5.1 | -5.2 |  |  |  |
|  | RaceHorses | -1.7 | -2.6 | -2.9 |  |  |  |
| Class D | BasketballPass | -2.9 | -4.6 | -4.2 |  |  |  |
|  | BQSquare | -3.6 | -4.5 | -4.0 |  |  |  |
|  | BlowingBubbles | -3.3 | -4.6 | -5.2 |  |  |  |
|  | RaceHorses | -2.2 | -3.0 | -3.3 |  |  |  |
| Class E | Vidyo1 | -2.9 | -3.4 | -4.0 |  |  |  |
|  | Vidyo3 | -4.7 | -5.6 | -6.6 |  |  |  |
|  | Vidyo4 | -3.4 | -5.6 | -6.8 |  |  |  |
| Average of Class B |  | -1.9 | -5.4 | -5.4 |  |  |  |
| Average of Class C |  | -2.8 | -4.8 | -4.7 |  |  |  |
| Average of Class D |  | -3.0 | -4.2 | -4.2 |  |  |  |
| Average of Class E |  | -3.7 | -4.9 | -5.8 |  |  |  |
| Overall Average |  | -2.7 | -4.9 | -5.0 | 132% | 102% | 102% |

## Random Access, Low Complexity

|  | Sequences | BD-rate Y (%) | BD-rate Y (%) | BD-rate Y (%) | EncTime | DecTime (w/ yuv out) | DecTime (w/o yuv out) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class A | Traffic | -1.3 | -2.2 | -2.6 |  |  |  |
|  | PeopleOnStreet | -1.3 | -2.7 | -2.5 |  |  |  |
|  | Nebuta | -1.3 | 1.3 | -0.2 |  |  |  |
|  | SteamLocomotive | -1.9 | -3.3 | -2.2 |  |  |  |
| Class B | Kimono | -1.0 | -1.5 | -1.7 |  |  |  |
|  | ParkScene | -1.3 | -1.0 | -1.3 |  |  |  |
|  | Cactus | -1.6 | -2.4 | -2.4 |  |  |  |
|  | BasketballDrive | -2.7 | -7.0 | -6.0 |  |  |  |
|  | BQTerrace | -1.7 | -2.5 | -2.6 |  |  |  |
| Class C | BasketballDrill | -1.7 | -2.9 | -3.0 |  |  |  |
|  | BQMall | -2.6 | -3.5 | -3.5 |  |  |  |
|  | PartyScene | -2.1 | -2.5 | -2.1 |  |  |  |
|  | RaceHorses | -1.2 | -2.2 | -2.3 |  |  |  |
| Class D | BasketballPass | -1.5 | -2.3 | -2.4 |  |  |  |
|  | BQSquare | -2.4 | -1.6 | -1.5 |  |  |  |
|  | BlowingBubbles | -2.4 | -2.5 | -2.6 |  |  |  |
|  | RaceHorses | -1.2 | -2.2 | -2.2 |  |  |  |
| Average of Class A |  | -1.5 | -1.7 | -1.8 |  |  |  |
| Average of Class B |  | -1.6 | -2.9 | -2.8 |  |  |  |
| Average of Class C |  | -1.9 | -2.8 | -2.7 |  |  |  |
| Average of Class D |  | -1.9 | -2.2 | -2.2 |  |  |  |
| Overall Average |  | -1.7 | -2.4 | -2.4 | 128% | 104% | 105% |

## Low Delay B, Low Complexity

|  | Sequences | BD-rate Y (%) | BD-rate Y (%) | BD-rate Y (%) | EncTime | DecTime (w/ yuv out) | DecTime (w/o yuv out) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class B | Kimono | -2.2 | -2.9 | -2.5 |  |  |  |
|  | ParkScene | -2.5 | -1.1 | -2.4 |  |  |  |
|  | Cactus | -2.9 | -3.8 | -4.2 |  |  |  |
|  | BasketballDrive | -5.6 | -11.2 | -10.0 |  |  |  |
|  | BQTerrace | -2.6 | -5.9 | -6.6 |  |  |  |
| Class C | BasketballDrill | -3.2 | -4.0 | -4.4 |  |  |  |
|  | BQMall | -4.8 | -6.4 | -5.7 |  |  |  |
|  | PartyScene | -3.7 | -4.1 | -4.2 |  |  |  |
|  | RaceHorses | -2.0 | -2.3 | -2.3 |  |  |  |
| Class D | BasketballPass | -3.7 | -4.1 | -3.7 |  |  |  |
|  | BQSquare | -4.3 | -4.0 | -4.9 |  |  |  |
|  | BlowingBubbles | -4.0 | -3.0 | -4.0 |  |  |  |
|  | RaceHorses | -2.0 | -2.2 | -2.1 |  |  |  |
| Class E | Vidyo1 | -4.7 | -3.8 | -3.8 |  |  |  |
|  | Vidyo3 | -7.6 | -7.6 | -9.4 |  |  |  |
|  | Vidyo4 | -5.0 | -6.4 | -6.8 |  |  |  |
| Average of Class B |  | -3.2 | -5.0 | -5.1 |  |  |  |
| Average of Class C |  | -3.4 | -4.2 | -4.2 |  |  |  |
| Average of Class D |  | -3.5 | -3.3 | -3.7 |  |  |  |
| Average of Class E |  | -5.8 | -5.9 | -6.6 |  |  |  |
| Overall Average |  | -3.8 | -4.6 | -4.8 | 132% | 104% | 104% |

# Patent rights declaration(s)

NCTU may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

2M=1<<Log2TrSize.