| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F694 m21177 |
| --- | --- |

| Title: | HM3.2 fine granularity slice implementation issues to be clarified |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akira Osamoto | Tel: Email: | +1-214-450-4969 osamoto@ti.com |
| Source: | Texas Instruments Inc |  |  |

_____________________________

# Abstract

# It turns out that there are several issues in the current HM3.2 fine granularity slice implementation which should be clarified. This contribution reports the current implementation and the suggested changes.

# Introduction

HM3.2 introduced the fine granularity slice. We found the several issues in its implementation. This contribution reports the suggested changes regarding:

- Availability derivation of neighboring pixels for intra prediction with fine granularity slice.

- Sub-LCU level Qp prediction with fine granularity slice.

- Deblocking filtering with fine granularity slice when filtering across slice boundaries is disabled.

# Problem statement

## Availability derivation of neighboring pixels for intra prediction with fine granularity slice

For the neighboring pixel for intra prediction, HM3.2 checks its availability by examining only the top-most (for left and bottom-left neighbors) or the left most (for upper and upper-right neighbors) when constrained intra prediction is disabled. This causes some of truly available neighbors marked as “unavailable”.

Fig. 1 depicts an example of this issue. For the upper neighbors for the current PU (“Cur PU”), HM3.2 checks the left-most pixel’s (the gray square in the figure) availability for all of the upper neighboring pixels. Since it belongs to a different slice, all of them are marked as “unavailable for intra prediction”, although some of right pixels should be available because they belong to the same slice.

Fig. 1: Example of HM3.2 availability derivation of neighboring pixels for intra prediction with fine slice granularity slice

### Suggested change in HM3.2 software

In TComPatten.cpp, function initAdiPattern(), line 248, add the following text in red.

if ( pcCU->getSlice()->getPPS()->getConstrainedIntraPred() ¦¦ pcCU->getSlice()->getPPS()->getSliceGranularity () > 0) { // … checking availability for each minimum CU size } else {

Similarly, in TComPatten.cpp, function initAdiChromaPattern(), line 511, add the following text in red.

if ( pcCU->getSlice()->getPPS()->getConstrainedIntraPred() ¦¦ pcCU->getSlice()->getPPS()->getSliceGranularity () > 0) { // … checking availability for each minimum CU size } else {

### Suggested change in WD3

No change is needed.

## Sub-LCU level Qp prediction with fine granularity slice

HM3.0 introduced the sub-LCU level dQp, and Qp prediction is taken from the left neighboring block if it is available. HM3.2 checks the left block’s availability with the top-left corner of the left “Qp minimum CU block”.

Fig. 2 depicts an example of the HM3.2 issue about Qp prediction. Since the top-left corner of the left Qp block belongs to a different slice, it is considered to be “not available”. We can check the bottom-right corner of the left block (the red block in the figure), instead, to know the actual availability. The bottom-right corner is chosen in order to maximize the left neighbor’s availability.

Fig. 2: Example of HM3.2 Qp prediction derivation with fine granularity slice

### Suggested change in HM3.2 software

In TComDataCU.cpp, function getQpMinCuLeft():

Line 1871

Replace

uiLPartUnitIdx = g_auiRasterToZscan[ uiAbsRorderQpMinCUIdx – uiNumPartInQpMinCUWidth ];

with

uiLPartUnitIdx = g_auiRasterToZscan[ uiAbsRorderQpMinCUIdx + uiNumPartInCUWidth * (uiNumPartInQpMinCUWidth – 1) – 1 ];

Line 1883:

Replace

uiLPartUnitIdx = g_auiRasterToZscan[ uiAbsRorderQpMinCUIdx + uiNumPartInCUWidth – uiNumPartInQpMinCUWidth ];

with

uiLPartUnitIdx = g_auiRasterToZscan[ uiAbsRorderQpMinCUIdx + uiNumPartInCUWidth * uiNumPartInQpMinCUWidth – 1 ];

### Suggested change in WD3

In 7.4.9 Transform coefficient semantics,

cu_qp_delta can change the value of QPY for a quantization group of coding units where the quantization group of coding units is specified as follows:

- If a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 0 and the log2CUSize is greater than or equal to log2MinCUDQPSize, the quantization group includes this coding unit only.

- Otherwise, if a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 1 and the log2CUSize is equal to log2MinCUDQPSize, the quantization group includes all coding units split from this coding unit.

The decoded value of cu_qp_delta shall be in the range of –( 26+ QpBdOffsetY / 2 ) to +( 25+ QpBdOffsetY / 2 ), inclusive. cu_qp_delta shall be inferred to be equal to 0 when it is not present for any quantization group of coding units.

The value of QPY is derived as

QPY = ( ( ( QPY,PREV + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) - QpBdOffsetY	(2-1)

where QPY,PREV is the luma quantization parameter, QPY, of last CU in coding order of the left neighbor quantization group of coding units in the current slice. If the left neighbor quantization group in the current slice is not available, QPY,PREV is the luma quantization parameter, of the previous quantization group in decoding order in the current slice. For the first quantization group of coding units in the slice QPY,PREV is initially set equal to SliceQPY at the start of each slice.

## Deblocking filtering with fine granularity slice when filtering across slice boundaries is disabled

When loop_filter_across_slice_flag is set to 0, deblocking filter is not applied for slice boundary pixels. As for CU boundaries, HM3.2 checks the presence of slice boundary by examining at top-left pixel of the CU, so it results in wrong decision.

Fig. 3 depicts an example of this issue. HM3.2 checks left CU’s availability by examining at the top-left corner of the current CU (the gray block in the figure). Since the left neighboring pixel at this position belongs to a different slice, the whole left edge of the current CU is excluded from filtering. The lower part of the CU left boundary (marked in red in the figure), however, has the left neighboring pixels in the same slice, so the deblocking filter should be applied on them.

Fig. 3: Example of HM3.2 deblocking filter with fine granularity slice when filtering across slice boundaries is disabled

### Suggested change in HM3.2 software

TComLoopFilter::xSetLoopfilterParam() sets up m_stLFCUParam.bLeftEdge and m_stLFCUParam.bTopEdge for CU boundaries, but they should be ignored if slice granularity is more than 0.

The concrete code change is not available in this contribution.

### Suggested change in WD3

No change is needed.

# References

[1]	T. Wiegand, B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, “WD3: Working Draft 3 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-E603, Geneva, April 2011.

# Patent rights declaration(s)

Texas Instruments, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation |ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).