| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F697 |
| --- | --- |

| Title: | CE6.b Crosscheck for additional simulation results of RQT and SDIP harmonization from NEC and Motorola Mobility(JCTVC-F684) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Changcai Lai Lingzhi Liu Jianhua Zheng | Tel: Email: | +86-0755-28422002 laichangcai@huawei.com |
| Source: | HiSilicon |  |  |

_____________________________

# Abstract

This contribution document reports the cross verification results of NEC and Motorola Mobility’s additional simulation results of RQT and SDIP harmonization (JCTVC-F684). The source code was implemented based on CE6.b test1 software in which RQT and SDIP harmonization was implemented based on HM3.0-SDIP branch software. The cross-check has been completed successfully and the results match those provided by proponent of JCTVC-F684.

# Test description and Results

The source code was implemented based on CE6.b test1 software, and the anchor is the HM3.0-SDIP branch software with default configurations.

In CE6.b Test1 [1], one scheme (which is referred as Case2 in CE6.b Test1) allows the maximum RQT depth value 2 for rectangular PUs (16x4, 4x16, 32x8, and 8x32 PUs); another one (which is referred as Case1 in CE6.b Test1) restricts the maximum RQT depth value to 1 for 32x8 and 8x32 PUs. In this proposal, for case 1 and case 2, RQT depth values 1 and 2 are crosschecked, when RQT depth is 3, the tests are the same as tested in JCTVC-F533[2]. Therefore, four cases are crosschecked, the encoder and decoder reconstructed YUV files match. These results match the BD-rate results provided by proponents of JCTVC- F684. The decoding time and encoding time change are very similar with those of proponent’s

The experimental results of the rate-distortion are shown in Error: Reference source not found to Table 4 for all Intra case.

Table 1 Results of Case1 RQT depth=2 results (Case1 Harmonization vs. No-Harmonization)

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.01 | 0.06 | 0.08 | -0.19 | -0.69 | -0.54 |
| Class B | -0.03 | 0.20 | 0.22 | -0.24 | -0.60 | -0.53 |
| Class C | -0.03 | 0.21 | 0.24 | -0.26 | -0.47 | -0.47 |
| Class D | -0.02 | 0.22 | 0.22 | -0.23 | -0.35 | -0.36 |
| Class E | -0.04 | 0.34 | 0.34 | -0.32 | -1.15 | -1.09 |
| All | -0.03 | 0.20 | 0.22 | -0.24 | -0.63 | -0.57 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 2 results of Case1 RQT depth=1 results (Case1 Harmonization vs. No-Harmonization)

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.02 | 0.07 | 0.29 | -0.29 | -0.80 | -0.76 |
| Class B | -0.03 | 0.16 | 0.22 | -0.46 | -0.83 | -0.91 |
| Class C | -0.05 | 0.22 | 0.24 | -0.44 | -0.72 | -0.74 |
| Class D | -0.05 | 0.18 | 0.20 | -0.35 | -0.51 | -0.49 |
| Class E | -0.04 | 0.29 | 0.27 | -0.59 | -1.75 | -1.58 |
| All | -0.04 | 0.18 | 0.24 | -0.42 | -0.88 | -0.86 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 3: Results of Case2 RQT depth=2 results (Case2 Harmonization vs. No-Harmonization)

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.03 | -0.02 | 0.05 | -0.18 | -0.65 | -0.52 |
| Class B | -0.22 | -0.15 | -0.15 | -0.56 | -0.63 | -0.61 |
| Class C | -0.06 | 0.11 | 0.14 | -0.32 | -0.48 | -0.50 |
| Class D | -0.08 | 0.13 | 0.13 | -0.31 | -0.37 | -0.38 |
| Class E | -0.47 | -0.37 | -0.31 | -0.85 | -1.35 | -1.18 |
| All | -0.16 | -0.05 | -0.02 | -0.43 | -0.66 | -0.61 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Table 4: Results of Case2 RQT depth=1 results (Case2 Harmonization vs. No-Harmonization)

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.02 | 0.07 | 0.29 | -0.29 | -0.80 | -0.76 |
| Class B | -0.03 | 0.16 | 0.22 | -0.46 | -0.83 | -0.91 |
| Class C | -0.05 | 0.22 | 0.24 | -0.44 | -0.72 | -0.74 |
| Class D | -0.05 | 0.18 | 0.20 | -0.35 | -0.51 | -0.49 |
| Class E | -0.04 | 0.29 | 0.27 | -0.59 | -1.75 | -1.58 |
| All | -0.04 | 0.18 | 0.24 | -0.42 | -0.88 | -0.86 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 97% |  |  |

- Reference

[1] A. Tabatabai et al. "Core Experiment 6: Intra Prediction Improvement," JCTVC-E706, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, Switzerland, 16-23 March, 2011.

[2] X. Cao, X. Peng, C. Lai et al. “CE6.b Report on test1 Harmonization of SDIP and RQT,” JCTVC-F533, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 6th Meeting: Torino, IT, 14-22 July, 2011.