| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F701 |
| --- | --- |

| Title: | Encoding optimization to improve coding efficiency for low delay cases |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu Feng Wu Houqiang Li |  |  |
| Source: | University of Science and Technology of China Microsoft |  |  |

_____________________________

# Abstract

This document presents some encoder only actions to improve the performance of low delay configuration. Two methods will be introduced, one is reference frame selection; the other is lambda and QP adjustment (bit allocation). The experimental results show that average 9% bits saving can be obtained from that encoder only decision. The maximum bit saving is more than 19%.

# Introduction

In the current design of HEVC, when low delay configuration is used, the nearest 4 reference frames are used as the reference frame. Actually, similar to H.264/AVC, when the number of reference frame in the buffer reaches max_num_ref_frames, which frame to be removed from the buffer can be specified by MMCO. The experimental results show that by MMCO, about 3% bits saving can be obtained. Another attempt we tried is changing the bit allocation according to the content. If the bit allocation is changed according to the content, about another 6% bits saving can be obtained.

# Solution

## Reference frame selection

Let us denote n+1 is the number of frames to be encoded (count from frame 0, so the last frame is frame n); r is the number of reference frames can be used for one frame. s(i) is the reference frame selection for frame i. To encode the whole sequence, it is to optimize the next equation,

s1,s2,...,snopt=argmins1,s2,...,sni=1nDi+λiRi,s.t.si≤r∧sn+1⊆sn∪n

This optimization problem can be solved by dynamic programming with many cores under some reasonable assumptions.

## Lambda and QP adjustment

The propagation of the distortion from one frame to the other frames can be estimated. The lambda is determined by the propagation factor. We change the RD cost calculation from D+λR to ωiD+λR, where ωi is the weighting factor for frame i, means how much the total RD-cost will be influenced by changing one unit’s distortion in frame i. ωi is estimated as ωi=1+Δj=i+1nJjΔDi, where J is the RD cost.

After determining the QP for each frame can be performed to choose the best QP to match that estimated lambda. Actually deciding lambda and QP can be treated as one step.

## The iteration between reference frame selection and Lambda (QP) adjustment

The result of the first step can be used as the initial status of the second step, and also the result of the second step can also be used as the initial status of the first step. The iteration can further improve the coding efficiency.

# Experimental Results

The low delay high efficiency configuration was tested. As the complexity of the first step is very high, only the first 32 frames in each sequence were tested with the help of cloud. The summary results are shown in Table 1 (JCTVC-F701_ref4_averagePSNR.xls) and Table 2 (JCTVC-F701_ref2_averagePSNR.xls). In the following table, pass 0 (3, 6) means the results of the reference frame selection, pass 2 (5, 8) means the results of lambda and QP adjustment. The results of only lambda adjustment are skipped.

Table 1 Summary of using 4 reference frames (first 32 frames, Y BD-Rate)

|  |  | pass0 | pass2 | pass3 | pass5 | pass6 | pass8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| class B | Kimono | -0.2 | -6.1 | -6.0 | -6.5 | -6.3 | -6.5 |
|  | ParkScene | -0.4 | -4.6 | -4.6 | -4.8 | -4.8 | -5.1 |
|  | Cactus | -3.1 | -8.9 | -8.8 | -9.1 | -9.0 | -8.9 |
|  | BasketballDrive | -0.4 | -4.0 | -3.9 | -4.4 | -4.6 | -4.6 |
|  | BQTerrace | -8.4 | -14.2 | -14.9 | -15.5 | -15.1 | -15.6 |
| class C | BasketballDrill | -5.0 | -17.7 | -17.7 | -19.3 | -19.2 | -19.2 |
|  | BQMall | -3.5 | -10.2 | -9.9 | -10.2 | -10.1 | -9.8 |
|  | PartyScene | -4.2 | -10.4 | -10.3 | -13.4 | -13.3 | -12.4 |
|  | RaceHorses | -2.0 | -6.4 | -6.2 | -6.5 | -6.4 | -6.5 |
| class D | BasketballPass | -0.9 | -7.3 | -7.2 | -8.4 | -8.2 | -8.5 |
|  | BQSquare | -13.2 | -18.9 | -18.8 | -18.8 | -18.4 | -18.9 |
|  | BlowingBubbles | -2.0 | -7.0 | -7.0 | -7.2 | -7.1 | -7.1 |
|  | RaceHorses | -1.5 | -5.4 | -5.0 | -5.4 | -5.0 | -5.1 |
| class E | Vidyo1 | -0.5 | -5.1 | -5.0 | -5.5 | -4.8 | -4.5 |
|  | Vidyo3 | -0.2 | -6.5 | -6.4 | -7.2 | -6.8 | -7.3 |
|  | Vidyo4 | -0.2 | -8.2 | -8.3 | -8.6 | -8.3 | -8.5 |
| Average |  | -2.9 | -8.8 | -8.8 | -9.4 | -9.2 | -9.3 |

Table 2 Summary of using 2 reference frames (first 32 frames, Y BD-Rate)

|  |  | pass0 | pass2 | pass3 | pass5 | pass6 | pass8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| class B | Kimono | -0.2 | -6.2 | -6.2 | -6.5 | -6.4 | -6.5 |
|  | ParkScene | -1.5 | -5.5 | -5.5 | -5.6 | -5.5 | -5.6 |
|  | Cactus | -3.3 | -9.4 | -9.4 | -9.6 | -9.6 | -9.5 |
|  | BasketballDrive | -1.0 | -4.6 | -4.6 | -5.0 | -5.0 | -4.9 |
|  | BQTerrace | -10.3 | -13.8 | -13.9 | -15.3 | -15.3 | -15.1 |
| class C | BasketballDrill | -5.8 | -19.4 | -19.4 | -18.7 | -18.7 | -20.3 |
|  | BQMall | -2.9 | -10.0 | -10.1 | -10.7 | -10.7 | -10.5 |
|  | PartyScene | -5.3 | -11.6 | -11.8 | -12.2 | -12.2 | -14.1 |
|  | RaceHorses | -1.4 | -6.0 | -5.9 | -6.3 | -6.3 | -6.3 |
| class D | BasketballPass | -1.2 | -7.9 | -7.3 | -7.6 | -7.3 | -7.4 |
|  | BQSquare | -13.9 | -20.3 | -21.2 | -22.5 | -22.6 | -23.6 |
|  | BlowingBubbles | -2.7 | -7.8 | -7.8 | -7.9 | -7.6 | -7.4 |
|  | RaceHorses | -1.5 | -6.0 | -5.8 | -6.0 | -5.9 | -6.0 |
| class E | Vidyo1 | -1.1 | -5.9 | -5.5 | -5.7 | -5.5 | -5.4 |
|  | Vidyo3 | -3.8 | -9.2 | -9.0 | -9.6 | -9.4 | -9.3 |
|  | Vidyo4 | -0.5 | -7.7 | -7.4 | -8.0 | -7.7 | -8.5 |
| Average |  | -3.5 | -9.5 | -9.4 | -9.8 | -9.7 | -10.0 |

The results in Table 1 and 2 are average PSNR, which means the PSNR is calculated in each frame and the sequence PSNR is the average of the PSNR of each frames. As we changed the lambda and QP in the second step, we also calculated the overall PSNR, which means we sum the distortion in each frame and the sequence PSNR is calculated by the distortion of the whole sequence. The average results show that the Y BD-Rate of average PSNR and overall PSNR do not have significant difference. The detailed results are in JCTVC-F701_ref4_overallPSNR.xls and JCTVC-F701_ref2_overallPSNR.xls.

Especially for the first step (reference frame selection), we also did more experiments. The reference frame selection algorithm introduced was only tested for the first 32 frames, thus the weight of the bits in the first I frame is very large. As we did not change anything of the coding of the first I frame in that step, we also calculated the BD-Rate without the first I frame. The summary results (average PSNR) are shown in Table 3 (JCTVC-F701_ref4_pass0_BOnly.xls and JCTVC-F701_ref2_pass0_BOnly.xls).

Table 3 Summary Results of the first step without I frame (32-1 frames, Y BD-Rate)

| sequence | 4 ref | 2 ref |
| --- | --- | --- |
| Kimono | -0.2 | -0.2 |
| ParkScene | -0.5 | -1.8 |
| Cactus | -4.0 | -4.2 |
| BasketballDrive | -0.5 | -1.1 |
| BQTerrace | -11.5 | -13.8 |
| BasketballDrill | -5.7 | -6.6 |
| BQMall | -4.5 | -3.7 |
| PartyScene | -4.9 | -6.3 |
| RaceHorses | -2.3 | -1.6 |
| BasketballPass | -1.1 | -1.4 |
| BQSquare | -17.1 | -18.1 |
| BlowingBubbles | -2.3 | -3.2 |
| RaceHorses | -1.7 | -1.7 |
| Vidyo1 | -0.7 | -1.7 |
| Vidyo3 | -0.3 | -5.2 |
| Vidyo4 | -0.2 | -0.7 |
| average | -3.6 | -4.5 |

For the reference frame selection, we did more experimental results. Instead of setting the reference frame as the 4 nearest frames, we used 1 nearest frame and 3 high quality frames (POC%4==0, as the rate GOP size in the default test configuration is 4) as the reference frame. In this kind of reference frame setting, the reference frame buffer required at the decoder did not change. max_num_ref_frames was added at SPS (this syntax element should be there but currently not). When the size of reference frame buffer reached max_num_ref_frames and the current frame will be used as reference frame, which frame to be removed out from the reference frame buffer is sent to the decoder. For the default low delay setting, no more than 4 frames reference frame buffer are required at the decoder side. The summary result of 1+3 reference frame setting is shown in Table 4 (JCTVC-F701_Ref4(1+3).xls). We tested the whole sequence in this experiment.

Table 4 Summary result of 4 reference frames (1 nearest + 3 high quality) (Y BD-rate)

|  | LD_HE | LD_LC | LD_P_HE | LD_P_LC |
| --- | --- | --- | --- | --- |
| Class B | -2.8 | -3.2 | -2.5 | -2.3 |
| Class C | -4.3 | -4.6 | -4.0 | -4.0 |
| Class D | -4.7 | -5.2 | -4.7 | -4.9 |
| Class E | -1.2 | -1.6 | -1.4 | -1.1 |
| All | -3.3 | -3.8 | -3.2 | -3.2 |
| Enc Time | 104% | 106% | 109% | 111% |
| Dec Time | 98% | 98% | 100% | 101% |

# Patent rights declaration(s)

University of Science and Technology of China, and Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).