| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, Italy, 14-22 July, 2011 | Document: JCTVC-F706 |
| --- | --- |

| Title: | Cross-verification of Intel’s Proposal JCTVC-F502 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Authors: | Mei Guo Xun Guo North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 | Email: | mei.guo@mediatek.com xun.guo@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck JCTVC- F502 on cross-channel techniques about intra chroma prediction, which is submitted by Intel. It is reported that the results match those provided by Intel exactly.

# Introduction

Intel provided the HM3.0-based source code as described in JCTVC-F502 [1]. In JCTVC-F502 [1], two techniques are proposed to improve intra chroma prediction. The first one applies a 2-dimensional 3x2 filter for LM intra chroma prediction mode. And the second one uses local adaptive prediction selection (LAPS) process to produce the intra chroma prediction sample from the comparison of the reconstructed luma sample of target location to the local reconstructed luma samples in the neighborhood. The verification of the first technique is carried out by setting the following parameters in the provided code:

#define INTEL_LM_FILTER              1.

The verification of the second technique is performed by setting the following parameters:

#define INTEL_LAPS                   1

#if INTEL_LAPS

#define INTEL_ADD_MODE               1

#define INTEL_LAPS_PREDLIST_THREE    0

#endif.

In the test of both two techniques, the parameters are set as:

#define INTEL_LM_FILTER              1.

#define INTEL_LAPS                   1

#if INTEL_LAPS

#define INTEL_ADD_MODE               1

#define INTEL_LAPS_PREDLIST_THREE    0

#endif.

The test is performed under the conditions defined in JCTVC-E700 [2] with high efficiency all-intra (HE-AI), and low complexity all-intra (LC-AI) configurations. In this crosscheck, we compare the results we got with Intel’s software and the results provided by Intel. They are exactly the same.

# Results

Our experimental results match those provided by Intel exactly. The results are listed in JCTVC-F706.xls.

# Conclusion

The results presented by Intel in JCTVC-F502 are confirmed.

# References

- Y. Chiu, Y. Han, L. Xu, W. Zhang, and H. Jiang, “Cross-channel techniques to improve intra chroma prediction”, JCTVC-F502, 6th Meeting: Torino, Italy, 14-22 July, 2011.

- F. Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E700, 5th Meeting: Geneva, Switzerland, 16-23 March, 2011.