| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F707 |
| --- | --- |

| Title: | CE4 Subtest 2: Cross check report of JCTVC-F661 for MinCUDQPSize = 32x32 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Muhammed Coban 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | +1 858 658 3973 mcoban@qualcomm.com |
| Source: | Qualcomm Inc |  |  |

_____________________________

# Abstract

This contribution provides a summary of a cross-check results of spatial QP prediction method composed of a combination of the test 2.3.g, 2.3.f and 2.3.e of CE4 performed by Qualcomm of JCTVC-F661. The results of the crosscheck matched those reported by NEC. The encoding and decoding times also matched closely.

# Results

Tables 1 shows the summary of experimental results with MinCUDQPSize = 32x32. No YUV files were written when the decoding times were measured. Encoding as well as decoding was done on a LINUX cluster.

Table 1: Summary results of test 2.3.g+2.3.f+2.3.e.r1 vs. CE4 anchor [3], MinCUDQPSize = 32x32.

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | 0.0 | -0.1 | 0.0 | -5.6% | 0.0 | 0.0 | 0.0 | -4.8% |
| Class B | -0.1 | -0.1 | -0.1 | -9.8% | -0.1 | -0.1 | -0.1 | -8.2% |
| Class C | -0.1 | -0.1 | -0.1 | -11.0% | -0.1 | -0.1 | -0.1 | -10.8% |
| Class D | 0.0 | 0.0 | 0.0 | -8.7% | 0.0 | 0.0 | 0.0 | -9.3% |
| Class E | -0.2 | -0.3 | -0.2 | -19.0% | -0.2 | -0.2 | -0.2 | -16.3% |
| All | -0.1 | -0.1 | -0.1 | -10.4% | -0.1 | -0.1 | -0.1 | -9.5% |
| Enc Time[%] | 105% |  |  |  | 103% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 102% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | 0.0 | -0.1 | -0.5 | -3.1% | 0.0 | -0.1 | 0.1 | -2.4% |
| Class B | -0.1 | -0.1 | 0.0 | -5.0% | -0.1 | -0.1 | -0.1 | -4.7% |
| Class C | -0.1 | -0.1 | 0.0 | -8.8% | -0.1 | -0.1 | -0.1 | -9.4% |
| Class D | -0.1 | -0.2 | 0.0 | -6.8% | -0.1 | 0.0 | 0.0 | -7.6% |
| Class E |  |  |  |  |  |  |  |  |
| All | -0.1 | -0.1 | -0.1 | -5.9% | -0.1 | -0.1 | 0.0 | -6.0% |
| Enc Time[%] | 96% |  |  |  | 94% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 0.0 | -0.1 | -0.3 | -4.4% | 0.0 | -0.1 | -0.2 | -3.6% |
| Class C | -0.1 | -0.2 | -0.2 | -8.7% | -0.2 | -0.2 | -0.1 | -9.0% |
| Class D | 0.0 | -0.2 | -0.1 | -6.6% | -0.1 | 0.2 | -0.1 | -7.5% |
| Class E | 0.0 | 0.4 | 0.4 | -8.4% | -0.1 | 0.0 | -0.3 | -7.8% |
| All | -0.1 | 0.0 | -0.1 | -6.8% | -0.1 | 0.0 | -0.2 | -6.7% |
| Enc Time[%] | 95% |  |  |  | 96% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |  |

# Conclusions

We have verified results reported in JCTVC-F661 for MinCUDQPSize = 32x32. The results of the crosscheck matched those reported by NEC. The encoding and decoding times also matched closely.

# References

- F. Bossen, “Common test conditions and software reference configurations,” JCT-VC document JCTVC-E700, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 5th meeting, Geneva, March 2011.

- H. Aoki, et al., “CE4 Subtest 2: Spatial QP prediction: combination of test 2.3.g, 2.3.f and 2.3.e,” JCT-VC document JCTVC-F661, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.

- K. Sato, et al., “Description of Core Experiment 4: Quantization,” JCT-VC document JCTVC-E704, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 5th meeting, Geneva, March 2011.